16-Ch HD Video Distributor User’s Manual

V1.0.0

Important Safeguards and Warnings

Please read the following safeguards and warnings carefully before using the product in order to avoid damages and losses.

Note

- Do not expose the device to lampblack, steam or dust. Otherwise it may cause fire or electric shock.

- Do not install the device at position exposed to sunlight or in high temperature. Temperature rise in device may cause fire.

- Do not expose the device to humid environment. Otherwise it may cause fire.

- The device must be installed on solid and flat surface in order to guarantee safety under load and earthquake. Otherwise, it may cause device to fall off or turnover.

- Do not place the device on carpet or quilt.

- Do not block air vent of the device or ventilation around the device. Otherwise, temperature in device will rise and may cause fire.

- Do not place any object on the device.

- Do not disassemble the device without professional instruction.

Warning

- Please use battery properly to avoid fire, explosion and other dangers.

- Please replace used battery with battery of the same type.

- Do not use power line other than the one specified. Please use it properly. Otherwise, it may cause fire or electric shock.

Special Announcement

- This manual is for reference only.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website for more information.

Table of Contents

1	Product Overview	- 3 -

1.1 Product Features	- 3 -

1.2 Typical Application	- 3 -

2	Device Structure	- 4 -

2.1	Front Panel	- 4 -

2.2	Rear Panel	- 4 -

Appendix 1 Technical Specifications	- 6 -

# Product Overview

## 16-Ch HD Video Distributor realizes 16-ch coaxial HD video input into 32-ch output, which guarantees lossless real-time transmission for the HD video signal, which is mainly applied in the device room.

## Product Features

  - Support 16-ch HDCVI Input, 32-ch output, each input is corresponding to 2 outputs.

  - Support HDCVI video format: 720P/25, 720P/30, 720P/50, 720P/60, 1080P/25 and 1080P/30.

  - Support positive video, coaxial positive data, coaxial positive audio and coaxial reverse control.

  - Lossless, real-time transmission.

  - Compatible CVBS, AHD, HDTVI signal distribution.

  - Appearance dimension: enterprise-level metal structure design, 19-inch 1U chassis.

  - Working temperature: -10℃～55℃, max power consumption: 2W.

  - Video access indicator and power indicator.

## Typical Application

The typical application of the device is shown in Figure 1-1.

- Figure 1-1

# Device Structure

## Front Panel

The front panel of the device is shown in Figure 2-1.

- Figure 2-1

- Refer to Sheet 2-1 for more details about indicators.

| Indicator | Note | Color | Device status |
| --- | --- | --- | --- |
| PWR | Power indicator | Green | Power connection normal |
| V1/V2/V3/V4/V5/ V6/V7/V8/V9/V10/ V11/V12/V13/V14/ V15/V16 | Video access indicator | Green | Video access normal |

- Sheet 2-1

- Refer to Sheet 2-2 for more details about ports.

| Port name | Note |
| --- | --- |
| IN1/IN2/IN3/IN4/ IN5/IN6/IN7/IN8/ IN9/IN10/IN11/ IN12/IN13/IN14/ IN15/IN16 | 16-ch HD video input. Interface type: BNC. |

- Sheet 2-2

## Rear Panel

16-ch HD video input through IN port and output with 32 channels. The rear panel of the device is shown in Figure 2-2.

Figure 2-2

Please refer to Sheet 2-3 for more details about the ports.

| Port Name | Note |
| --- | --- |
| OUT1/OUT2/OUT3/OUT4/ OUT5/OUT6/OUT7/OUT8/ OUT9/OUT10/OUT11/ OUT12/OUT13/OUT14/ OUT15/OUT16 | 16-ch HD video output port. Port type: BNC. |
| OUT1ˊ/OUT2ˊ/OUT3ˊ/OUT4ˊ/ OUT5ˊ/OUT6ˊ/OUT7ˊ/OUT8ˊ/ OUT9ˊ/OUT10ˊ/OUT11ˊ/OUT12ˊ /OUT13ˊ/OUT14ˊ/OUT15ˊ/OUT16ˊ | 16-ch HD video loop output. Port type: BNC. |
| 100-240V AC | Power port |

Sheet 2-3

- Appendix 1 Technical Specifications

| Product model | PFT2690 |
| --- | --- |
| - Input |  |
| Physical port | 16*BNC |
| Input impedance | 75Ω |
| Output |  |
| Physical port | 32*BNC |
| Output impedance | 75Ω |
| - Functions and Features |  |
| - Video format | - HDCVI 720P/25、720P/30、720P/50、720P/60、1080P/25、1080P/30 |
| - HDCVI signal | - Positive video, coaxial positive data, coaxial positive audio, coaxial reverse control |
| - Video compatibility | - CVBS、AHD、HDTVI |
| - Signal features | - Lossless, real-time |
| Other |  |
| Lightning protection level | Level 3 |
| Indicator | Power indicator, video access indicator |
| Power | 100-240V AC |
| Power consumption | ≤2W |
| Application humidity | 10%～90% |
| High and low temperature | ﹣10℃～55℃ |
| Weight | 2.78kg |
| Dimension | 440mm×200mm×43.65mm |
| - Installation | 19-inch standard rack |

Note

- This user’s manual is for reference only.

- Slight difference may be found in user interface.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website for more information.