HDMI-HDCVI Converter User’s Manual

V1.0.0

Important Safeguards and Warnings

Please read the following safeguards and warnings carefully before using the product in order to avoid damages and losses.

Note

- Do not expose the device to lampblack, steam or dust. Otherwise it may cause fire or electric shock.

- Do not install the device at position exposed to sunlight or in high temperature. Temperature rise in device may cause fire.

- Do not expose the device to humid environment. Otherwise it may cause fire.

- The device must be installed on solid and flat surface in order to guarantee safety under load and earthquake. Otherwise, it may cause device to fall off or turnover.

- Do not place the device on carpet or quilt.

- Do not block air vent of the device or ventilation around the device. Otherwise, temperature in device will rise and may cause fire.

- Do not place any object on the device.

- Do not disassemble the device without professional instruction.

Warning

- Please use battery properly to avoid fire, explosion and other dangers.

- Please replace used battery with battery of the same type.

- Do not use power line other than the one specified. Please use it properly. Otherwise, it may cause fire or electric shock.

Special Announcement

- This manual is for reference only.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website for more information.

Table of Contents

1	Product Overview	- 1 -

1.1 Product Features	- 1 -

1.2 Typical Application	- 1 -

2	Device Structure	- 2 -

2.1	Front Panel	- 2 -

2.2	Rear Panel	- 2 -

Appendix 1 Technical Specifications	- 3 -

# Product Overview

## Product Features

  - Input HDMI video signal, self-adaptive output HDCVI signal.

  - HDMI support mode：1024*768@60Hz, 1280*720@50/60Hz, 1280*1024@60Hz,

- 1600*1200@60Hz,1920*1080@50/60Hz.

- HDCVI output format：720P/50, 720P/60, 1080P/25, 1080P/30.

- LED status indicator shows the working condition of video converter.

- Fully enclosed dustproof design.

## Typical Application

The typical application of HDMI-HDCVI Converter is shown in Figure 1-1.

- Figure 1-1

Note:

- When input HDMI video format is 1280*720@50Hz, output HDCVI format is 720P/50.

- When input HDMI video format is 1024*768@60Hz, 1280*720@60Hz or

- 1280*1024@ 60Hz, output HDCVI format is 720P/60.

- When input HDMI video format is 1920*1080@50Hz, output HDCVI format is 1080P/25.

- When input HDMI video format is 1600*1200@60Hz or 1920*1080@60Hz,output HDCVI format is 1080P/30.

# Device Structure

## Front Panel

The front panel of HDMI-HDCVI Converter is shown in Figure 2-1.

- Figure 2-1

- Please refer to sheet 2-1 for more details about port.

| Port | Note |
| --- | --- |
| HDMI | HDMI video input |

- Sheet 2-1

## Rear Panel

The rear panel of HDMI-HDCVI Converter is shown in Figure 2-2.

Figure 2-2

Please refer to Sheet 2-2 for more details about indicator light.

| Indicator light | Note | Color | Device status |
| --- | --- | --- | --- |
| PWR | Power status indicator | Green | Normal power connection |
| VIDEO | Video source indicator | Green | Normal video access |
| ALR | Alarm indicator | Red light flashing | Video format which is not supported |

Sheet 2-2

Please refer to Sheet 2-3 for more details about port.

| Port | Note |
| --- | --- |
| HDCVI | HDCVI video output |

Sheet 2-3

- Appendix 1 Technical Specifications

| Product model | - PFT2100 |
| --- | --- |
| - Input HDMI Specifications |  |
| Physical port | HDMI |
| Port specification | HDMI A Type |
| HDMI version | HDMI 1.3,backward compatibility HDMI1.2 HDMI1.1 and etc. |
| HDCP version | HDCP1.2 |
| HDMI signal format | EIA/CEA-861D standard |
| HDMI resolution | - 1024*768@60Hz,1280*720@50/60Hz, 1280*1024@60Hz,1600*1200@60Hz, - 1920*1080@50/60Hz |
| Max transmission rate | 10.2Gbps |
| Input output TMDS signal | 0.5~1.5Vp-p（TDMS） |
| Input output DDC signal | 5Vp-p（TTL） |
| Input cable length | ≤5m（AWG26 HDMI1.3 Cable Standard） |
| Output HDCVI Specifications |  |
| Physical port | BNC |
| HDCVI format | 720P/50、720P/60、1080P/25、1080P/30 |
| Output level | 1Vp-p |
| Output cable length | 1080P：75-5 coaxial cable support 300m 720P：75-5 coaxial cable support 500m |
| Output impedance | 75Ω |
| Video bandwidth | 45MHz |
| Other Specifications |  |
| Indicator light | Power indicator, video source indicator, alarm indicator |
| Power | DC12V 1A |
| Power consumption | <5W |
| Application humidity | 10%～90% |
| High and low temperature | ﹣10℃～55℃ |
| Weight | 315g |
| Dimension | 104.6mm×96.2mm×29.5mm |
| Installation mode | Wall-mounted |

Note

- This user’s manual is for reference only.

- Slight difference may be found in user interface.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website for more information.