PVR200 User’s Manual
[image: image1.jpg]



Table of Contents

61
General Introduction


61.1
Feature


61.2
Specification


82
Framework


113
PVR Interface and Operation


113.1
Boot up and Shutdown


113.2
System Monitor and Management Interface


123.3
Record Operation


123.4
Playback


163.5
Format


163.6
Parameter Setup


173.6.1
Time Setup


173.6.2
Power Setup


183.6.3
System Information and Firmware Upgrade


183.6.4
Firmware Upgrade


193.6.5
Audio/Video


193.6.6
Boot up Interface


193.7
Language


203.8
Shut Down




Welcome
Thank you for purchasing our PVR200 series product!

This user’s manual is designed to be a reference tool for the installation and operation of your system. 
Please read the following safeguard and warnings carefully before you use this series product!

Please keep this user’s manual well for future reference!

[image: image40.jpg]E E sl onTrRoL”




[image: image2.png]



Safeguard and Warnings

1. Do not dismantle PVR device without authorization 

All the installation and operations here should conform to your local electric safety rules.

We assume no liability or responsibility for all the fires or electrical shock caused by unauthorized handling or installation.

Be sure to use all the accessories recommended by manufacturer.
2. Please handle PVR device carefully

This device shall avoid strike or strong vibration. PVR device may be damaged if you use or storage improperly.

3. Do not leave the PVR device in corrosive environment such as chemical factory.
4. Do not expose the camera to strong light for a long time. It may damage the sensitive components. 
5. Do not remove the SD card when you implement the SD card operation such as record, playback, format, delete, upgrade and etc. Otherwise it may result in SD card file damage!
6. When system is upgrading, please make sure the battery power capacity is sufficient. Please connect the device to the power charger if possible. During the whole upgrade process; do not remove the SD card! Otherwise it may result in upgrade failure and system can not restore to the proper running status!
7. Please pay attention to SD card installation direction. Otherwise it may result in SD card damage or PVR damage!
8. Please make sure PVR device connection is right before you begin recording.
1 General Introduction
1.1 Feature 

This portable video recorder (PVR) is law enforcement equipment for individual suitable for special industries such as police force, army force. It can completely record the law enforcement process and provide on-site audio record, video record, playback function. It is portable and is compatible with half-fixed use. PVR is very convenient to carry and is easy to operate.
1.2 Specification 

	Name 
	PVR200

	Video 
	Record Resolution 
	VGA(640*480）

	
	Compression Algorithm
	MPEG4

	Audio 
	Compression Mode 
	MP3

	Battery 
	Battery working period (Main unit only)
	About 5 hours.

	
	Battery capacity threshold alarm
	When battery left power capacity is less than
10 minutes (adjustable), the buzzer begins beeping regularly to alert you to recharge.

	
	LCD power management 
	System can automatically shut down LCD power after the PVR stopped operation for specified time such as ten seconds.

	Unit Port 
	Mini USB  port 
	USB2.0 high-speed port 

	
	1394b port 
	Connect the main unit to the cable control module.

	
	Recharge port 
	To recharge the device.

	
	SD card
	One SD card port

	
	Key 
	Nine keys (Including one on/off button and 8 special keys.)

	Storage 
	SD card
	Support SD1.0、SD1.1、SD2.0 standard.

	
	SD card alarm
	When the file is about to full (about five-minutes recording time left), you can hear buzzer beeps to alert you to replace a SD card. 



	Status Indication Light 
	Cable control light 
	Record status：Cable control light is on.
Standby or stop status: cable control light flashes once four seconds.
Shut down mode: Cable control light all becomes off.


2 Framework
This series PVR has the following interfaces. 

· Cable control ports.

· External power port 

· MINI USB port 

· SD card port 

· External control keys (Total nine keys)
Please refer to Figure 2‑1 for general structure.
Please refer to Figure 2‑2 for external keys.

Please refer to Figure 2‑3for external ports.
Please refer to Figure 2‑4 for cable control information.

[image: image41.jpg]



[image: image3.png]




Figure 2‑1
[image: image4.png]it





Figure 2‑2

[image: image5.png]=D





Figure 2‑3
[image: image6.jpg]



Figure 2‑4
· Cable control：Here you can input cable control to provide audio and video signal, the key control signal input and status indication light.

· External power port (it is round and has a socket in the middle.): It is to provide power to the device.
· SD card port：Input ordinary SD card. Please pay attention to SD card installation direction. It supports SD1.0, SD1.1 and SD 2.0 standard.
· Nine keys (including one power button and eight special keys. Among which, the snapshot button is not available right now.)

Cable control sub-system includes one camera, one microphone, two control keys and one status indication light. System provides analog audio and video signal input and keystroke control signal input. Status indication light can represent various statuses (On/Off/Flash. Please refer to Chapter 1.2 Specification for detail information)

3 PVR Interface and Operation

3.1  Boot up and Shutdown

Press the power button for over two seconds, PVR system boots up.  System goes to real-time monitor status by default.
When PVR is running properly, press the power button or cable control key for three seconds; you can turn off the PVR.  You can see shut down interface and then system begins three-second countdown. System stops internal operation (such as record or play) first and then shut down. 

Note
When there is strong interference from the outside or there is invalid system operation, you can turn off the power and reload again.
3.2 System Monitor and Management Interface

After you booted up the system, you can see monitor interface is shown as in Figure 3‑1.
The white block at the bottom means you are in monitor interface.
[image: image7.png]2009-08-05 16:39:22 [soRol ]

BRlight:Auto Video

W 00:00:00 Cord Free: 437 1B




Figure 3‑1
① SD card status：[image: image8.bmp] ：There is no SD card；[image: image9.bmp] ：There is SD card.

② Cable control status： [image: image10.bmp] ：There is no cable control connection；[image: image11.bmp] :There is cable control connection。
③ Battery capacity：When battery left power capacity is less than 10 minutes, the buzzer begins beeping regularly to alert you to recharge.

④ Record status：[image: image12.jpg]


: : Stop recording；[image: image13.jpg]


 : Pause recording；[image: image14.jpg]


 : System is recording now.
⑤ Screen backlight mode: Press OK for a long time to switch.

⑥ Lock button. Press the left and right direction keys within two seconds.
3.3 Record Operation

After system boots up, please click Record button or cable control key to begin recording. The red icon at the bottom and the blue recorder icon at the middle of the top means you are in record interface now. See Figure 3‑2.
[image: image15.jpg]2009-08-05 17:16:50 [soRod )]

BRlight:Auto Video o 00:00:15 Card Free: 43 Min.




Figure 3‑2
When system is recording, you can click Record button again to pause current operation. The blue icon at the bottom means you are in pause interface now. See Figure 3‑3.
[image: image16.png]2009-08-05 17:16:50  HT 00

BRlight:Auto Video | 00:00:03 Card Free: 43 Min.




Figure 3‑3
In pause status, you can click Record button to continue recording. 
In record or pause status, you can press ESC button to stop recording. 

System default recording package time is 60 minutes. When the file is about to full (about five-minutes recording time left), you can hear buzzer beeps to alert you to replace a SD card. 

All recorded files are automatically created and are storage under RECORD folder.

File name format is: Year_Month_Date_Hour_Minute_Second.asf.
3.4 Playback

Press OK button you can go to main menu. You can use “Left” and “Right”button  to highlight playback item. See Figure 3‑4.
[image: image17.png]2009-08-05 17:11:34 00

&

Playback

BRlight:Auto Video

W 00:00:00 Cord Free: 468 1B




Figure 3‑4
Please click OK button to go to playback file list interface. See Figure 3‑5.

[image: image18.png]Playback Files:2 00

1 200908 05 16 34 16.asf La00aE

(© Press "Lett’ key nore than 3 seconds
to enter file-deleting interface!





Figure 3‑5
In playback file list interface you can view all recorded files in SD card RECORD folder. They are arranged in accordance with time. The latest file is on the top. 
System maximally displays 1024 files. Please use Up and Down to select playback file and then click OK button to view.  System automatically plays the next file after completing current one. After it played all files in RECORD folder, system goes back to playback list interface (Figure 3‑5). 

In playback list interface, select one file and then click Left button for a long time, you can go to remove file dialogue box. Please click OK button to delete current file or you can click ESC to exit current interface. Please note you can not restore the deleted file!
During the playback period, click OK button for a long time, you can see a playback control bar shown as in Figure 3‑6 . You can click Cancel button for a little bit while or press OK button for a long time to hide the playback control bar. 
[image: image19.png]2009-08-05 16:53:09 .

Detailed Info

00





Figure 3‑6


[image: image20.png][i] Ha > <> >l
- 00:00:10,/00:00:13 ¢





Figure 3‑7
Please refer to the following sheet for detailed information.
	1
	Previous

	2
	Fast backward

	3
	Play

	4
	Stop

	5
	Volume

	6
	Detail information

	7
	Next

	8
	Fast forward

	9
	Pause

	10
	Set time to play


You can use Left and Right button to select corresponding function and then click OK button to confirm. 
Detail information operation

Please use Left and Right button to highlight [image: image21.jpg]


 and then click OK button to confirm. System pops up the following interface. See Figure 3‑8. Click Cancel button you can exit current interface. 
[image: image22.png]2009-08-05 17:06:49 1 00

udio : W3 2 AudChamel 32000 B 60,000 bps
Video : WEGA 640 X 480 0,030 fps 1,171,136 bps

Detailed Info

HE RN ]
B 00:01:00/00:01:02 ¢





Figure 3‑8
Play previous or next file

Please use Left and Right button to select  [image: image23.jpg]


、[image: image24.jpg]


and then click OK button to confirm. System can playback the previous or next file. When the playback control bar is hidden, you can click Left or Right button once to play.

Fast forward and fast backward

Please use Left and Right button to select  [image: image25.jpg]£



、[image: image26.jpg]


and then click OK button to confirm. Now you can realize fast forward or backward function. When the playback control bar is hidden, you can press Left or Right button for a long time to realize these two functions.

Play/Pause/Stop

When system is playback, please use Left and Right button to select [image: image27.jpg]


and then click OK button to confirm. Now you can see system is in pause mode. You can use Left and Right button to select [image: image28.jpg]


 to restore playback. When the playback control bar is hidden, you can click OK button for a short while to realize pause and restore function. Please use click Left or Right button to select  [image: image29.jpg]


 to stop playback. System goes back to file list interface (Figure 3‑5) after you stopped playback. When the playback control bar is hidden, you can press Cancel button for a short while to stop current playback.

Volume
You can use Up and Down button to adjust volume.
Accurate playback 
Please use Left or Right button to highlight [image: image30.jpg]


 and then click OK button to confirm. There is a red icon at the bottom of the playback bar.  See Figure 3‑9.You can use the Left or Right button to move the red icon, and then click OK button to confirm. Now you can see the playback symbol goes to the red icon position and then begin playback. You can press the Cancel button for a short while to exit current accurate playback.
[image: image31.png]2009-07-15 14:06:11 &1 [soRol ]

00:00:04

[[] Mapp <4 >p 11 B
- 00:00:08/00:00:20 45
S —





Figure 3‑9
3.5 Format

In the main menu, select Format item and then click OK button. See Figure 3‑10.You can go to the format interface. Please follow the prompt to click OK button to begin format process. You can click Cancel button to go back to previous interface.
[image: image32.png]2009-08-05 16:38:17 W 00

Formatting

BRlight:Auto Video & 00:01:11 Card Free: 42 Min.




Figure 3‑10
Important!
All data in the CS card will be removed completely and can not be restored!

Before format operation, please make sure you have backup all important files and data!

3.6 Parameter Setup

Click OK button to go to main menu, highlight parameter item and then click OK to confirm. See Figure 3‑11.

[image: image33.png]2009-08-05 17:03:27 00

BRlight:Auto Video

Lt

Parameters

W 00:00:00 Cord Free: 468 1B




Figure 3‑11
3.6.1 Time Setup

In Figure 3‑11, please use Left and Right button to highlight time item and then click OK button to go to time setup interface. See Figure 3‑12. Click Up and Down button to move the cursor to select year/month/date/hour/minute. You can use Left and Right button to increase or decrease number.  
Please use Up and Down button to move the cursor to highlight setup button and then click OK button to save current modification. You can click ESC button to go back to previous interface after you completed setup.

[image: image34.png]Paran. :Time

Month
Day
Hour

Minute

BRlight:Auto Video

16
39
V3

W 00:00:00 Cord Free: 437 1B




Figure 3‑12
3.6.2 Power Setup

In Figure 3‑11, please use Left and Right button to highlight power item. Please click OK button to go to power setup interface. See Figure 3‑13.

System shut down time in battery mode
In battery power supply mode, system automatically shuts down time when there is no system operation (Exclude record or playback) for specified time.
System shut down time in DC mode
In DC power supply mode, system automatically shuts down time when there is no system operation (Exclude record or playback) for specified time.  

Backlight time: LCD automatically shuts down time when there is no system operation (Exclude record or playback) for specified time.
Key auto lock: When use lock function, lock function becomes active when there is no system operation (Exclude record or playback) for specified time. Please note cable control key will not be locked.
[image: image35.png]Paran. :Pouer 00

Battery
DC pover
Pouer off 10 minutes
Backlight 10 seconds
Key Locked Never

BRlight:Auto Video W 00:00:00 Cord Free: 468 1B




Figure 3‑13
3.6.3 System Information and Firmware Upgrade

In Figure 3‑11, please use Left and Right button to highlight System item and then click OK button to go to system setup interface. See Figure 3‑14.

Here you can view hardware version, software version and SD card capacity. In this interface, you can also upgrade system.

[image: image36.png]Paran. :System Info. 00

HU Version: PURZ00_U1_00

SU Version: PURZ00 U1.00
500000874

SD Card Total: 500255 KB
Used: 51895 KB

BRlight:Auto Video W 00:00:00 Cord Free: 437 IB




Figure 3‑14
3.6.4 Firmware Upgrade

Please go to our website to get the latest upgrade software and then copy it to SD card UPDATE folder. Now you can insert the SD card into the PVR and then go to system setup interface. Click OK button, you can see system pops up a dialogue box asking you to confirm upgrade. Please follow the prompt to click OK button to begin upgrade. You can click Cancel button to exit.  

During the upgrade period, you can refer to the process bar for reference. System needs to restart after completing upgrade. Please note during the upgrade period, system stops all other operations. 

Warning!
Before you upgrade, please make sure the battery capacity is sufficient. 
To guarantee upgrade safety, please connect the device to the recharge. During the upgrade period, do not remove the SD card!

During the whole upgrade period, insufficient battery capacity or SD card remove operation can result in system upgrade failure! System can not operate normally and it can not restore! 
3.6.5 Audio/Video
In Figure 3‑11, please use Left and Right button to highlight Audio/Video item and then click OK button to go to the following interface. See Figure 3‑15.
Please use Left and Right button to select the corresponding file type.

Important!
If system is recording right now, your setup here can not be valid immediately. You need to stop the record first and then enable record again to activate current setup.
[image: image37.png]Paran. :Record Type 00

Uhich record media type?

@ fudiodVideo Ofudio Only

O TR ———

BRlight:Auto Video W 00:00:00 Cord Free: 437 IB




Figure 3‑15
3.6.6 Boot up Interface 
In Figure 3‑11, p, please use Left and Right button to highlight Boot up Interface item and then click OK button to go to the boot up dialogue box. You can set to use your self-defined boot up int6erface.
Click Cancel you can go to the previous interface, click Ok you can update the boot up interface.

Important

Before modification, Please copy the logo.bmp file (16-bit or 24-bit color, 360*240) to the UPDATE folder.

Please make sure the boot up file name is logo.bmp, other file names are invalid. 

3.7 Language 

In the main menu, use Left and Right button to highlight Language item and then click OK button to go to the language setup interface. See Figure 3‑16. Please use Left and Right button to select the language you desire. 
Please note some series product support only one language. 

[image: image38.png]2009-08-05 17:27:48  + 00

Language

BRlight:Auto Video & 00:01:04 Card Free: 42 Min.




Figure 3‑16
3.8 Shut Down 

Please stop record operation first, and then in the main menu, use Left and Right button to highlight Shutdown item and then click OK button to turn off the PVR directly. See Figure 3‑17.
You can press the power button for a long time or cable control key to turn off the PVR.

[image: image39.png]2009-08-05 17:32:03 00

BRlight:Auto Video

Pouer-of £

W 00:00:00 Cord Free: 465 1B




Figure 3‑17
Note
Slight difference may be found in user interface.

All the designs and software here are subject to change without prior written notice.

Please visit our website for more information.





























































































































































































































































































IR light 





Indication light


Off


On


Camera





Microphone





Cable control buckle





Cable control port 





SD port 





External power port 




















Snapshot





OK  Right





Chassis up cover 





Battery cover





Chassis bottom cover





接口侧盖





Port side cover





Function key





Cable control slot











6             7             8           9        10           





          1            2          3        4            5                          





Power button














17





Mini USB port 





Up


Left


Down





Record





Cancel                





We assume no liability or responsibility for all the fires or electrical shock caused by unauthorized handling or installation.


 








PAGE  
4

