PVR210 User’s Manual
[image: image73.png]Paran. :Netuwork

Server url
Server IP 192 . 168 . 1 . 101

i]2[3]4]5]e]7 s ]5]

BRlight:n Video MOD:O000 nems Prev Info et





Version 1.3
Table of Contents

71
General Introduction


71.1
Features


71.2
Specifications


92
Framework


92.1
General Structure


92.2
Front Panel


102.3
Port


102.4
Cable Control Introduction


123
PVR Interface and Operation


123.1
Boot up and Shutdown


123.2
Standby and Wake up


123.3
Lock/Unlock


133.4
System Monitor and Management Interface


153.5
Record Operation


163.6
Main Menu


163.6.1
Playback


193.6.2
Snapshot


203.6.3
Parameter Setup


323.6.4
Format


333.6.5
Shut Down


34Appendix Toxic or Hazardous Materials or Elements




Welcome
Thank you for purchasing our PVR210 series product!

This user’s manual is designed to be a reference tool for the installation and operation of your system. 
Please read the following safeguard and warnings carefully before you use this series product!

Please keep this user’s manual well for future reference!

Safeguard and Warnings

1．Electrical safety
· All the installation and operations here should conform to your local electric safety rules.

· We assume no liability or responsibility for all the fires or electrical shock caused by unauthorized handling or installation.
2. Environments 
· Always transport, storage and install the device in the specified temperature and humidity environments. 

· Do not leave the PVR device in corrosive environment such as chemical factory.
3. Transportation and Installation 
· Please handle PVR device carefully

· This device shall avoid strike or strong vibration. PVR device may be damaged if you use or storage improperly.

4. Accessories 
· Be sure to use all the accessories recommended by the manufacturer.
· Do not expose the camera to strong light for a long time. It may damage the sensitive components such as the CMOS. 
5. SD card 
· Please pay attention to SD card installation direction. Otherwise it may result in SD card damage or PVR damage!
· Do not remove the SD card when you implement the SD card operation such as record, playback, format, delete, upgrade and etc. Otherwise it may result in SD card file damage!
· When system is upgrading, please make sure the battery power capacity is sufficient. Please connect the device to the power charger if possible. During the whole upgrade process; do not remove the SD card! Otherwise it may result in upgrade failure and system can not restore to the proper running status!
· Please make sure PVR device connection is right before you begin recording.
Warning 
· Do not dismantle PVR device without authorization!
· Improper battery use may result in fire, explosion, or personal injury! 

· When replace the battery, please make sure you are using the same model!
Some functions listed in this user’s manual are optional. Please make sure you purchased model supports current function before your operation!

1 General Introduction
1.1 Features
This portable video recorder (PVR) is law enforcement equipment for individual suitable for special industries such as police force, army force. It can completely record the law enforcement process and provide on-site audio record, video record, playback function. It is portable and is compatible with semi-stational use. PVR is very convenient to carry and is easy to operate.
1.2 Specifications
	Name 
	PVR210

	Video 
	Record resolution 
	Max 1024*768

	
	Compression algorithm
	H.264

	Audio 
	Compression mode 
	AAC

	Battery 
	Battery working period (Main unit only)
	For the general device, it is about 4.5 hours.
For the device of the network module, it is 2-3 hours. 

	
	Battery capacity threshold alarm
	When battery left power capacity is less than 10 minutes, the buzzer begins beeping regularly to alert you to recharge.

	
	LCD power management 
	System can automatically shut down LCD power after the PVR stopped operation for specified time such as ten seconds (adjustable).

	
	Standby time 
	About 40 hours

	Unit Port 
	Mini USB  port 
	USB2.0 high-speed port 

	
	1394b port 
	Connect the main unit to the cable control module.

	
	Recharge port 
	To recharge the device.

	
	SD card
	One SD card port

	
	Key 
	Nine keys (Including one on/off button and eight special keys.)

	Storage 
	SD card
	Support SD1.0, SD1.1, and SD2.0 standard.

	
	SD card alarm
	When the card is about to full (about five-minutes recording time left), you can hear buzzer beeps to alert you to replace a SD card. 

	Status Indication Light 
	Cable control light 
	· Monitor status: Cable control light flashes once in every three seconds.

· Record status：Cable control light is on.
· Pause status: Cable control light flashes once per second.
· Standby or stop status: Cable control lights all become off.

· Shut down mode: Cable control lights all become off.

	General 

Parameter 
	Working temperature 
	0℃－＋55℃

	
	Working humidity 
	10%－90%

	
	Dimensions (L*W*H)
	105mm×75mm×34mm

	
	Weight 
	0.14Kg(excluding battery and cable control )


2 Framework
2.1 General Structure
Please refer to Figure 2‑1 for general structure.
[image: image1.jpg]


[image: image71.jpg]E E sl onTrRoL”




[image: image72.jpg]



[image: image2.png]




Figure 2‑1
2.2 Front Panel 
Please refer to Figure 2‑2 for external keys.
There are nine keys. It includes one power button and eight special keys. 
[image: image3.png]it





Figure 2‑2
2.3 Port 
Please refer to Figure 2‑3 for external ports.
· Cable control port：Here you can input cable control to provide audio and video signal, the key control signal input and status indication light.
· SD card port：Input ordinary SD card. Please pay attention to SD card installation direction. It supports SD1.0, SD1.1 and SD 2.0 standard.

· External power port (it is round and has a socket in the middle.): It is to provide power to the device.
· Mini USB port: You can connect the device to the PC via this port. 

[image: image4.png]=D





Figure 2‑3
2.4 Cable Control Introduction 
Please refer to Figure 2‑4 for cable control information.
Cable control sub-system includes one camera, one microphone, two control keys and one status indication light. System provides analog audio and video signal input and keystroke control signal input. Status indication light can represent various statuses (On/Off/Flash. Please refer to Chapter 1.2 Specifications for detailed information)
[image: image5.jpg]



Figure 2‑4
About the cable control lens daily maintenance 
· Do not touch the camera of the cable control; you can use the blower to clean the dust on the surface of the lens. You can use the dry cloth with some alcohol or mild detergent to clear if necessary.
· Do not use the volatility solvent such as the benzene or thinner, or detergent with strong abradibility. It may result in lens damage or it may adversely affect the device performance. 
3 PVR Interface and Operation

3.1  Boot up and Shutdown
There are two ways for you to boot up the device.

· Press the Power button of the device for over two seconds, PVR system boots up.  System goes to real-time monitor status by default.
· Press the Record button of the cable control for over two seconds, PVR system boots up.  System goes to record status by default.

There are two ways for you to shutdown the device.
· When PVR is running properly, press the Power button for three seconds; you can turn off the PVR.  You can see shut down interface and then system begins three-second countdown. System stops internal operation (such as record or play) first and then shut down. Before the count down ending, you can release the Power button to terminate current shut down operation. 
· You can go to the menu and then shutdown the device via the system menu option. See chapter 3.6.5 Shutdown.
Note
When there is strong interference from the outside or there is invalid system operation, you can turn off the power and reload again.
3.2 Standby and Wake up
There are two ways for you to let the device standby.

· When PVR is running properly, press the Standby button of the cable control for three seconds; you can let the PVR standby.  You can see standby interface and then system begins three-second countdown. System stops internal operation (such as record or play) first and then become standby. Before the count down ending, you can release the standby button to terminate current standby operation. 

· You can go to the menu and then set the standby time via the system menu option. See chapter 3.6.3.2 Power setup.
There are two ways for you to wake up the device.
· When the device is standby, you can press the Power button at the front panel for three seconds to wake up the device. System goes to the real-time monitor status by default. 

· You can press the Record button of the cable control for over three seconds to go to the record status. 
3.3 Lock/Unlock 
Press the left/right button once respectively within two seconds, you can lock the device, Repeat the operation, you can unlock the device 

If you press any button when the device is locked, system pops up the following dialogue box. See Figure 3‑1.
[image: image6.png]2011-05-03 12:25:11 | Ep) 26

Keys are locked,Please press
() ESC’ and OK’ to unlock.

BB rress Esc to Exit

© camectors! keys axe wmlocked,
il local keys are Lockedt

Blight:n Video MOD:O0:00 nens Prev Tnfo e




Figure 3‑1
3.4 System Monitor and Management Interface

After the system booted up, press the Power button of the device for a long time, you can go to the monitor interface. See Figure 3‑2.
The white block at the bottom means you are in monitor interface.
[image: image7.jpg]BRlight:On Video WO0D:00:00 mer Prev Ing

10 e 12 13 14 15 16





Figure 3‑2
You can refer to the following sheet for detailed information. 
	SN
	Name 
	Function 

	1
	System time 
	Display system current date and time 

	2
	Network signal 
	It is to display current network signal intensity.

	3
	Network status 
	It is highlighted when the device connected to the network. Otherwise it is grey. 

	4
	Working status 
	You can see the corresponding icon when the device is playback, record or snapshot. 

	5
	SD card status
	This icon is grey when there is no SD card.
This icon becomes highlighted when there is SD card. 

	6
	Cable control status
	This icon is grey when there is no cable control.

This icon becomes highlighted when the cable control is available. 

	7
	Battery capacity
	When the battery left power capacity is less than 10 minutes, the buzzer begins beeping regularly to alert you to recharge.

	8
	/
	Interface operation prompt information. 

	9
	Camera zoom control bar 
	You can use the up/down button to adjust the zoom when you want to record or snapshot. “+” is to zoom in and the “-” is to zoom out. 

	10
	Screen backlight mode
	Press OK for a long time to switch.



	11
	Record type 
	There are two types: one is with audio/video together. One is with audio only. 

	12
	Record status
	· :Stop recording；
[image: image8.jpg]


 : Pause recording；
[image: image9.jpg]


 : System is recording now.

	13
	Menu 
	Click it to go to the main interface.

	14
	Preview 
	Click it to go to the audio/video setting interface (chapter 3.6.3.5). 

	15
	Information 
	Click it to go to the SD card and GPS information interface. 
· SD card interface is to display the SD card status and capacity. See Figure 3‑3.
· GPS interface is to display the device geographical information. See Figure 3‑4. 

	16
	Net 
	Click it to go to the network setup interface. 


The SD card interface is shown as in Figure 3‑3.
The SD card information includes the card status and information. When the device is recording, you can view the record remaining time here. In other statuses, you can view the total capacity and free capacity. 
[image: image10.png]2000-01-01 01:48:03 |

SD card state: Available.
Capacity: Total: 15,663MB.
Free: 1,835MB.

Back

BRLGht:(N Video MOD:O000 nemn Prev Info et




Figure 3‑3
The GPS interface is show as in Figure 3‑4.
Please note the GPS is the optional function. Please make sure your purchased model supports this function.
In this interface, you can view the information such as GPS status, GPS real-time latitude and longitude, altitude, satellite. 
[image: image11.png]2000-01-01 01:47:53

GPS Infornation
GPS state:Invalid

Latitude: 0° 0 9.000000 “
Longitude: 0° 0 ‘9.000000
Altitude: ©0.000000m

Working Satellite nunber: 0

Confirm[0K]  Exit[ESC]

BRMGht:(N Video MOD:O000 nens Prev Info et




Figure 3‑4
3.5 Record Operation

There are two ways for you to go to the record interface.

· You can click the Record button of the cable control when the device is off.

· After system boots up, please click Record button of the cable control or the Record button at the front panel to begin recording. 
The red icon at the bottom of the interface and the blue recorder icon at the top of the interface mean you are in record interface now. See Figure 3‑5.
You can refer to chapter 3.4 for general interface button information.

You can refer to the following sheet for detailed information. 

	Icon 
	Function 

	[image: image12.bmp]
	When the device is recording, press the Record button of the front panel, you can pause current record operation. You can see the icon [image: image13.bmp] at the bottom of the interface.

	[image: image14.bmp]
	In pause status, you can click Record button of the cable control or of the front panel to continue recording. You can see the icon [image: image15.bmp] at the bottom of the interface.

	[image: image16.bmp]
	In record or pause status, you can press ESC button to stop recording. You can see the icon[image: image17.bmp] at the bottom of the interface.


[image: image18.jpg]2009-08-05 17:16:50 [soRod )]

BRlight:Auto Video o 00:00:15 Card Free: 43 Min.




Figure 3‑5
System default recording package time is 60 minutes. When the file is about to full (about five-minutes recording time left), you can hear buzzer beeps to alert you to replace a SD card. 

All recorded files are automatically created and are storage under RECORD folder.

File name format is: Year_Month_Date_Hour_Minute_Second.mp4.
3.6 Main Menu 
Press OK button in the monitor interface, you can go to the main menu. The main menu buttons include playback, snapshot, parameter, and format and shut down. 

You can use Left/Right button to highlight the corresponding item and then click the OK button to go to the corresponding interface. See Figure 3‑6.
[image: image19.png]2011-05-03 12:22:20 | E) 200

&

Playback

BRlight:n Video MOD:00:00 nes Prev Tnfo et




Figure 3‑6
3.6.1 Playback 
Please highlight the Playback button and then click OK button to go to playback file list interface. See Figure 3‑7.

[image: image20.png]Files: 1/5 YR & B

record/2011_05_03/

2011 05 03 1223 32.3p4
2011 05 03 1225 10.3pa
2011 05 03 12 27_56.p4
2011 05 03 12_26_a7.pa

(© Press Fight! ey nore than 3 seconds
to enter file-deleting interface!




Figure 3‑7
Basic Information 

In playback file list interface you can view all recorded files in SD card RECORD folder. They are arranged in accordance with time. The latest file is on the top. System maximally displays 1024 files in one folder.
Detailed Operation 

Please use Up/Down to select playback file and then click OK button to view. System automatically plays the next file after completing current one. After it played all files in RECORD folder, system goes back to playback list interface (Figure 3‑7). 
In playback list interface, select one file and then click right button for a long time, you can go to remove file dialogue box. Please click OK button to delete current file or you can click ESC to exit current interface. 
Important

Please note you can not restore the deleted file if you remove here!
Playback control bar 

During the playback period, click OK button for a long time, you can see a playback control bar shown as in Figure 3‑8 . You can click Cancel button for a little bit while or press OK button for a long time to hide the playback control bar. 
[image: image21.png]2009-08-05 16:53:09 .

Detailed Info

00





Figure 3‑8


[image: image22.png][i] Ha > <> >l
- 00:00:10,/00:00:13 ¢





Figure 3‑9
Please refer to the following sheet for detailed information.
	Icon 
	Name 
	Function 

	[image: image23.jpg]



	Detailed information 
	Here you can view current playback file basic information. 

Click the cancel button for a short time; you can hide the detailed information.

	[image: image24.jpg]


/

[image: image25.jpg]



	Play previous or next file
	Please use Left and Right button to select [image: image26.jpg]


/[image: image27.jpg]


and then click OK button to confirm. System can playback the previous or next file. When the playback control bar is hidden, you can click Left or Right button once to play.

	[image: image28.jpg]£



/

[image: image29.jpg]



	Fast forward /fast backward
	Please use Left and Right button to select [image: image30.jpg]£



/[image: image31.jpg]


and then click OK button to confirm. Now you can realize fast forward or backward function. 
When the playback control bar is hidden, you can press Left or Right button for a long time to realize these two functions.

	[image: image32.jpg]



	Play
	You can use Left and Right button to select [image: image33.jpg]


 to restore playback. 

When the playback control bar is hidden, you can click OK button for a short while to realize pause and restore function.

	[image: image34.jpg]



	Pause
	When system is playback, please use Left and Right button to select [image: image35.jpg]


and then click OK button to confirm. Now you can see system is in pause mode.

When the playback control bar is hidden, you can click OK button for a short while to realize pause and restore function.

	[image: image36.jpg]



	Stop 
	Please use click Left or Right button to select  [image: image37.jpg]


 to stop playback. System goes back to file list interface (Figure 3‑7) after you stopped playback. 
When the playback control bar is hidden, you can press Cancel button for a short while to stop current playback.

	[image: image38.jpg]



	Accurate playback
	Please use Left or Right button to highlight [image: image39.jpg]


 and then click OK button to confirm. There is a red icon at the bottom of the playback bar.  See Figure 3‑11.You can use the Left or Right button to move the red icon, and then click OK button to confirm. 
Now you can see the playback symbol goes to the red icon position and then begin playback. 
You can press the Cancel button for a short while to exit current accurate playback.


[image: image40.png]02. 2011 0503 12 267 Ep &) (0 @)
>5Precs '(K' key to altemate Play and Pause
>5Precs '0K' key more than 3 secands o pop
playbeck nem

- 00:00:03/00:01:35 4y




Figure 3‑10
 [image: image41.png]02. 2011 0503 12 267 Ep &) (0 @)

o0:00:25

[E] W4 p¥ <4 pr
> 00:00:44/00:01:35 5 .,
T





Figure 3‑11
3.6.2 Snapshot 
In the main menu, select Snapshot item and then click OK button. You can see the image file list. See Figure 3‑12.
[image: image42.png]Files: 27?7 YR = B

picture/2011_05_03/

2011_05_05 123 11303

2011 05 03 123 06.9pg
2011 05 03 123 02.3pg
2011 05 03 12 38 00.3pg
2011 05 03 12 37_57.3p0
2011 05 03 12 37_53.9p0

(© Press Fight! ey nore than 3 seconds
to enter file-deleting interface!




Figure 3‑12
Basic Information 
In playback file list interface you can view all recorded images in SD card PICTURE folder. They are arranged in accordance with time. The latest file is on the top. System maximally displays 1024 files in one folder. 

Detailed Operation 

Please use the left/right button to switch between the file folders and file list frame. Please use Up and Down to select playback file or folder and then click OK button to go to the file folder or begin the playback. 
Press the OK button for a long time in the image browser interface; you can view the following control bar. You can click the Cancel button to hide the bar. See Figure 3‑13.
[image: image43.png]03. 2011 011523 4 7 @ 0 0 @0 0





Figure 3‑13
Please refer to the following sheet form detailed information. You can use the left/right button to select and then click the OK button to confirm current operation.
	Icon 
	Name 
	Function 

	[image: image44.jpg]



	Zoom 
	Zoom in to the original size or zoom out to suite the LCD size. 

You can use the direction keys (up/bottom/left/right) to view after you zoom in. 
You can use the OK button to view if the control bar is hidden. 
You can see the zoom icon at the top right corner after you click this button. 

	[image: image45.jpg]



	Detailed information 
	Here is to display the play file basic information. 

	[image: image46.jpg]


/[image: image47.jpg]



	Play previous/Play next 
	Play the previous file or play the next. 


3.6.3 Parameter Setup
Click OK button to go to main menu, highlight parameter item and then click OK to confirm, you can go to the parameter setup interface. See Figure 3‑14.

[image: image48.png]2009-08-05 17:03:27 00

BRlight:Auto Video

Lt

Parameters

W 00:00:00 Cord Free: 468 1B




Figure 3‑14
3.6.3.1 Time Setup
In Figure 3‑14, please use Left and Right button to highlight time item and then click OK button to go to time setup interface. See Figure 3‑15. Click Up and Down button to move the cursor to select year/month/date/hour/minute. You can use Left and Right button to increase or decrease number.  
Please use Up and Down button to move the cursor to highlight setup button and then click OK button to save current modification. 
You can click ESC button to go back to previous interface after you completed setup.

[image: image49.png]Paran. :Time

TR 206

Month
Day
Hour

Minute

ERlight:0n  Video

5
E)
12
39
V3

WOD:00:00 wew Prev Info Nt




Figure 3‑15
3.6.3.2 Power Setup
In Figure 3‑14, please use Left and Right button to highlight power item. Please click OK button to go to power setup interface. See Figure 3‑16.
Please refer to the following sheet for detailed information. 

	Name 
	Function 

	Sleep in battery mode 
	In battery power supply mode, system automatically standby time when there is no system operation (Exclude record or playback) for specified time.

	Standby in DC mode 
	In DC power supply mode, system automatically standby time when there is no system operation (Exclude record or playback) for specified time.  

	Backlight time
	The device can auto close the screen once it is backlight for the specified time.  

	Key auto lock
	When use lock function, lock function becomes active when there is no system operation (Exclude record or playback) for specified time.

 Please note cable control key will not be locked.


[image: image50.png]Battery
DC pover
Sleep Never
Backlight Never
Key Locked Never

BRlight:n Video MOD:O000 nems Prev Info et




Figure 3‑16
3.6.3.3 System Information and Firmware Upgrade
System Information 

In Figure 3‑14, please use Left and Right button to highlight System item and then click OK button to go to system setup interface. See Figure 3‑17.

Here you can view hardware version, software version and SD card capacity. In this interface, you can also upgrade system.

[image: image51.png]Param. :System Info. 7 E) [sof o Jm)

HU Version: PURZ10U_U1_12
SW Version: PURZ10 U0.01 S00002131
Camera Uersion: U1.00
SD Card Total: 15655444 KB

Used: 1657316 KB

BRlight:n Video MOD:O000 nems Prev Info et




Figure 3‑17
Restore factory default setup 

In the above interface, use the cursor to highlight the Restore button and then click the OK button, you can restore device configuration information to the factory default setup.

Firmware Upgrade

Please go to our website to get the latest upgrade software and then copy it to SD card UPDATE folder. Now you can insert the SD card into the PVR and then go to system setup interface. Click OK button, you can see system pops up a dialogue box asking you to confirm upgrade. Please follow the prompt to click OK button to begin upgrade. You can click Cancel button to exit.  
During the upgrade period, you can refer to the process bar for reference. System needs to restart after completing upgrade. Please note during the upgrade period, system stops all other operations. 
Warning!
· Before you upgrade, please make sure the battery capacity is sufficient. 
· To guarantee upgrade safety, please connect the device to the recharge. During the upgrade period, do not remove the SD card!

· During the whole upgrade period, insufficient battery capacity or SD card remove operation can result in system upgrade failure! System can not operate normally and it can not restore! 
3.6.3.4 Audio/Video
In Figure 3‑14, please use Left and Right button to highlight Audio/Video item and then click OK button to go to the following interface. See Figure 3‑18.
You can click the Preview button at the bottom of the interface to go to the following interface. 

Please use Left and Right button to select the corresponding file type.
Important!
· Please note you can see the corresponding prompting dialogue box if you want to set the video resolution, video frame rate, video bit rate, audio bit rate when the device is recording!

· If system is recording right now, your setup here can not be valid immediately. You need to stop the record first and then enable record again to activate current setup.
[image: image52.png]Param. :Record Mode | E) 2060
Record +ure KR

Audio:SampleRate 16K bps 46k
Video: Resolution 960x720

fps 25 bps z.0n
Record auto focus on

Photo focus node Manual
Sbstrem  Confim  Cancel  Restore

BRlight:n Video MOD:O000 nems Prev Info et




Figure 3‑18
[image: image53.png]Param. :Record Mode | E) 26
Substream

Resotut ion AR
fps Var.

bps 200k

Confirm  Cancel  Restore

Then ein strean resolution is 1024768,

the 03 of substresn vill be less than 15
BRLight:00 Video M 00:00:00 gery Prev Tnfo

e




Figure 3‑19
You can refer to the following sheet for detailed information. 

	Name 
	Function 

	Record type 
	There are two types: one is with audio/video and the other one is with the audio only. 

	Audio sampling rate 
	Right now the device support 16K only. 

	Audio frame rate 
	There are five options: 24k/32k/48k/64k/80k.

	Video resolution 
	There are five options: 320*240/640*480/704*576/960*720/1024*768.

	Frame rate 
	When the resolution is 320*240/960*720, the frame rate can be: VBR/3/5/6/8/10/12/15/16/18/20/22/25k/s.
When the resolution is 640*480/704*576, the frame rate can be：VBR/5/6/8/10/12/15/18/20/22/25/30.
When the resolution is 1024*768, the frame rate can be：VBR/3/4/5/6/7/8/9/10/11/12/13/14.

	Bit rate 
	The bit rate may vary due to different video resolution and frame rate setup. 

	Auto focus 
	There are two options: on/off. 

When this function is on, the device can automatically adjust the lens focus. 

When this function is off, you can use the up/down button to adjust the focus manually.

	Snapshot focus mode 
	There are two options: auto/manual. 

When in the auto mode, the device can automatically adjust the focus distance when snapshot. 

When in the manual mode, you can use the up/down button to adjust the focus manually.

	Extra stream 
	Click it to go to the extra stream setup interface.



	OK
	You can click the OK button to save your current setting after you complete the setup. Before your modification, the frame rate and bit rate will maintain the same. 

	Cancel 
	Click it to cancel your current setting.

	Restore 
	Here is to restore main stream factory default setup. Click the Restore button, you can see the system pops up a dialogue box asking you to restore factory default setup or not. Click OK button, system will begin restoring. 

Please note if the device is recording, system can not restore the factory default setup. You can see the corresponding dialogue box. 


The extra stream setup interface is shown as in Figure 3‑20.

[image: image54.png]Param. :Record Mode | E) 26
Substream

Resotut ion AR
fps Var.

bps 200k

Confirm  Cancel  Restore

Then ein strean resolution is 1024768,

the 03 of substresn vill be less than 15
BRLight:00 Video M 00:00:00 gery Prev Tnfo

e




Figure 3‑20
You can refer to the following sheet for detailed information. 

	Name 
	Function 

	Video resolution 
	There are three options: 320*240/640*480/704*576.

	Frame rate 
	The corresponding frame rate can be VBR/3/5/6/8/10/12/14/15/18/20/22/25.
Note:
The max extra stream frame rate is 14f/s when the main stream resolution is 1024x768.

	Bit rate 
	The bit rate may vary due to different video resolution and frame rate setup. 

	OK
	You can click the OK button to save your current setting after you complete the setup. Before your modification, the frame rate and bit rate will maintain the same. 

	Cancel 
	Click it to cancel your current setting.

	Restore 
	Here is to restore extra stream factory default setup. Click the Restore button, you can see the system pops up a dialogue box asking you to restore factory default setup or not. Click OK button, system will begin restoring. 

Please note if the device is recording, system can not restore the factory default setup. You can see the corresponding dialogue box. 


Important 
· In the record mode, you can not change the video resolution, video frame rate, video bit rate and audio bit rate. You can see the corresponding dialogue box.

· Please note if the device is recording, system can not restore the factory default setup. You can see the corresponding dialogue box.
3.6.3.5 Boot up Interface 
In Figure 3‑14, please use Left and Right button to highlight Boot up Interface item and then click OK button to go to the boot up dialogue box. You can set to use your self-defined boot up int6erface.
Click Cancel you can go to the previous interface, click Ok you can update the boot up interface.

Important

Before modification, Please copy the logo.bmp file (16-bit or 24-bit color, 360*240) to the UPDATE folder.

Please make sure the boot up file name is logo.bmp, other file names are invalid. 

3.6.3.6 Network Setup 

Important 

Before the operation, please make sure your device has the network module. 

In the Parameter interface, please highlight the network icon and then click the OK button to go to the network interface. 
3.6.3.6.1 WIFI Setup 

The WIFI setup interface is shown as in Figure 3‑21.
[image: image55.png]Paran. :Netuwork

Comnect.

TR 206

Info Register

Netuork Tupe

WIFT

SSID:LJ

State: Unconnected Online: 00:00:00

IP Conf igure

Advanced

Auto-start Net off

Conf igure

ELight:00  Video

WOD:00:00 wewy Prev Info Nt




Figure 3‑21
Please refer to the following sheet for detailed information. 

	Name
	Function

	Connection
	· It is to display the current network connection status：
1）SSID：It is the SSID of the router you are going to connect to.
2）State：Here you can see current network status. It includes two statuses: connected or disconnected.
3）Online time：It is to display the connection time.
· You can set the network parameter if necessary.
· Please refer to the following operation contents for detailed information.

	Information
	It is to display current network configuration information.

	Register
	· Register the device to the corresponding server according to your setup here.
· Please refer to the following operation contents for detailed information.


Detailed operation 
3.6.3.6.1.1 Connect
· Network type: WIFI.
· IP configure: Manual. 

Please highlight the Configure button and then click the OK button, the interface is shown as in Figure 3‑22. The IP address, subnet mask and the default gateway is to display the previous setup information. Please click the OK button at the item you want to set; you can see the system pops up a dialogue box for you to input the number for the IP address, subnet mask, default gateway and DNS. Click the Input button to verify the value. 

Please highlight the Confirm button and then click the OK to save current setup. 

Important 

The max value of the first byte of IP address of the subnet mask is 255. All the first byte of the IP address is 223. The max value of rest bytes is 255. Once you input is out of the threshold, system just saves the max value. 
[image: image56.png]Paran. :Netuork Yo B 2060
IP Address El s =
o

Netmask 7 W

Gateuay 0 .6

DNS Configure  Man.

Primary NS 192 . 168 . 1 . 248

Secondory DNS 192 . 168 . 1 . 249
Conf irn Cancel

BRLGht:(N Video MOD:O000 nems Prev Info et




Figure 3‑22
· Advanced 
Click the Advanced button in Figure 3‑21; you can go to the following interface. See Figure 3‑23. Please select a router SSID you want to connect from the list and then click the OK button. 
[image: image57.png]Paran. :Netuork Yo B 000 m

Direless nework  1/6

restlz

1
lboy
oL

Manual Cancel
You way selest & ssid in the List or input ssid namelly
BRLGht:(N Video MOD:O000 ners Prev Info e





Figure 3‑23
If current router has been encrypted, you need to input the corresponding password. See Figure 3‑24.
You can also input SSID, encrypt way, password manually and then click the OK button to save current setup. 
[image: image58.png]Encrypt Uay Open

Passuord pre—

V3

591D i beteen 1 and 32 characters

Passuord 1s blank

BRLGht:(N Video MOD:O000 nems Prev Info et




Figure 3‑24
· Connect
Click the Connect button in Figure 3‑21 to connect to the network. You can see the corresponding dialogue box. See Figure 3‑25.

[image: image59.png]Paran. :Netuwork

Comnect ULAN OKt

Conf irm[0K] Exit[ESC]

BRlight:n Video MOD:00:00 nes Prev Tnfo et




 Figure 3‑25
System will check the IP conflict occurred or not after WIFI connection. You can see the following dialogue box if the IP conflict occurred. See Figure 3‑26.

[image: image60.png]Param.:Network

206

IP conflict, discomnect net?

Yes[OK1 No[ESC1

BRUGht:(N Video MOD:O000 nens Prev Tnfo e




Figure 3‑26
· Disconnect
Click Disconnect button in Figure 3‑21 to disconnect to the WIF network. 
3.6.3.6.1.2 Information 

Here you can view the corresponding device network connection information of your setup. See Figure 3‑27.

[image: image61.png]Paran. :Netuwork

206

IP pddress 10.6.15.12
Netmask 255.255.0.0
Gateway 10.6.0.1
Primary DNS 192.168.1.248
Secondory DNS 192.168.1.249
MAC Address 00:25:£0:50:af :33

BRLGht:(N Video MOD:O000 nems Prev Info et




Figure 3‑27
3.6.3.6.1.3 Register 

Highlight the Register button and then click the OK, the interface is shown as below. See Figure 3‑28.
[image: image62.png]Paran. :Netuwork

Server IP 192 . 168 . 1 . 101
Server port 12366
Device 1D PUR-775591397
Active reg.  OFf

Conf irn Cancel

BRlight:n Video MOD:O000 nems Prev Info et




Figure 3‑28
Please refer to the following sheet for detailed information.
	Name 
	Function 

	Server domain name 
	You can input server domain name here. 

	Server IP
	You can input connection server IP here. 

	Server port 
	You can input server port here. 

	Device ID
	·  It is the unique symbol that the device registered to one media server. The
string format is: PVR-random number. 

·  You can modify if necessary. 

	Auto register 
	There are two options: On/off. 

On; The device can automatically register to the corresponding server. 

Off: The server connects to the device. 

	Confirm 
	You can click it to save current setup. 

It can remind you that current setup is old once you have not modified. 


3.6.3.6.2 About the input method

When you are input router password, set server domain name and the device ID, system adopts the T9 input method. You can see the system pops up the following dialogue box. See Figure 3‑29. Please use the direction keys to select the character and then click the OK button to confirm current option. You can see the selected character displayed in the No1 position. For other input interface, you can see a dialogue box on the right pane of the Figure 3‑29.
[image: image63.png]123

[ oo TN 3/ghi >

4/jk1 S/mo  6/pars «
tw  Baxyz  9/Cancel  Delete
Sign ) Suitch  Enter

BRlight:n Video MOD:O000 nems Prev Info et




Figure 3‑29
You can refer to the following sheet for detailed information. 
	Name 
	Function 



	Switch

 (Input method)
	It includes four options: Chinese, capitalize English character, small English character, number. 

You can click to switch. 

In Figure 3‑29, you can see the current input mode at number 2 position. 

	Character information
	It is to prompt the input character and remaining character amount. 



	Sign 
	Click it to switch between the current input mode and the sign. 

When you are in sign input mode, you can see the corresponding symbol bar at the number 3 position.

	Cancel 
	It is only valid in Pinyin mode (For Chinese character input only). It can remove current input Pinyin and then input again. 

	<<
	Previous page 

	>>
	Next page 

	Delete 
	Click it to remove the last input character. It can delete two bytes in Pinyin mode and it can delete one byte in other input mode. 

	Enter 
	Output the current input contents to the above layer. 


3.6.3.6.3 3G Network 

The 3G network interface is shown as in Figure 3‑30.

[image: image64.png]Paran. :Netuwork

Comnect.

TR 206

Info Register

Netuork Tupe

36

Netuork card type:EUDD
State: Unconnected Online: 00:00:00

IP Conf igure

Advanced

Auto-start Net off

Conf igure

ELight:00  Video

WOD:00:00 wewy Prev Info Nt




Figure 3‑30
Please refer to the following sheet for detailed information. 

	Name 
	Function 

	Connect 
	Display current 3G network connection status.

· Network card type: There are two options: EVDO/WCDMA according to the different 3G module. 

· State: Current network connection status. There are two options: connect/disconnect. 

· Online time: Here you can view network connection time. 

You can set the network. You do not need to set the IP address.

Please refer to the following contents for detailed operation information. 

	Information 
	You can see current network setup information. 

	Register 
	· You can set the setup information so that the device can connect to the corresponding server. 
· Please refer to the following contents for detailed operation information.


3.6.3.6.3.1 Connect 
· Network type: 3G.

· IP configure: You do not need to set. The IP address, subnet mask, gateway, DNS parameter and etc can get from the operator after you successfully connected to the 3G network. 
· Advanced 

Highlight the Advanced button and then click the Confirm button; you can go to the 3G advanced setup. Please click the Confirm to save current setup. See Figure 3‑31.
[image: image65.png]Dial Number. #m

Usernane card
Passuord card
Confirm Cancel

BRLGht:(N Video MOD:O000 nems Prev Info et




Figure 3‑31
Please refer to the following sheet for detailed information. 

	Name 
	Function 

	Extra command 
	You can input the additional 3G dial command manually.  

	Dial number 
	You can input dial number manually.

	User name and password 
	The user name and password you get from your operator. It can be modified if necessary. 


· Connect
Click the Connect button in Figure 3‑30 to connect to the network. You can see the corresponding dialogue box. See Figure 3‑32.

[image: image66.png]Comnecting network succeeded?

Conf irm[0K] Exit[ESC]

BRUGht:(N Video MOD:O000 nens Prev Tnfo e




 Figure 3‑32
If the SIM card is not proper, system will display the following dialogue. See Figure 3‑33.
You can see the system can not connect to the 3G network. 
[image: image67.png]Paran. :Netuork Yo B [sofoJa}

: SIN card does not exist!

Conf irm[0K] Exit[ESC]

BRUGht:(N Video MOD:O000 nens Prev Tnfo e




Figure 3‑33
· Disconnect
Click Disconnect button in Figure 3‑30 to disconnect to the 3G network. 
3.6.3.6.3.2 Information 

You can view the corresponding IP address, subnet mask, gateway, DNS information from the operator after successful network connection. 

3.6.3.6.3.3 Register 

Please refer to the register section of the WIFI setup.  
3.6.4 Format
In the main menu, select Format item and then click OK button. See Figure 3‑34.You can go to the format interface. Please follow the prompt information to click OK button to begin format process. You can click Cancel button to go back to previous interface.
[image: image68.png]2011-05-04 11:54:03 | E) 006 m

Formatting

BRlight:n Video MOD:00:00 nes Prev Tnfo et




Figure 3‑34
Important!
All data in the CS card will be removed completely and can not be restored!

Before format operation, please make sure you have backup all important files and data!

3.6.5  Shut Down 
Please stop record operation first, and then in the main menu, use Left and Right button to highlight Shutdown item and then click OK button to turn off the PVR directly. See Figure 3‑35.
You can press the power button for a long time or cable control key to turn off the PVR.

[image: image69.png]2011-05-04 11:55:38 | E) 006 m

o

Pouer-of £

BRlight:n Video MOD:00:00 nes Prev Tnfo et




Figure 3‑35
Appendix Toxic or Hazardous Materials or Elements
	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Construction Material
	○
	○
	○
	○
	○
	○

	Circuit Board
	○
	○
	○
	○
	○
	○

	Internal Wire/Cable
	○
	○
	○
	○
	○
	○

	Cable Control Lens
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


Note

O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.

X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. 

Note:

· This manual is for reference only. Slight difference may be found in the user interface.
· All the designs and software here are subject to change without prior written notice. 
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website or contact your local retailer for more information.
[image: image70.jpg]
































































































































































































































































































IR light 





Indication light


Off


On


Camera





Microphone





Cable control buckle





Cable control port 





SD port 





External power port 




















Snapshot





OK  Right





Chassis up cover 





Battery cover





Chassis bottom cover





接口侧盖





Port side cover





Function key





Cable control slot





②





③





6             7             8           9        10           





          1            2          3        4            5                          





Power button














17





Mini USB port 





Up


Left


Down





Record





Cancel                





①








PAGE  
3

