1 Megapixel HDCVI Mini IR Vandal-proof Dome Camera

User's Manual

Version 1.0.1

Table of Contents

1	General Introduction	1

1.1	Overview	1

1.2	Features	1

1.3	Functions	1

1.4	Specifications	2

2	Framework and Dimensions	3

3	Installation	4

Appendix Toxic or Hazardous Materials or Elements	6

Welcome

Thank you for purchasing our HDCVI camera!

This user’s manual is designed to be a reference tool for your system.

Please read the following safeguard and warnings carefully before you use this series product!

Please keep this user’s manual well for future reference!

Important Safeguards and Warnings

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes.

The power shall conform to the requirement in the SELV (Safety Extra Low Voltage) and the Limited power source is rated 12V DC (24V AC) in the IEC60950-1.

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation.

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Do not apply power to the camera before completing installation.

Please install the proper power cut-off device during the installation connection.

Always follow the instruction guide the manufacturer recommended.

If this product is installed in the ceiling, please make sure the installation position can sustain the min 50N.

4．Qualified engineers needed

All the examination and repair work should be done by the qualified service engineers.

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

This series camera should be installed in a cool, dry place away from direct sunlight or strong light, inflammable, explosive substances and etc.

This series camera shall work on the specified working temperature. Please keep it away from the electromagnetic radiation object and environment.

Please keep the sound ventilation.

Do not allow the water and other liquid falling into the camera.

6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.

7.   Daily Maintenance

Please shut down the device and then unplug the power cable before you begin daily maintenance work.

Use the dry soft cloth to clean the device.

If there is too much dust, please use the water to dilute the mild detergent first and then use it to clean the device. Finally use the dry cloth to clean the device.

Please put the dustproof cap to protect the CCD (CMOS) component when you do not use the camera.

# General Introduction

## Overview

This series megapixel HD camera conforms to the HDCVI standard. It supports video signal high-speed long distance transmission without any delay. It can be controlled by the DVR conforming to the HDCVI.

## Features

- High-performance CMOS image sensor, megapixel definition.

- Support HDCVI video output.

- Support 75-3 coaxial cable transmission without any loss. Transmission distance is over 500m.

- High speed, long distance real-time transmission.

- Support ICR switch to realize surveillance both in the daytime and at night.

- Support auto exposure, auto white balance, auto electronic shutter and auto gain function.

- Support DC12V power supplying.

- Support IP66, IK10.

- Support intelligent IR function.

## Functions

HDCVI Specification

HDCVI（High Definition Composite Video Interface） is an over-coaxial –cable HD video transmission standard. The technology renders two HD video formats by progressive scanning.

ICR

The IR cut removal is to filter the IR light in the daytime and then auto switch to the general fitter at night. This function allows the camera to output the high sensitivity and clear video.

Smart IR technology

The sensor controls the IR light on/off via the combination work of the hardware and software, which realizes the automatically IR light compensation according to the environment illumination.

Auto gain function

To output the standard video signal in the different illumination environments, the amplifier needs to adjust in a wide range. The system can enhance the camera sensitivity in low illumination and enhance the video signal output to get the clear and high definition video.

Auto white balance

The white balance refers to the camera to restore the white object color. It allows the camera to automatically adjust the color temperature in indoor and outdoor environment, just like our human eyes does.

Auto exposure

System can automatically set shutter speed and iris value according to the snapshot video exposure condition.

Auto electronic shutter

The system can automatically adjust the electronic shutter when the environment light changes.

## Specifications

| Model Parameter | 1 Megapixel HDCVI Mini IR Vandal-proof Dome Camera Series |
| --- | --- |
|  | HAC-HDBW1100EP/N |
| Video Processor | 1/2.9" 1.0 Mega Pixels CMOS |
| Video Format | PAL/NTSC |
| Effective Pixel | 1280（H）* 720（V） |
| Min Illumination | 0.01Lux @（F1.2,AGC ON），0Lux with IR |
| Electronic Shutter | PAL：1/50s~1/100,000s NTSC：1/60s~1/100,000s |
| Video Standard | HDCVI compliance |
| Video Frame Rate | PAL：1280×720@25 fps NTSC：1280×720@30 fps |
| Lens Port | M12 |
| Lens Type | 2.8/3.6/6/8mm optional |
| Day/Night Switch | ICR auto switch |
| Sync Mode | INT |
| Video Output | 1-channel HDCVI high definition video output |
| White Balance | Auto |
| Gain Control | Auto |
| BLC | Auto |
| Transmission Distance | Over 500m via 75-3 coaxial cable. |
| Smart IR | Support |
| Max IR Distance | 20m |
| Protection Level | IP66 |
| IK Rating | IK10 |
| Working Temperature /Humidity | -30℃~+60℃. Humidity is less than 95%(no condensation) |
| Power | DC 12V±10% |
| Power Consumption | 2.5W MAX (Input: 12V0.21A) |
| Dimension(mm) | φ110×81 |
| Weight | 300g |
| Installation Mode | Wall mount/In-ceiling mount |

# Framework and Dimensions

Please refer to the following figures for dimension information. The unit is mm. See Figure 2-1 to Figure 2-3.

Figure 2-1

Figure 2-2

Figure 2-3

# Installation

Important

- Before the installation, please make sure the installation surface can sustain at least 3X weight of the bracket and the camera.

The dome camera usually uses the ceiling installation. It can be installed on the ceiling or the wall.

- For side exit cable installation, please make sure the side exit cable direction is the same as shown in the installation position map.

- Before you secure the screws on the pedestal, please put the cable through the cable exit of the pedestal.

- Before you complete the installation and debug, do not remove the electrostatic film. After you remove the film, do not touch the enclosure in case there is any scratch.

Figure 3-1

Step 1

Take the installation position map from the accessories bag and then put it on the surface of the ceiling or the wall according to the cable exit position. Dig three holes according to the installation position map and then take three expansion bolts from the accessories bag and put them to the holes you just dug.

Step 2

Use inner hex wrench to remove the three enclosure fastening screws and then remove the dome enclosure.

Step 3

Adjust the pedestal position according to the cable exit mode (ceiling/side). Pull the cable through the cable side exit at the installation surface and the pedestal (Please skip this step if you want to use ceiling exit mode). Line up the screw holes at the bottom of the pedestal to the expansion bolts in the installation surface. Put three self-tapping screws to expansion bolts and then secure firmly. Now the pedestal is on the installation surface.

Step 4

Adjust the camera module angle to a proper position and then fix the screws to fasten.

Figure 3-2

Step 5

Use three enclosure fastening screws to secure the dome housing on the pedestal.

Now you have completed the device installation and cable connection.

You can use the terminal encode devices to view the monitor video.

Attention:

Original device supports IP66, pay attention to fasten screws after installation to ensure that the dome cover and pedestal are waterproof, which is to avoid water leakage due to improper installation.

# Appendix Toxic or Hazardous Materials or Elements

| Component Name | Toxic or Hazardous Materials or Elements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pb | Hg | Cd | Cr VI | PBB | PBDE |
| Circuit Board Component | ○ | ○ | ○ | ○ | ○ | ○ |
| Device Construction Material | ○ | ○ | ○ | ○ | ○ | ○ |
| Wire and Cable | ○ | ○ | ○ | ○ | ○ | ○ |
| Packing Components | ○ | ○ | ○ | ○ | ○ | ○ |
| Accessories | ○ | ○ | ○ | ○ | ○ | ○ |

O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.

X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets. The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.

Note

- This manual is for reference only. Slight difference may be found in the user interface.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks mentioned are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website or contact your local service engineer for more information.