1 Megapixel Mini IR Waterproof Fixed HDCVI Camera

User’s Manual

Version 1.0.0

Table of Contents

1	General Introduction	1

1.1	Overview	1

1.2	Features	1

1.3	Functions	1

1.4	Specifications	2

2	Framework and Dimensions	4

2.1	Dimensions	4

2.2	Structure	4

3	Installation	5

Appendix Toxic or Hazardous Materials or Elements	7

Welcome

Thank you for purchasing our analog camera!

This user’s manual is designed to be a reference tool for your system.

Please read the following safeguard and warnings carefully before you use this series product!

Please keep this user’s manual well for future reference!

Important Safeguards and Warnings

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes.

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation.

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Do not apply power to the camera before completing installation.

Always follow the instruction guide the manufacturer recommended.

4．Qualified engineers needed

All the examination and repair work should be done by the qualified service engineers.

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

This series analog camera should be installed in a cool, dry place away from direct sunlight or strong light, inflammable, explosive substances and etc.

This series camera shall work on the specified working temperature.  Please keep it away from the electromagnetic radiation object and environment.

Please keep the sound ventilation.

Do not allow the water and other liquid falling into the camera.

6.  Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.

7.   Daily Maintenance

Please shut down the device and then unplug the power cable before you begin daily maintenance work.

# General Introduction

## Overview

This series megapixel HD camera conforms to the HDCVI standard. It supports video signal high-speed long distance transmission without any delay. It can be controlled by the DVR conforming to the HDCVI.

## Features

- High-performance CMOS image sensor, megapixel definition.

- Support HDCVI video output.

- Support 75-3 coaxial cable transmission without any loss. Transmission distance is over 500m.

- High speed, long distance real-time transmission.

- Support ICR switch to realize surveillance both in the daytime and at night.

- Support auto exposure, auto white balance, auto electronic shutter and auto gain function.

- Support DC12V power supplying.

- IP66 compliance.

- Support intelligent IR function.

## Functions

HDCVI Specification

HDCVI（High Definition Composite Video Interface） is an over-coaxial –cable HD video transmission standard. The technology renders two HD video formats by progressive scanning.

ICR

The IR cut removal is to filter the IR light in the daytime and then auto switch to the general fitter at night. This function allows the camera to output the high sensitivity and clear video.

Smart IR technology

The sensor controls the IR light on/off via the combination work of the hardware and software, which realizes the automatically IR light compensation according to the environment illumination.

Auto gain function

To output the standard video signal in the different illumination environments, the amplifier needs to adjust in a wide range. The system can enhance the camera sensitivity in low illumination and enhance the video signal output to get the clear and high definition video.

Auto white balance

The white balance refers to the camera to restore the white object color. It allows the camera to automatically adjust the color temperature in indoor and outdoor environment, just like our human eyes does.

Auto exposure

System can automatically set shutter speed and iris value according to the snapshot video exposure condition.

Auto electronic shutter

The system can automatically adjust the electronic shutter when the environment light changes.

## Specifications

| Model Parameter | 1 Megapixel Mini IR Waterproof Fixed HDCVI Camera |
| --- | --- |
|  | HAC-HFW1100EP/N |
| Video Processor | 1/2.9" 1.0 Mega Pixels CMOS |
| Video Format | PAL/NTSC |
| Effective Pixel | 1280（H）* 720（V） |
| Min Illumination | 0.01Lux @（F1.2,AGC ON），0Lux with IR |
| Electronic Shutter | PAL：1/50s~1/100,000s NTSC：1/60s~1/100,000s |
| Video Standard | HDCVI compliance |
| Video Frame Rate | PAL：1280×720@25 fps NTSC：1280×720@30 fps |
| Lens Port | M12 |
| Lens Type | 2.8/3.6/6/8mm optional |
| Day/Night Switch | ICR auto switch |
| Sync Mode | INT |
| Video Output | 1-channel HDCVI high definition video output |
| White Balance | Auto |
| Gain Control | Auto |
| BLC | Auto |
| IR Light Control Plan | Advanced non-photosensor control plan |
| Transmission Distance | Over 500m via 75-3 coaxial cable. |
| Smart IR | Support |
| Max IR Distance | 20M |
| Protection Level | IP66 |
| Working Temperature /Humidity | -30℃~+60℃. Humidity is less than 95%(no condensation) |
| Power | DC 12V±10% |
| Power Consumption | 3.5W MAX (Input: 12V0.29A) |
| Dimension(mm) | 69.9×178.4×70 (Provide internal cable exit bracket ) |
| Angle Adjustment | Pan: 0°～360°: Tilt: 0°～90°: Flip: 0°～360° |
| Weight | 500g |
| Installation Mode | Wall mount |

# Framework and Dimensions

## Dimensions

Please refer to the following for dimension information. The unit is mm. See Figure 2-1.

Figure 2-1

## Structure

Please refer to the following figure for structure information. See Figure 2-2.

Figure 2-2

Please refer to the following sheet for detailed information.

| SN | Name | Function |
| --- | --- | --- |
| 1 | Lens | It is to receive the optical signal from the environments. |
| 2 | Sunshield Cover | / |
| 3 | Bracket | Support camera body and rotate to adjust monitor angle. |
| 4 | Power input port | Connect to the DC 12V power to input the power. |
| 5 | Video output port | BNC port is to output HDCVI video signal. You can connect to the devices such as the DVR or the NVS conforming to the HDCVI specifications. |

# Installation

Important

- Before the installation, please make sure the installation surface can sustain at least 3X weight of the bracket and the camera.

Please refer to the steps listed below for installation information.

Figure 3-3

Step 1

Install the camera bracket according to your installation mode.

Before you install the bracket, please pull the cable through the cable exit of the installation surface or pull the cable through the cable exit of the bracket.

- Please install the expansion bolt if you want to install in the cement wall (Please make sure the installation holes of the expansion bolts are the same with the bracket.) Then you can install the bracket. See Figure 3-3.

- If you want to install in the wood surface, please skip the first step and then use the self-tapping screws to install the bracket directly.

Step 2

Put the two self-tapping screws to the two expansion blots at the top left and the top right. Secure the bracket on the self-tapping screws. See (A) in Figure 3-3.

Important

The screw depth shall reserve 8mm-10mm to secure the bracket.

Step 3

Refer to Figure 3-3 to fix other two self-tapping screws and the secure. Now the camera is fixed on the installation surface.

Step 4

Use the L wrench (provided) to loosen the adjusting screw to make the camera can monitor the corresponding surveillance zone. Use the L wrench to adjust the adjusting screw to fix the camera firmly. Please refer to (B) and (C) in Figure 3-3.

Step 5

Connect the video output port of the device cable to the rear-end encode device such as DVR or NVS. Then connect the power port of the device cable to the corresponding power supplying source.

# Appendix Toxic or Hazardous Materials or Elements

| Component Name | Toxic or Hazardous Materials or Elements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pb | Hg | Cd | Cr VI | PBB | PBDE |
| Circuit Board Component | ○ | ○ | ○ | ○ | ○ | ○ |
| Device Construction Material | ○ | ○ | ○ | ○ | ○ | ○ |
| Wire and Cable | ○ | ○ | ○ | ○ | ○ | ○ |
| Packing Components | ○ | ○ | ○ | ○ | ○ | ○ |
| Accessories | ○ | ○ | ○ | ○ | ○ | ○ |

O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.

X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets. The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.

Note

- This manual is for reference only. Slight difference may be found in the user interface.

- All the designs and software here are subject to change without prior written notice.

- All trademarks and registered trademarks are the properties of their respective owners.

- If there is any uncertainty or controversy, please refer to the final explanation of us.

- Please visit our website or contact your local service engineer for more information.