
HD Network Dome Camera User’s Manual
Version 1.2.3
Welcome

Thank you for purchasing our network camera!

This user’s manual is designed to be a reference tool for your system. 

Please read the following safeguard and warnings carefully before you use this series product!
Please keep this user’s manual well for future reference!
Important Safeguards and Warnings 

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

The power shall conform to the requirement in the SELV (Safety Extra Low Voltage) and the Limited power source is rated 12V DC or 24V AC in the IEC60950-1. (Refer to general introduction) Please note: Do not connect two power supplying sources to the device at the same time; it may result in device damage! The product must be grounded to reduce the risk of electronic shock.
We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

We are not liable for any problems caused by unauthorized modification or attempted repair.

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Do not apply power to the camera before completing installation. 

Please install the proper power cut-off device during the installation connection.

Always follow the instruction guide the manufacturer recommended.

4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

This series network camera should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

Please keep it away from the electromagnetic radiation object and environment.

Please make sure the CCD (CMOS) component is out of the radiation of the laser beam device. Otherwise it may result in CCD (CMOS) optical component damage.

Please keep the sound ventilation. 

Do not allow the water and other liquid falling into the camera.

Thunder-proof device is recommended to be adopted to better prevent thunder.

The grounding studs of the product are recommended to be grounded to further enhance the reliability of the camera.

6. Daily Maintenance

Please shut down the device and then unplug the power cable before you begin daily maintenance work. 

Do not touch the CCD (CMOS) optic component. You can use the blower to clean the dust on the lens surface.

Always use the dry soft cloth to clean the device.  If there is too much dust, please use the water to dilute the mild detergent first and then use it to clean the device. Finally use the dry cloth to clean the device.

Please put the dustproof cap to protect the CCD (CMOS) component when you do not use the camera.

Dome enclosure is the optical component, do not touch the enclosure when you are installing the device or clean the enclosure when you are doing maintenance work. Please use professional optical clean method to clean the enclosure. Improper enclosure clean method (such as use cloth) may result in poor IR effect of camera with IR function.  

7. Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.

	Accessory Name
	Amount

	Network Camera
	1

	Quick Start Guide
	1

	Installation Accessories Bag 
	1

	Installation Position Diagram
	1

	CD
	1


Table of Contents 

11
General Introduction


11.1
Overview


11.2
Features


21.3
Specifications


21.3.1
Performance


102
Structure


102.1
Dimensions


112.2
Port Description


132.3
Listening


142.4
Bidirectional talk


142.4.1
Device-end to PC-end


142.4.2
PC-end to the Device-end


142.5
Alarm Setup


173
Installation


173.1
Device Installation Introduction


173.2
Device Installation Steps


173.2.1
General Installation


203.2.2
Manual Zoom Lens Focus Operation


213.2.3
Side Cable Exit


213.2.4
Cable Connection


223.3
Micro SD Card Installation


244
Quick Configuration Tool


244.1
Overview


244.2
Operation


265
Web Operation


265.1
Network Connection


265.2
Login and Main Interface


286
FAQ


29Appendix Toxic or Hazardous Materials or Elements





1 General Introduction 
1.1 Overview 
This series network camera combines traditional camera and network video technology into one unit which integrates A/V data collection and transmission through network without aux devices. It uses advanced H.264 and MJPEG video compression technology and G.711a audio compression technology, which maximally guarantee the audio and video quality.
This series network camera enclosure has the strong resistance capacity, which can guarantee the proper work performance under heavy strike. It supports real-time monitor and listening at the same time. It supports analog video output and dual-way bidirectional talk. 
It can be used alone or used in a network area. When it is used lonely, you can connect it to the network and then use a network client-end. Due to its multiple functions and various uses, this series network camera is widely used in many environments such home, office, bank, road monitor and etc.
1.2 Features
	User Management
	· Different user rights for each group, one user belongs to one group.

· The user right shall not exceed the group right.
· You can edit each group at anytime.

	Storage
Function
	· Support central server backup function in accordance with your configuration and setup in alarm or schedule setting

· Support record via Web and the recorded file are storage in the client-end PC.
· Support built-in Micro SD card.
· Support network storage like FTP, support short-time storage when encounter disconnection.

	Alarm Function
	· Real-time respond to external local alarm input and video detect as user pre-defined activation setup and exert corresponding message in screen and audio prompt(allow user to pre-record audio file)

· Real-time video detect: motion detect, tampering.
· Can generate an alarm when network abnormal, Micro SD card abnormal event occurred.

	Network Monitor 
	· Network camera supports one-channel audio/video data transmit to network terminal and then decode. Delay is within 270ms (network bandwidth support needed)

· Max supports 20 connections.
· Adopt the following audio and video transmission protocol: HTTP, TCP, UDP, MULTICAST, RTP/RTCP, RTSP and etc.
· Support web access, widely used in WAN.

	Network Management
	· Realize network camera configuration and management via Ethernet.
· Support device management via web or client-end.  

· Support various network protocols.

	Peripheral Equipment 
	· Support the on-off alarm device to alarm via the sound or the light.

	Power 
	· External power adapter DC12V/AC 24V (Only vandal proof series support AC 24V)
· Support PoE. 
Warning！
Do not connect two power supplying sources to the device at the same time; it may result in device damage!

	Assistant Function
	· Log function
· Support PAL/NTSC
· Day/Night mode auto switch (electromagnetic ICR switch).
· Auto iris adjustment
· Built-in IR light. Support IR night vision (For IR Series only).
· Backlight compensation: screen auto split to realize backlight compensation to adjust the bright.
· Support electronic shutter and gain setup.
· Support 3D NR.
· Support video watermark function to avoid vicious video modification.


1.3 Specifications 
1.3.1 Performance 
Please refer to the following sheets for network camera performance specification.
	[image: image27.jpg]E E sl onTrRoL”




                              Model

Parameter
	IPC-HD/HDW3100 Series
	IPC-HD/HDW3200 Series

	System 
	Main Processor 
	TI Davinci high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network monitoring, local record, and remote operation. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3 inch CMOS
	1/2.8 inch CMOS

	
	Pixel
	1280(H)*960(V)
	1920 (H)*1080(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC Mode
	On/Off

	
	Electronic Shutter 
	Manual/Auto
It ranges from 1/3 to 1/10000.

	
	Video Compression Standard
	H.264 / H.264H/H.264B/MJPEG
Note: Some versions do not support H.264H.

	
	Video Frame Rate
	PAL:
Main stream (1280*960@15fps)

extra stream (704*576@15fps)/
Main stream (1280*720@25fps)

Extra stream (704*576@25fps)

NTSC:
Main stream (1280*960@15fps)

extra stream (704*480@15fps)
 Main stream (1280*720@30fps)

Extra stream (704*480@30fps)
	Main stream (1920*1080@25fps)

extra stream (704*576@25fps)

NTSC:
Main stream (1920*1080@30fps)

extra stream (704*480@30fps)



	
	Video Bit Rate
	H.264: 56Kbps-6144Kbps. It is adjustable

MJPEG: adjustable and bit rate is adjustable.
Support customized setup. 
	H.264: 56Kbps-8192Kbps. It is adjustable

MJPEG: adjustable and bit rate is adjustable.
Support customized setup. 

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as bright, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	2.8~12mm@F1.4
	3.3~12mm@F1.4

	
	Lens Interface
	¢14. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe RCA LINE IN 

	
	Audio Output 
	1-channel RCA LINE OUT

	
	Audio Bit Rate
	16kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 
Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Privacy Mask
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card storage, NAS storage

	  Network 
	Wired Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPoE,IPv4/v6,SNMP,QoS,UPnP,NTP, HTTP, 802.1x, Bonjour

	
	Protocol
	PSIA (optional), ONVIF

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	 AUX Interface
	Video Output
	N/A
	1-ch analog output, BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 meters (For IR series product only)

	 General Parameter
	Power
	Support DC12V and PoE 

	
	Power Consumption
	General series: below 3W. 
IR series: below 5W.
	General series: below 5W. 
IR series: below 7W.

	
	Working Temperature
	-20℃～+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢140.3×113.7

	
	Weight
	0.5 kg

	
	Installation
	Support various installation modes


	[image: image28.jpg]


                              Model

Parameter
	IPC-HDB/HDBW3100 Series
	IPC-HDB/HDBW3200 Series
	IPC-HDB/HDBW3301 Series

	System 
	Main Processor 
	TI Davinci high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network monitoring, local record, and remote operation. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3 inch CMOS
	1/2.8 inch CMOS
	1/3 inch CMOS

	
	Pixel
	1280(H)*960(V)
	1920 (H)*1080(V)
	2048(H)*1536(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto

It ranges from 1/3 to 1/10000.

	
	Video Compression Standard
	H.264 / H.264H/H.264B/MJPEG
Note: Some versions do not support H.264H.

	
	Video Frame Rate
	PAL:
Main stream (1280*960@15fps)
extra stream (704*576@15fps)
Main stream (1280*720@25fps)
extra stream (704*576@25fps)

NTSC:
Main stream (1280*960@15fps)
extra stream (704*480@15fps)
Main stream (1280*720@30fps)
extra stream (704*480@30fps)
	PAL:
Main stream (1920*1080@25fps)
extra stream (704*576@25fps)
NTSC:
Main stream (1920*1080@30fps)
extra stream (704*480@30fps)

	PAL:
Main stream (2048*1536@15fps)
extra stream (704*576@15fps)

Main stream (1920*1080@25fps)
extra stream (704*576@25fps)
NTSC:
Main stream (2048*1536@15fps)
extra stream (704*480@15fps)
Main stream (1920*1080@30fps)
extra stream (704*480@30fps)


	
	Video Bit Rate
	H.264: 56Kbps-6144Kbps. It is adjustable
	H.264: 56Kbps-8192Kbps. It is adjustable 

	
	
	MJPEG is adjustable. Bit rate is adjustable and support customization.

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as bright, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	2.8~12mm@F1.4
	3.3~12mm@F1.4

	
	Lens Interface
	¢14. Lens is the default accessories.


	    Audio 
	Audio Input
	1-channe RCA LINE IN 

	
	Audio Output 
	1-channel RCA LINE OUT

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	16kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 
Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Privacy Mask
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card storage, NAS storage

	  Network 
	Wired Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPoE,IPv4/v6,SNMP,QoS,UPnP,NTP, HTTP, 802.1x, Bonjour

	
	Protocol
	PSIA (optional), ONVIF

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	 AUX Interface
	Video Output
	N/A
	1-ch analog output, BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 meters (For IR series product only)
	IR distance 10~20 meters (For IR series product only)

	 General Parameter
	Power
	Support DC12V and PoE 
	Support AC 24V/DC12V and PoE

	
	Power Consumption
	General series: below 3W. 
IR series: below 5W.
	General series: below 5W. 
IR series: below 7W.
	General series: below 7W. 
IR series: below 8W.

	
	Working Temperature
	-20℃～+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151.1×118.9
	¢151×119

	
	Weight
	1.25 kg

	
	Installation
	Support various installation modes


	                              Model

Parameter
	IPC-HDB/HDBW5100 Series
	IPC-HDB/HDBW5200 Series

	System 
	Main Processor 
	High performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network monitoring, local record, and remote operation. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3 inch CMOS
	1/2.8 inch CMOS

	
	Pixel
	1280(H)*960(V)
	1920 (H)*1080(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto

It ranges from 1/3 to 1/100000.

	
	Video Compression Standard
	H.264 / H.264H/MJPEG

	
	Video Frame Rate
	PAL:
Main stream (1280*960@25fps)

extra stream 1 (704*576@25fps)

extra stream 2 (1280*720@8fps)
NTSC:
Main stream (1280*960@30fps)

extra stream 1 (704*480@30fps)
extra stream 2 (1280*720@1fps),


	PAL: 
Main stream (1920*1080@25fps)

extra stream 1 (704*576@25fps)
extra stream 2 (1280*720@11fps)
NTSC:
Main stream (1920*1080@30fps)

extra stream 1 (704*480@30fps)

extra stream 2 (1280*720@1fps)


	
	
	Note:

Extra stream 2’s actual frame depends on device max capacity set which can be adjusted within its range.

	
	Video Bit Rate
	H.264H: 40Kbps-8192Kbps adjustable

MJPEG: 40Kbps-16384Kbps adjustable and bit rate is adjustable.
Support customized setup. 

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as bright, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering, overlay.

	
	Lens 
	2.7~9mm@F1.4
	2.7~12mm@F1.4

	
	Lens Interface
	¢14. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe RCA LINE IN (additional cable required)

	
	Audio Output 
	1-channel RCA LINE OUT (additional cable required)

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	8kbps 16bit or 16kbps 16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold 0-100.
Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Privacy Mask
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card storage, NAS storage

	  Network 
	Wired Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPoE,IPv4/v6,SNMP,QoS,802.1x, UPnP, NTP, QoS, 802.1x

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Input Protocol
	PSIA, ONVIF

	 AUX Interface
	Video Output
	1-ch analog output, BNC port (need additional cable)

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20~30 meters (For IR series product only)

	 General Parameter
	Power
	Support AC24V, DC12V and PoE (AC 24V port is inside the device.)

	
	Power Consumption
	General series: below 3W. 
IR series: below 5W.
	General series: below 5W. 
IR series: below 7W.

	
	Working Temperature
	-20℃～+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151×119

	
	Weight
	1.25 kg

	
	Installation
	Support various installation modes


	                              Model

Parameter
	IPC-HDB/HDBW5100-A2 Series
	IPC-HDB/HDBW5300

	System 
	Main Processor 
	High performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network monitoring, local record, and remote operation. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3 inch CMOS

	
	Pixel
	1280(H)*960(V)
	2048(H)×1536(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto

It ranges from 1/3 to 1/100000.

	
	Video Compression Standard
	H.264/H.264H/MJPEG

	
	Video Frame Rate
	PAL:
Main stream (1280*960@25fps)

extra stream 1(704*576@25fps)/
extra stream 2(1280*720@8fps)

NTSC:
Main stream (1280*960@30fps)

extra stream 1(704*480@30fps)/
 extra stream 2(1280*720@1fps)
	PAL:

Main stream (2048*1536@20fps)

extra stream 1(704*576@25fps)

extra stream 2(1280*720@1fps)

Main stream (1920*1080@25fps)

extra stream 1(704*576@25fps)

extra stream 2(1280*720@11fps)

NTSC:

Main stream(2048*1536@20fps)

extra stream 1(704*480@20fps)

extra stream 2(1280*720@1fps)

Main stream(1920*1080@20fps)

extra stream 1(704*480@30fps)

extra stream 2(1280*720@1fps)



	
	
	Note:

Extra stream 2’s actual frame depends on device max capacity set which can be adjusted within its range.

	
	Video Bit Rate
	H.264H: 40Kbps-8193Kbps. It is adjustable

MJPEG: 40Kbps-16384Kbps adjustable and bit rate is adjustable.
Support customized setup. 

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as bright, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering, overlay.

	
	Lens 
	2.7~9mm@F1.2
	2.7mm～12mm@F1.4

	
	Lens Interface
	¢14. Lens is the default accessories.

	    Audio 
	Audio Input
	2-channe RCA LINE IN (additional cable required)
	1-channe RCA LINE IN (additional cable required)

	
	Audio Output
	Not support
	1-channe RCA LINE OUT (additional cable required)

	
	Bidirectional Talk Input
	Not support
	Reuse the first audio input channel

	
	Audio Bit Rate
	8kbps 16bit or 16kbps 16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold 0-100.
Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Privacy Mask
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card (64G) storage, NAS storage

	  Network 
	Wired Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPoE,IPv4/v6,SNMP,QoS,UPnP,NTP, QoS, 802.1x.

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Protocol
	PSIA, ONVIF

	 AUX Interface
	Video Output
	1-ch analog output, BNC port (need additional cable)

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20~30 meters (For IR series product only)

	 General Parameter
	Power
	Support AC24V, DC12V and PoE (AC 24V port is inside the device.)

	
	Power Consumption
	General series: below 3W. 
IR series: below 5W.
	General series: below 5W. 
IR series: below 7W.

	
	Working Temperature
	-30℃～+60℃
	-20℃～+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151×119

	
	Weight
	1.25 kg

	
	Installation
	Support various installation modes


2 Structure 
2.1 Dimensions
You can refer to the following figures for dimension information. The Unit is mm. See Figure 2- 1(vandal-proof series), Figure 2- 2 (vandal-proof series), Figure 2- 3 (non vandal-proof series) and Figure 2- 4 (non vandal-proof series).
[image: image1.png]



Figure 2- 1 Dimension illustration 1
[image: image2.png]



Figure 2- 2 Dimension illustration 2
[image: image3.png]



Figure 2- 3 Dimension illustration 3
[image: image4.png]



Figure 2- 4 Dimension illustration 4
2.2 Port Description 
The port is shown as in Figure 2- 5 and Figure 2- 6.
[image: image5.png]



Figure 2- 5 Port 1
[image: image6.png]



Figure 2- 6 Port 2
Please refer to the following sheet for external connection port definition information.
	SN
	Port
	Port Name
	Connector
	Function Description

	1
	POWER
	AC 24V power port 
	/
	Connect to AC 24V power. 

Please note some series need additional wiring.

	2
	POWER
	DC 12V power port 
	/
	Connect to DC 12V power.

	3
	LAN
	RJ45 network port
	Ethernet port 
	Network cable port.

	4
	I/O
	I/O port 
	/
	It includes alarm input/output, audio and analog output.

	5
	RESET
	Reset button 
	/
	Reset button. It is to restore factory default setup. 

	6
	Micro SD
	Micro SD card slot entry
	Micro SD card
	Connect to Micro SD card to realize local storage. 

	7
	Fan port 
	/
	/
	Connect to fan to reduce device internal problem. 
Please note only some series support this function.

	8
	Status indicator light
	/
	/
	Display device status


Please refer to the following sheet for I/O port cable function information. 
	Port Name
	Cable SN
	Cable Port Name 
	Function Description 

	I/O Port
	1
	ALARM_NO
	Alarm output port. Output alarm signal to alarm device.

NO: Normal open alarm output end. 

	
	2
	ALARM_COM
	Alarm output public end. 

	
	3
	GND
	Ground end. 

	
	4
	ALARM_IN
	Alarm input port. It is to receive the on-off signal from the external alarm source. 

	
	5
	GND
	Ground end. 

	
	6
	AUDIO_IN/AUDIO_IN 1
	Input audio signal. It is to receive the analog audio signal from the devices such as pickup. 

	
	7
	AUDIO_OUT (optional)
	This port may have either of the following function, depending on model of the device: 

· Output audio signal to devices such as loud speaker. 
· Input audio signal. It is to receive the analog audio signal from the devices such as pickup.

	
	
	AUDIO_IN 2 (optional)
	

	
	8
	GND
	Ground end. 

	
	9
	VIDEO_OUT
	Output analog video signal. It can connect to TV monitor to view video. 


Note: 
The external connection port above is based on non-IR series which is identical with the external connection port of IR series.
2.3 Listening
Note: 

Some products support listening function only. 
Device Connection
Firstly connect the pickup to the audio input port of device, and then connect earphone to the audio output port of PC. Log in WEB page of the device, click on either one of [image: image7.png]


 or both to enable listening function. When listening function is enabled, the button will become orange. You may click on the button again to disable listening function. 
Listening Operation
At the device end, speak via the pickup, and then you can get the audio from the earphone or sound box at the pc-end.  
2.4 Bidirectional talk 
2.4.1 Device-end to PC-end 
Device Connection 
Please connect the speaker to the audio input port of the device. Then connect the earphone to the audio output port of the PC.
Login the Web and then click the Talk button to enable the bidirectional talk function. 
You can see the button becomes orange after you enabled the bidirectional talk function. 
Click Talk button again to stop the bidirectional talk function. 
Listening Operation
At the device end, speak via the pickup, and then you can get the audio from the earphone or sound box at the pc-end.  
2.4.2 PC-end to the Device-end
Device Connection 
Connect the speaker to the audio input port of the PC and then connect the earphone to the audio output port of the device.
Login the Web and then click the Talk button to enable the bidirectional talk function. 
You can see the button becomes orange after you enabled the audio talk function. 
Click Talk button again to stop the bidirectional talk function.  
Please note the on-site listening operation is null during the bidirectional talk process.
Listening Operation
At the PC-end, speak via the pickup, and then you can get the audio from the earphone or sound box at the device-end.  
2.5 Alarm Setup 
The alarm interface is shown as in Figure 2- 7. Please follow the steps listed below for local alarm input and output connection.
1) Connect the alarm input device to the alarm input port (No.4 pin) of the I/O cable.
2) Connect the alarm output device to the alarm output port (No.1 pin) and alarm output public port (No.2 pin). The alarm output port supports NO (normal open) alarm device only. 
3) Open the Web, go to the Figure 2- 7. Please set the alarm input 01 port for the first channel of the I/O cable (No.4 pin). Then you can select the corresponding type (NO/NC.)
4) Set the WEB alarm output. The alarm output 01 is for the alarm output port of the device. It is the No.1 pin of the I/O cable.
[image: image8.png]IP Camera

Camera Relay Activation
Network
able

Event
> Video Detect

> Abnormalty

storage.
System

[T e Second(10-300

F Relay-out

Alarm Delay

B
=

Default Refiesh





Figure 2- 7 Alarm
Please refer to the following figure for alarm input information. See Figure 2- 8.
Alarm input: When the input signal is idle or grounded, the device can collect the different statuses of the alarm input port. When the input signal is connected to the 3.3V or is idle, the device collects the logic “1”. When the input signal is grounded, the device collects the logic “0”.
[image: image9.png]\'Alarm Input
o

Network Camera

Collecting

GND





Figure 2- 8 Alarm input
Please refer to the following figure for alarm output information. See Figure 2- 9.
Port ALARM_C and Port ALARM_NO composes an on-off button to provide the alarm output. 
If the type is NO, this button is normal open. The button becomes off when there is an alarm output.
[image: image10.png]ALM_C

Network Camera

ALM_NO

Alarm Output





Figure 2- 9 Alarm output
3 Installation 
Important 
· Before you complete the installation and setup, do not remove the electrostatic attraction film on the transparent enclosure. Otherwise it may result in injury. 
· After remove electrostatic attraction film, do not touch dome enclosure in case it may leave stain.  
· Before the installation, please make sure the installation surface can sustain at least 3X weight of the bracket and the camera. 
3.1 Device Installation Introduction 
Please refer to Figure 3- 1 (vandal-proof series) and Figure 3- 2 (non vandal-proof series) for device installation space information. You can see there are installation position diagram and installation screws in the accessories bag for you to install the device conveniently. 
[image: image11.png]| ©130.9





Figure 3- 1 Device installation 1
[image: image12.png]



Figure 3- 2 Device installation 2
3.2 Device Installation Steps 
3.2.1 General Installation
The general interface is shown as in Figure 3- 3.
[image: image13.png]



Figure 3- 3 General installation 1
Step 1
Take the installation position diagram from the accessories bag and then paste it on the installation ceiling or the wall according to the monitor area. Please dig three bottom holes of the plastic expansion bolts according to the diagram. Take three expansion bolts from the accessories bag and then insert them to the holes you just dug and then fix firmly. If you need to dig a hole to pull through the cable, you need to dig a cable exit hole on the installation surface according to the installation positioning diagram. 
Step 2
Remove the dome camera enclosure according to the following steps.
a) For vandal-proof series, use the inner hex wrench from the accessories bag to unfasten the 3 hex screws on the dome camera enclosure and then remove it
b) For non vandal-proof series, rotate the dome camera enclosure counterclockwise as in Figure 3- 4 until you can remove it.
[image: image14.png]



Figure 3- 4 General installation 2
Step 3
Adjust the device chassis and pull the cable through the exit hole. Make fixture hole on chassis face the three plastic expansion bolts (in Step 1). Take 3 ST3.0 self-tapping screw out from the accessories bag and fix them on the three plastic expansion bolts to secure the chassis on installation surface. 
Note: 
· Make host GND grounded which can improve device reliability. For outside installation, the host GND must be grounded. 
·  GND of vandal-proof series locates next to its exit hole. GND of non vandal-proof series locates at rear of the camera.
· Device grounding screw dimension is M3X0.5 with valid depth 5mm.
Step 4

Adjust angle of the lens according to actual need.
a) For the IR series product, you can skip current step and go the step b) directly. For the non-IR series product, push the port slightly to remove the decoration enclosure from the black plastic enclosure. See Figure 3- 5.
[image: image15.png]



Figure 3- 5 General installation 3
b) Lens pan rotation angle setup. Please refer to Figure 3- 6  to unfasten the lock screw A and adjust the pan monitor angle to the proper position. Then fix the lock screw A. The pan angle ranges from 0°~350°.

c) Lens tilt rotation angle. Please refer to Figure 3- 6 to unfasten the lock screw B and lock screw C and adjust the tilt monitor angle to the proper position. Then fix the lock screw B and lock screw C. The tilt angle ranges from -20°~+70°.
d). Image pan rotation angle setup. Please refer to Figure 3- 6  to turn lock screw D to adjust the video pan angle. Then fix the lock screw B and C. The video pan angle ranges from 0°~+350°. 
[image: image16.png]



Figure 3- 6 General installation 4
e) Please refer to Chapter 3.2.2 for the lens zoom and focus operation of the manual focus series product.

f) For the IR series product, please skip current step and complete the angle setup directly.  For the non-IR series product, please put back the black plastic decoration enclosure to complete the angle setup. 

Important 

Please note Figure 3- 6 is based on non-IR series. For IR series, the lock screw position and the lens angle adjustment are the same. 

Step 5 

Line up the dome camera protection enclosure and follow step 2 reversely to put it back. 
Note
· Usually we recommend, after the installation, please take the three white static protection gaskets from the accessories bag and insert them to the screw holes of the protection enclosure. It is to enhance device reliability. 
· If you want to install Micro SD card, please refer to chapter 3.3. 
3.2.2 Manual Zoom Lens Focus Operation 

The manual zoom lens focus interface is shown as in Figure 3- 7.
[image: image17.png]



Figure 3- 7 Manual zoom lens
Step 1

Slightly loosen the adjusting screw E and push the adjust screw E to make it swing. Adjust the lens focus to the proper position according to the displayed video. 

Step 2

Slightly loosen the adjusting screw F and push the adjust screw F to make it swing. Adjust the lens to get the clear video and then fix the adjusting screw firmly. 

Step 3
When you are securing the adjusting screw F, you can see the video may become blur. Please push the adjusting screw E to adjust the video slightly. Please secure the adjust screw E if you get a clear video. 

3.2.3 Side Cable Exit 

If you adopt side cable exit when you are installing the device, you need to reserve space for the side cable exit and thirl. For vandal-proof series, it locates on side plastic decoration plug. For non vandal-proof series, it locates on side of its enclosure. Use the proper tool to dig through the U-shape slot to form a cable exit. Pull the cable through the hole. 

For some special user, he may need the metal protection tube to protect when he pulls through the cable from the side cable. For vandal-proof series, PG11 screw thread port is provided. Please remove the plastic decoration plug from the side panel of the chassis and pull through the cable to the tunnel of the PG11 screw thread. Now secure the tunnel in the PG11 screw threaded hole of the device. 

3.2.4 Cable Connection 
The vandal-proof series reserves two cable exits. The pin diameter shall be less than 15mm. One of the cable exits has M22 screw thread and can work with the default combination cable to remove the risk of the dragging and pulling of the cable. 

The vandal-proof series has two waterproof airproof plugs (One default position is the cable exit of the chassis of the device and the other is in the accessories bag.). The waterproof airproof plug has two functions. One is to fill in the cable exit and pull through the cable. It supports the cable whose diameter ranges from 4.0~6.0 mm. It is very convenient for you to do the waterproof work when you pull the cable through your own exit. Please refer to the steps listed. 

Step 1

Take the waterproof airproof plug out, pull the cable (diameter ranges from 4.0 to 6.0 mm) through the waterproof airproof plug. See Figure 3- 8.
[image: image18.png]:§E>




Figure 3- 8 Cable connection
Step 2

Before you go to Step 3 in Chapter 3.2.1, please pull through cable with the waterproof airproof plug to the device chassis via the installation hole at the bottom of the chassis and then connect the cable pins. 

Step 3 

Refer to Step 3 in Chapter 3.2.1, install and connect the cable pin to the device and then follow the proper steps to go on the installation. 

For non vandal-proof series, pull the cable through the anti-dust plug on cable exit of the device chassis. 
Important 

This series product has the power connection pin and I/O connection pin for you to pull through the signal cable. 

3.3 Micro SD Card Installation 
Warning!

Please unplug the device power cable and then shutdown the device before you install the Micro SD card. 

Step 1

Please refer to Step 2 in chapter 3.2.1 to open the device protection enclosure. 

Step 2

Please find the “SD” mark inside the device and adjust the Micro SD card direction according to prompt direction. Insert the card to the slot and then install the Micro SD card. See Figure 3- 9.
[image: image19.png]



Figure 3- 9 Micro SD card
4 Quick Configuration Tool

4.1 Overview 

Quick configuration tool can search current IP address, modify IP address. At the same time, you can use it to upgrade the device. 

Please note the tool only applies to the IP addresses in the same segment.

4.2 Operation

Double click the “ConfigTools.exe” icon, you can see an interface is shown as in Figure 4‑1.

In the device list interface, you can view device IP address, port number, subnet mask, default gateway, MAC address and etc. 

[image: image20.png]ConfigTool V1.0.5

e

vice List

s

TP Address

Port

Subnet Mask

Defanlt Gatevsy

Mae Adress

<l

182,188, 1.108

Edit

7

255, 255.255.0

182188 1.1

90:02:49:Tb:50

Tip: You can click “login® button directly and
then input corresponding infornation to login

]

Uperade





Figure 4‑1 Search interface
Select one IP address and then right click mouse, you can see an interface is shown as in Figure 4‑2.

Select the “Open Device Web” item; you can go to the corresponding web login interface.
[image: image21.jpg]Device List

ou can click “login” button directly and
then imput. corresponding infornation to login.

parate | Refresh Login Logeut |

&





Figure 4‑2 Search interface 2
If you want to modify the device IP address without logging in the device web interface, you can go to the configuration tool main interface to set. 

In the configuration tool search interface (Figure 4‑1), please select a device IP address and then double click it to open the login interface. Or you can select an IP address and then click the Login button to go to the login interface. See Figure 4-3.

In Figure 4-3, you can view device IP address, user name, password and port. Please modify the corresponding information to login. 

Please note the port information here shall be identical with the port value you set in TCP port in Web Network interface. Otherwise, you cannot login the device. 

If you are using device background upgrade port 3800 to login, other setups are all invalid.

[image: image22.png]1P Ackress:

User Name:

Passuord:

Port

152.168.1.108

e —





Figure 4‑3 Login prompt
After you logged in, the configuration tool main interface is shown as below. See Figure 4-4.

[image: image23.jpg]etk Frsnten 108 Systen Tnernationl Svten Vst

~General Parameter

Coveenbe [ 2]
T —
Subnet Mask: ~ [255.255.0.0

O —
O

= =





Figure 4‑4 Main interface
For detailed information and operation instruction of the quick configuration tool, please refer to the Quick Configuration Tool User’s Manual included in the resources CD.

5 Web Operation

This series network camera products support the Web access and management via PC.

Web includes several modules: Monitor channel preview, system configuration, alarm and etc.

5.1 Network Connection 

1. Please follow the steps listed below for network connection.

2. Make sure the network camera has connected to the network properly.

3. Please set the IP address, subnet mask and gateway of the PC and the network camera respectively. Network camera default IP address is 192.168.1.108. Subnet mask is 255.255.255.0. Gateway is 192.168.1.1

4. Use order ping ***.***.***.***(* network camera address) to check connection is OK or not. 

5.2 Login and Main Interface 

Open IE and input network camera address in the address bar. See Figure 5- 1.

[image: image24.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 5- 1 IP address
The login interface is shown as below. See Figure 5- 2.

Please input your user name and password. 

Default factory name is admin and password is admin. 

Note: For security reasons, please modify your password after you first login.
[image: image25.png]



Figure 5- 2 Web login
After you successfully logged in, please install WEB plug-in unit. Please refer to the Web Operation Manual included in the resource CD for detailed operation instruction.
See Figure 5- 3.

[image: image26.png]IP camera TR 7 T AR T

s





Figure 5- 3 Web monitoring window
6 FAQ
	Bug 
	

	I cannot boot up the device.
	Please click RESET button for at least five seconds to restore factory default setup.

	Micro SD card write times
	Do not set the Micro SD card as the storage media to store the schedule record file. It may damage the Micro SD card duration. 

	I cannot use the disk as the storage media. 
	When disk information is shown as hibernation or capacity is 0, please format it first (Via Web). 

	I cannot upgrade the device via network.
	When network upgrade operation failed, you can use port 3800 to continue upgrading. 

	Recommended Micro SD card brand
	Kingston 4GB, Kingston 16GB, Kingston 32GB, Transcend 16GB， SanDisk  4GB, SanDisk  32GB.
Usually we recommend the 4GB (or higher) or industry-level high speed card in case the slow speed results in data loss.  

	Audio function
	Please use active device for the audio monitor input, otherwise there is no audio in the client-end.

	The lightproof  ring of the IR device 
	The lightproof ring of the IR device lens is a necessary component when it is working. You will be unable to view clear video when the IR light is on if you remove the lightproof ring. 


Appendix Toxic or Hazardous Materials or Elements

	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Circuit Board Component
	○
	○
	○
	○
	○
	○

	Device Case
	○


	○
	○
	○
	○
	○

	Wire and Cable
	○


	○
	○
	○
	○
	○

	Packing Components
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.

X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets.  The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.
Note
· This user’s manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.

· All trademarks and registered trademarks mentioned are the properties of their respective 
owners.

· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website for more information.

Lock Screw A


Adjust lend pan rotation angle. 





Lock screw D


Adjust lend pan rotation angle. 








Lock screw B/C


Adjust lens tilt rotation angle. 





Adjusting screw F








Adjusting screw E








Input your IP address here








PAGE  
i

