Dahua HD (IR) Vandal Proof Network Dome Camera User’s Manual
Version 1.6.1
Zhejiang Dahua Technology CO., LTD 
Welcome

Thank you for purchasing our network camera!

This user’s manual is designed to be a reference tool for your system. 

Please read the following safeguard and warnings carefully before you use this series product!

Please keep this user’s manual well for future reference!

Important Safeguards and Warnings 
1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

The power shall conform to the requirement in the SELV (Safety Extra Low Voltage) and the Limited power source is rated 12V DC or 24V AC in the IEC60950-1. (Refer to general introduction) 
Please note: Do not connect two power supplying sources to the device at the same time; it may result in device damage! The product must be grounded to reduce the risk of electronic shock.
We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

We are not liable for any problems caused by unauthorized modification or attempted repair.

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Do not apply power to the camera before completing installation. 

Please install the proper power cut-off device during the installation connection.

Always follow the instruction guide the manufacturer recommended.

4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

This series network camera should be installed in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

Please keep it away from the electromagnetic radiation object and environment.

Please make sure the CCD (CMOS) component is out of the radiation of the laser beam device. Otherwise it may result in CCD (CMOS) optical component damage.

Please keep the sound ventilation. 

Do not allow the water and other liquid falling into the camera.

Thunder-proof device is recommended to be adopted to better prevent thunder.

The grounding studs of the product are recommended to be grounded to further enhance the reliability of the camera.

6. Daily Maintenance

Please shut down the device and then unplug the power cable before you begin daily maintenance work. 

Do not touch the CCD (CMOS) optic component. You can use the blower to clean the dust on the lens surface.

Always use the dry soft cloth to clean the device.  If there is too much dust, please use the water to dilute the mild detergent first and then use it to clean the device. Finally use the dry cloth to clean the device.

Please put the dustproof cap to protect the CCD (CMOS) component when you do not use the camera.

Dome enclosure is the optical component, do not touch the enclosure when you are installing the device or clean the enclosure when you are doing maintenance work. Please use professional optical clean method to clean the enclosure. Improper enclosure clean method (such as use cloth) may result in poor IR effect of camera with IR function.  

7. Accessories

Be sure to use all the accessories recommended by manufacturer.

Before installation, please open the package and check all the components are included.

Contact your local retailer ASAP if something is broken in your package.

	Accessory Name
	Amount

	Network Camera
	1

	Quick Start Guide
	1

	Installation Accessories Bag 
	1

	CD
	1

	Warranty Card and Certificate
	1


Table of Contents 

11
General Introduction


11.1
Overview


11.2
Features


21.3
Specifications


21.3.1
Performance


162
Structure


162.1
Dimensions


162.2
Port Description


222.3
Bidirectional talk


222.3.1
Device-end to PC-end


222.3.2
PC-end to the Device-end


232.4
Alarm Setup


253
Installation


253.1
Device Installation Introduction


253.2
Device Installation Steps


293.2.1
Manual Zoom Lens Focus Operation


303.2.2
Side Cable Exit


303.2.3
Cable Connection


323.3
Micro SD Card Installation


334
Quick Configuration Tool


334.1
Overview


334.2
Operation


355
Web Operation


355.1
Network Connection


355.2
Login and Main Interface


376
FAQ


38Appendix Toxic or Hazardous Materials or Elements





1 General Introduction 

1.1 Overview 

This series network camera integrates the traditional camera and network video technology. It adopts audio and video data collection, transmission together. It can connect to the network directly without any auxiliary device. 

This series network camera uses standard H.264  and MJPEG video compression technology and G.711a audio compression technology, which maximally guarantee the audio and video quality.
This series network camera enclosure has the strong resistance capacity, which can guarantee the proper work performance under heavy strike. It supports real-time monitor and listening at the same time. It supports analog video output and dual-way bidirectional talk. 

It can be used alone or used in a network area. When it is used lonely, you can connect it to the network and then use a network client-end. Due to its multiple functions and various uses, this series network camera is widely used in many environments such office, bank, road monitor and etc.
1.2 Features
	User Management
	· Different user rights for each group, one user belongs to one group.
· The user right shall not exceed the group right.

	Storage

Function
	· Support central server backup function in accordance with your configuration and setup in alarm or schedule setting

· Support record via Web and the recorded file are storage in the client-end PC.

· Support built-in Micro SD card.
· Support local Micro SD card hot swap, support short-time storage when encounter disconnection.

	Alarm Function
	· Real-time respond to external local alarm input and video detect as user pre-defined activation setup and exert corresponding message in screen and audio prompt(allow user to pre-record audio file)

· Real-time video detect: motion detect, tampering.
· Can generate an alarm when network abnormal, Micro SD card abnormal event occurred.

	Network Monitor 
	· Network camera supports one-channel audio/video data transmit to network terminal and then decode. Delay is within 270ms (network bandwidth support needed)

· Max supports 20 connections.
· Adopt the following audio and video transmission protocol: HTTP, TCP, UDP, MULTICAST, RTP/RTCP, RTSP and etc.
· Support web access, widely used in WAN.

	Network Management
	· Realize network camera configuration and management via Ethernet.
· Support device management via web or client-end.  
· Support various network protocols.

	Peripheral Equipment 
	· Support the on-off alarm device to alarm via the sound or the light.

	Power 
	· External power adapter DC12V/AC 24V 

· Support PoE.
Warning！

Do not connect these two power supplying sources to the device at the same time; it may result in device damage!

	Assistant Function
	· Log function

· Support PAL/NTSC

· Day/Night mode auto switch (electromagnetic ICR switch).
· Support iris auto adjust.
· Built-in IR light. Support IR night vision (For HDBW Series only).
· Backlight compensation: screen auto split to realize backlight compensation to adjust the bright.

· Support electronic shutter and gain setup.

· Support video watermark function to avoid vicious video modification.


1.3 Specifications 
1.3.1 Performance 

Please refer to the following sheet for network camera performance specification.

	[image: image37.jpg]E E sl onTrRoL”




                            Model

Parameter
	IPC-HDB/HDBW3202 Series 
	IPC-HDB/HDBW3101 Series 

	System 
	Main Processor 
	TI Davinci high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/2.8-inch CMOS
	1/3-inch CMOS

	
	Pixel
	1920（H）*1080（V）
	1280(H)*960(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
It ranges from 1/3 to 1/10000.

	
	Motorized Focus 
	Support remote motorized zoom focus function. 
	Not support

	
	Video Compression Standard
	H.264 /H.264H/H.264B/MJPEG
Note: Some versions do not support H.264H.

	
	Video Frame Rate
	PAL:

Main stream（1920*1080@25fps）

extra stream（704*576@25fps）

NTSC:

Main stream（1920*1080@30fps）

extra stream（704×480@30fps）
	PAL:

Main stream（1280*960@25fps）

Extra stream（704*576@25fps）

Main stream（1280*720@25fps）

extra stream（704*576@25fps）

NTSC:

Main stream（1280*960@30fps）

extra stream（704×480@30fps）

Main stream（1280*720@30fps）

extra stream（704×480@30fps）



	
	Video Bit Rate
	H.264: 56Kbps-8192Kbps. It is adjustable
MJPEG adjustable and bit rate is adjustable. 
Support customized setup.

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as bright, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	3~9mm@F1.2 motorized zoom focus lens 
	φ14 interface, 2.8~12mm@F1.4

	
	Lens Interface
	CS. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	16kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card storage, NAS storage

	  Network 
	Wire Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,IPv4/v6,SNMP,QoS,UPnP,NTP, HTTPS, 802.1x, Bonjour

	
	Protocol
	PSIA (optional), ONVIF

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 10 to 20 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC24V/DC12V power. PoE

	
	Power Consumption
	General series: below 5W.

IR series: below 7W.
	General series: below 3W.

IR series: below 5W.

	
	Working Temperature
	-20℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	[image: image38.jpg]


                      Model

Parameter
	IPC-HDB/HDBW5202 Series
	IPC-HDB/HDBW5302 Series

	System 
	Main Processor 
	High performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/2.8-inch CMOS
	1/3-inch CMOS

	
	Pixel
	1920（H）*1080（V）
	2048（H）×1536（V）

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
PAL：It ranges from 1/3 to 1/100000.

NTSC：It ranges from 1/4 to 1/100000.

	
	Motorized Focus 
	Support remote motorized focus function. 

	
	Video Compression Standard
	H.264 /H.264H/MJPEG
Note: Some versions do not support H.264H.

	
	Video Frame Rate
	PAL:

Main stream（1920*1080@25fps）
extra stream 1（704*576@25fps）
extra stream 2（1280*720@11fps）
NTSC:

Main stream（1920*1080@30fps）
extra stream 1（704×480@30fps）
extra stream 2（1280*720@1fps）

	PAL: 

Main stream (2048*1536@20fps) 
extra stream 1(704*576@20fps)

extra stream 2(1280*720@1fps),
Main stream(1920*1080@25fps)
extra stream 1(704*576@25fps)

extra stream 2(1280*720@11fps),
NTSC: 
Main stream (2048*1536@20fps) 
extra stream 1(704*480@20fps)

extra stream 2(1280*720@1fps),
Main stream(1920*1080@30fps)
extra stream 1(704*480@30fps)

extra stream 2(1280*720@1fps),


	
	
	Note:

Extra stream 2’s actual frame rate depends on device max capacity set which can be adjusted within its range.

	
	Video Bit Rate
	H.264H: 40Kbps-8192Kbps. It is adjustable
MJPEG: 40Kbps-16384Kbps. adjustable and bit rate is adjustable. 
Support customized setup.

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering, overlay.

	
	Lens 
	3~9mm@F1.2 motorized zoom focus lens 

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	8kbps 16bit or 16kbps 16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card storage, NAS storage

	  Network 
	Wire Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,IPv4/v6,SNMP,QoS,UPnP,NTP, 802.1x

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Protocol
	PSIA, ONVIF

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V power. PoE (AC 24V port is inside the device.)

	
	Power Consumption
	General series: below 5W.

IR series: below 7W.

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	[image: image39.jpg]alhua



                      Model

Parameter
	IPC-HDB/HDBW5502 Series

	System 
	Main Processor 
	High performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status info, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/2.5-inch CMOS

	
	Pixel
	2560（H）*1920（V）

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
It ranges from 1/3 to 1/100000.

	
	Motorized Focus 
	Support remote motorized focus function. 

	
	Video Compression Standard
	H.264 /H.264H/MJPEG
Note: Some versions do not support H.264H.

	
	Video Frame Rate
	PAL: 

Main stream (2560*1920@12fps) 
Extra stream 1 (704*576@12fps)

Extra stream 2 (1280*720@9fps)

Main stream (1920*1080@25fps) 
Extra stream 1 (704*576@25fps)

Extra stream 2 (1280*720@11fps)

NTSC: 
Main stream (2560*1920@12fps) 
Extra stream 1 (704*480@12fps)

Extra stream 2 (1280*720@10fps)

Main stream (1920*1080@30fps) 
Extra stream 1 (704*480@30fps)

Extra stream 2 (1280*720@1fps)

	
	Video Bit Rate
	H.264H: 40Kbps-8192Kbps. It is adjustable
MJPEG: 40Kbps-16384Kbps. adjustable and bit rate is adjustable. 
Support customized setup.

	
	Video Flip
	Support mirror. 

Support flip, rotation function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering, overlay.

	
	Lens 
	4~9mm@F1.2 motorized zoom focus lens 

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channel, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	8kbps 16bit or 16kbps 16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Storage Management
	Support Micro SD card (64G) disconnection local storage, NAS storage

	  Network 
	Wire Network 
	1-channel wire Ethernet port, 10/100 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,IPv4/v6,SNMP,QoS,UPnP,NTP, 802.1x

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Protocol
	PSIA, ONVIF

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V power and PoE

	
	Power Consumption
	General series: below 5W.

IR series: below 7W.

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	                            Model

Parameter
	IPC-HDB/HDBW8101 Series
	IPC-HDB/HDBW8106 Series

	System 
	Main Processor 
	TI high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3-inch CMOS

	
	Pixel
	1280(H)×1024(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
PAL：It ranges from 1/3 to 1/30000.

NTSC：It ranges from 1/4 to 1/30000.

	
	Motorized Focus 
	Not support
	Support remote motorized focus function.

	
	NR
	Support 3D NR

	
	WDR
	120dB WDR

	
	Video Setting
	Support rotate mode

	
	Intelligent Detection
	Tripwire, cross warning zone, abandoned object detection, missing object detection, scene change and out of focus.


	
	Face Recognition
	Detect face in video and link according to the result

	
	Audio Detection
	Detect if there is audio source input, filter noise, screaming emergency and etc. 

	
	Anti-IR Overexposure
	Auto detect image brightness, anti-overexposure via internal algorithm, guarantee the background brightness at the same time

	
	Video Compression Standard
	H.264/H.264H/H.264B/MJPEG 

	
	Video Frame Rate
	PAL: 

Main stream max(1280*1024@25fps), extra stream max(704*576@25fps), third stream (1280*1024@25fps)
NTSC: 
Main stream max(1280*1024@30fps), extra stream max(704*480@30fps), third stream (1280*1024@30fps)


	
	Video Bit Rate
	16Kbps-14Mbps. It is adjustable
Support customized setup.

	
	Video Interface 
	1 channel composite analog video output, BNC（1.0Vp-p,75Ω）(Not provided)

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	2.7～12@F1.4mm manual zoom focus lens 
	3~9mm@F1.2 motorized zoom focus lens

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	64kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Interface
	1-channel input，1-channel output (on-off)

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Local Storage
	Support Micro SD card storage, max 64G SDXC card storage

	  Network 
	Wire Network 
	1-channel 100/1000 Ethernet, RJ 45 port 

	
	Network Protocol
	HTTP,TCP/IP,ARP,IGMP ,ICMP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,UPNP,NTP,Bonjour,SNMP

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Access Protocol 
	ONVIF (default)，GB28181

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V and PoE power supply 

	
	Power Consumption
	Max 14W (IR ON)

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	                            Model

Parameter
	IPC-HDB/HDBW8201 Series
	IPC-HDB/HDBW8206 Series

	System 
	Main Processor 
	TI high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3-inch CMOS

	
	Pixel
	1920(H)×1080(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
PAL：It ranges from 1/3 to 1/30000.

NTSC：It ranges from 1/4 to 1/30000.

	
	Motorized Focus 
	Not support
	Support remote motorized focus function.

	
	NR
	Support 3D NR

	
	WDR
	120dB WDR

	
	Video Setting
	Support rotate mode

	
	Intelligent Detection
	Tripwire, cross warning zone, abandoned object detection, missing object detection, scene change and out of focus.


	
	Face Recognition
	Detect face in video and link according to the result

	
	Audio Detection
	Detect if there is audio source input, filter noise, screaming emergency and etc. 

	
	Anti-IR Overexposure
	Auto detect image brightness, anti-overexposure via internal algorithm, guarantee the background brightness at the same time

	
	Video Compression Standard
	H.264/H.264H/H.264B/MJPEG 

	
	Video Frame Rate
	PAL: 

Main stream max(1920*1080@25fps), extra stream max(704*576@25fps), third stream (1920*1080@25fps)
NTSC: 
Main stream max(1920*1080@30fps), extra stream max(704*480@30fps), third stream (1920*1080@30fps)


	
	Video Bit Rate
	16Kbps-20Mbps. It is adjustable
Support customized setup.

	
	Video Interface 
	1 channel composite analog video output, BNC（1.0Vp-p,75Ω）(Not provided)

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	2.7～12@F1.4mm manual zoom focus lens 
	3~9mm@F1.2 motorized zoom focus lens

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	64kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Interface
	1-channel input，1-channel output (on-off)

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Local Storage
	Support Micro SD card storage, max 64G SDXC card storage

	  Network 
	Wire Network 
	1-channel 100/1000 Ethernet, RJ 45 port 

	
	Network Protocol
	HTTP,TCP/IP,ARP,IGMP ,ICMP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,UPNP,NTP,Bonjour,SNMP

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Access Protocol 
	ONVIF (default)，GB28181

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V and PoE power supply 

	
	Power Consumption
	Max 14W (IR ON)

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	                            Model

Parameter
	IPC-HDB/HDBW8301 Series
	IPC-HDB/HDBW8306 Series

	System 
	Main Processor 
	TI high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/3-inch CMOS

	
	Pixel
	2048(H)×1536(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
PAL：It ranges from 1/3 to 1/30000.

NTSC：It ranges from 1/4 to 1/30000.

	
	Motorized Focus 
	Not support
	Support remote motorized focus function.

	
	NR
	Support 3D NR

	
	WDR
	120dB WDR

	
	Video Setting
	Support rotate mode

	
	Intelligent Detection
	Tripwire, cross warning zone, abandoned object detection, missing object detection, scene change and out of focus.


	
	Face Recognition
	Detect face in video and link according to the result

	
	Audio Detection
	Detect if there is audio source input, filter noise and etc. 

	
	Anti-IR Overexposure
	Auto detect image brightness, anti-overexposure via internal algorithm

	
	Video Compression Standard
	H.264/H.264H/H.264B/MJPEG 

	
	Video Frame Rate
	PAL: 

Main stream max(2048*1536@25fps), extra stream max(704*576@25fps), third stream (1920*1080@25fps)
NTSC: 
Main stream max(2048*1536@30fps), extra stream max(704*480@30fps), third stream (1920*1080@30fps)


	
	Video Bit Rate
	16Kbps-32Mbps. It is adjustable
Support customized setup.

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	2.7～12@F1.4mm manual zoom focus lens 
	3～9mm@F1.2motorized zoom focus lens

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	64kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Input 
	1-channel input，1-channel output

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Local Storage
	Support Micro SD card storage

	  Network 
	Wire Network 
	1-channel wire Ethernet port, 100/1000 Base-T Ethernet

	
	Network Protocol
	HTTP,TCP,ARP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,IPv4/v6,SNMP,QoS,UPnP,NTP.

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V power. PoE.

	
	Power Consumption
	Max 14W (IR ON)

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


	                            Model

Parameter
	IPC-HDB/HDBW8281 Series

	System 
	Main Processor 
	TI high performance DSP

	
	OS
	Embedded LINUX

	
	System

Resources
	Support real-time network, local record, and remote operation at the same time. 

	
	User Interface
	Remote operation interface such as WEB, DSS, PSS

	
	System Status
	Micro SD card status, bit stream statistics, log, and software version.

	Video Parameter
	Image Sensor 
	1/1.9-inch CMOS

	
	Pixel
	1920(H)×1080(V)

	
	Day/Night Mode
	Support day/night mode switch and electromagnetic IR-CUT at the same time.

	
	Auto Iris
	DC drive

	
	Gain Control 
	Manual/Auto

	
	White Balance
	Manual/Auto

	
	BLC 
	On/Off

	
	Electronic Shutter 
	Manual/Auto
PAL：It ranges from 1/3 to 1/30000.

NTSC：It ranges from 1/4 to 1/30000.

	
	NR
	Support 3D NR

	
	WDR
	120dB WDR

	
	Video Setting
	Support rotate mode

	
	Intelligent Detection
	Tripwire, cross warning zone, abandoned object detection, missing object detection, scene change and out of focus.


	
	Face Recognition
	Detect face in video and link according to the result

	
	Audio Detection
	Detect if there is audio source input, filter noise, screaming emergency and etc. 

	
	Anti-IR Overexposure
	Auto detect image brightness, anti-overexposure via internal algorithm, guarantee the background brightness at the same time

	
	Video Compression Standard
	H.264/H.264H/H.264B/MJPEG 

	
	Video Frame Rate
	PAL: 

Main stream max(1920*1080@50fps), extra stream max(704*576@50fps), third stream (1920*1080@50fps)
NTSC: 
Main stream max(1920*1080@60fps), extra stream max(704*480@60fps), third stream (1920*1080@60fps)


	
	Video Bit Rate
	16Kbps-40Mbps. It is adjustable
Support customized setup.

	
	Video Interface 
	1 channel composite analog video output, BNC（1.0Vp-p,75Ω）(Not provided)

	
	Video Flip
	Support mirror. 

Support flip function.

	
	Snapshot
	Max 1f/s snapshot. File extension name is JPEG.

	
	Privacy Mask
	Supports max 4 privacy mask zones 

	
	Video Setup
	Support parameter setup such as brightness, contrast.

	
	Video Information
	Channel title, time title, motion detect, tampering.

	
	Lens 
	4～12mm@F1.8 manual zoom focus lens 

	
	Lens Interface
	φ14 interface. Lens is the default accessories.

	    Audio 
	Audio Input
	1-channe, RCA audio input

	
	Audio Output 
	1-channel RCA audio output

	
	Bidirectional Talk Input
	Reuse the first audio input channel

	
	Audio Bit Rate
	64kbps  16bit

	
	Audio Compression Standard
	G.711a/G.711Mu

	    Video 
	Motion Detect
	396 (18*22) detection zones; sensitivity level ranges from 0 to 100; area threshold ranges from 0 to 100. 

Activation event: alarm device, audio/video storage, image snapshot, log, email function and etc.

	
	Video Loss
	Activation event, alarm device, audio/video storage, image snapshot, log, email function and etc.

	Alarm Interface
	1-channel input，1-channel output (on-off)

	Record and Backup 
	Record Priority 
	Manual>External alarm >Video detect>Schedule

	
	Local Storage
	Support Micro SD card storage, max 64G SDXC card storage

	  Network 
	Wire Network 
	1-channel 100/1000 Ethernet, RJ 45 port 

	
	Network Protocol
	HTTP,TCP/IP,ARP,IGMP ,ICMP,RTSP,RTP,UDP,RTCP,SMTP,FTP,DHCP,DNS,DDNS,PPPOE,UPNP,NTP,Bonjour,SNMP

	
	Remote Operation
	Monitor, system setup, log information, maintenance , upgrade and etc

	
	Access Protocol 
	ONVIF (default)，GB28181

	 AUX Interface
	Video Output
	1-channel analog video output，BNC port

	
	Reset
	Built-in RESET button 

	
	IR light 
	IR distance 20 to 30 meters (For IPC-HDBW series product only)

	 General Parameter
	Power
	Support AC 24V/DC12V and PoE power supply 

	
	Power Consumption
	Max 14W (IR ON)

	
	Working Temperature
	-30℃~+60℃

	
	Working Humidify 
	≤95%

	
	Dimensions(mm)
	¢151x119

	
	Weight
	1.25Kg

	
	Installation
	Support various installation modes


2 Structure 
2.1 Dimensions
You can refer to the following figures for dimension information. The Unit is mm. See Figure 2‑1 and Figure 2‑2.
[image: image1.png]



Figure 2‑1 Dimension 1
[image: image2.png]



Figure 2‑2 Dimension 2
2.2 Port Description 

For the non-IR series product, the interface is shown as in Figure 2‑3 and Figure 2‑4.

Note：

Figure in this chapter are all for reference only, and please refer to model 1-a and 1-b according to actual product. 
[image: image3.png]1112





Figure 2‑3 Camera internal without IR (1-a)
[image: image4.png]N
......... l“ﬂaﬂa
=N
Y @
\ﬂ 4
\...h

i
g
/‘%\A w
alC

\

(e
\m\,, ——,Em-m_n“_-“\\%\\'ﬂ&





Figure 2‑4 Camera internal without IR (1-b)
See Figure 2‑5.
[image: image5.png]


              Figure 2‑5 Camera internal without IR (2)
See Figure 2‑6, Figure 2‑7 and Figure 2-8.
[image: image6.png]



Figure 2‑6 Motorized focus camera internal with IR (1-a)
[image: image7.png]



Figure 2‑7 Motorized focus camera internal with IR (1-b)
[image: image8.png]



Figure 2‑8 Motorized focus camera internal with IR (2)
See Figure 2-9, Figure 2-10 and Figure 2-11. 

[image: image9.png]



Figure 2‑9 Manual focus camera internal with IR (1-a)

[image: image10.png]



Figure 2‑10 Manual focus camera internal with IR (1-b)

[image: image11.png]



Figure 2‑11 Manual focus camera internal with IR (2)

Please refer to the following sheet for external connection port definition information.
	SN
	Port
	Port Name
	Connector
	Function Description

	1
	POWER
	AC 24V power port 
	/
	Connect to AC 24V power.

	2
	POWER
	DC 12V power port 
	/
	Connect to DC 12V power.

	3
	Cable exit of the external connected cable
	/
	/
	Cable exit.

	4
	LAN
	RJ45 network port
	Ethernet port 
	Network cable port.

	5
	I/O
	I/O port 
	/
	It includes alarm input/output, audio and analog output.

	6
	RESET
	Reset button 
	/
	Reset button. It is to restore factory default setup. 

	7
	AUTO FOCUS
	5-direction button 
	/
	Adjust lens angle of view and definition. 
Note: Only some models have this function. 

	8
	Status indicator light 
	/
	/
	Display device running status.

	9
	Fan port 
	/
	/
	Connect to fan to reduce device internal problem. 
Please note this component is optional. 

	10
	Micro SD
	Micro SD card slot entry
	Micro SD card
	Connect to Micro SD card to realize local storage. 

	11
	POWER
	12V power port
	/
	The power port of the external connected cable. 
Default input is DC 12V.

	12
	LAN
	Network port 
	Ethernet port
	· The network port of the external connected cable.

· Connect to standard Ethernet port. 

· Support PoE. 


Please refer to the following sheet for I/O port cable function information. Note: actual product may vary.
	Port Name
	Cable SN
	Cable Port Name 
	Function Description 

	I/O Port
	1
	ALARM_NO
	Alarm output port. Output alarm signal to alarm device.

NO: Normal open alarm output end. 

	
	2
	ALARM_COM
	Alarm output public end. 

	
	3
	GND
	Ground end. 

	
	4
	ALARM_IN
	Alarm input port. It is to receive the on-off signal from the external alarm source. 

	
	5
	GND
	Ground end. 

	
	6
	AUDIO_IN
	Input audio signal. It is to receive the analog audio signal from the devices such as pickup. 

	
	7
	AUDIO_OUT
	Output audio signal to devices such as sound box. 

	
	8
	GND
	Ground end. 

	
	9
	VIDEO_OUT
	Output analog video signal. It can connect to TV monitor to view video. 


2.3 Bidirectional talk 

2.3.1 Device-end to PC-end 

Device Connection 

Please connect the speaker or the MIC to the audio input port of the device. Then connect the earphone to the audio output port of the PC.

Login the Web and then click the Talk button to enable the bidirectional talk function. 

You can see the button becomes orange after you enabled the bidirectional talk function. 

Click Talk button again to stop the bidirectional talk function. 
Listening Operation
At the device end, speak via the speaker or the pickup, and then you can get the audio from the earphone or sound box at the pc-end.  

2.3.2 PC-end to the Device-end
Device Connection 

Connect the speaker or the MIC to the audio input port of the PC and then connect the earphone to the audio output port of the device.

Login the Web and then click the Talk button to enable the bidirectional talk function. 

You can see the button becomes orange after you enabled the audio talk function. 

Click Talk button again to stop the bidirectional talk function.  

Please note the on-site listening operation is null during the bidirectional talk process.

Listening Operation
At the PC-end, speak via the speaker or the pickup, and then you can get the audio from the earphone or sound box at the device-end.  

2.4 Alarm Setup 
The alarm interface is shown as in Figure 2‑12. Please follow the steps listed below for local alarm input and output connection.
1) Connect the alarm input device to the alarm input port (No.4 pin) of the I/O cable.
2) Connect the alarm output device to the alarm output port (No.1 pin) and alarm output public port (No.2 pin). The alarm output port supports NO (normal open) alarm device only. 
3) Open the Web, go to the Figure 2‑12. Please set the alarm input 01 port for the first channel of the I/O cable (No.4 pin). Then you can select the corresponding type (NO/NC.)
4) Set the WEB alarm output. The alarm output 01 is for the alarm output port of the device. It is the No.1 pin of the I/O cable.
[image: image12.png]@hua

Camera Relay Activation

Network

I Enable
Event

Relay-
> Video Detect i

> Abnormality Ant-Dither SensorType [NO

storage

F Record
system H e
Information Record Delay

¥ Relay-out
Alam Delay

I send Email

I snapshot

Default Refresh





Figure 2‑12 Alarm
Please refer to the following figure for alarm input information. See Figure 2‑13.
Alarm input: When the input signal is idle or grounded, the device can collect the different statuses of the alarm input port. When the input signal is connected to the 5V or is idle, the device collects the logic “1”. When the input signal is grounded, the device collects the logic “0”.
[image: image13.png]\'Alarm Input
o

Network Camera

Collecting

GND





Figure 2‑13 Alarm input
Please refer to the following figure for alarm output information. See Figure 2‑14.
Port ALARM_COM and Port ALARM_NO composes an on-off button to provide the alarm output. 

If the type is NO, this button is normal open. The button becomes on when there is an alarm output.

If the type is NC, this button is normal off. The button becomes off when there is an alarm output. 
[image: image14.png]ALM_C

Network Camera

ALM_NO

Alarm Output





Figure 2‑14 Alarm output
3 Installation 
Important 

· Before you complete the installation and setup, do not remove the electrostatic attraction film on the transparent enclosure. Otherwise it may result in injury. 

· After remove electrostatic attraction film, don’t touch dome enclosure in case it may leave stain.  
· Before the installation, please make sure the installation surface can sustain at least 3X weight of the bracket and the camera. 
3.1 Device Installation Introduction 
Please refer to Figure 3‑1 for device installation space information. You can use screws (diameter is less than 4.5mm) to secure the device. You can see there are installation position map and installation screws in the accessories bag for you to install the device conveniently. 
[image: image15.png]| ©130.9





Figure 3‑1 Installation 1
3.2 Device Installation Steps 
The interface is shown as in Figure 3‑2.
[image: image16.png]


          [image: image17.png]


    [image: image18.png]



IR Motorized Zoom Lens Series         Non-IR Series                  IR Manual Zoom Series 
Figure 3‑2
Step 1
Take the installation position map from the accessories bag and then paste it on the installation ceiling or the wall according to the monitor area. Please dig three bottom holes of the plastic expansion bolts according to the map. Take three expansion bolts from the accessories bag and then insert them to the holes you just dug and then fix firmly. If you need to dig a hole to pull through the cable, you need to dig a cable exit hole (The diameter is more than 28mm) on the installation surface according to the installation positioning map. 
Step 2
Use the inner hex wrench from the accessories bag to unfasten the 3 hex screws on the dome camera enclosure to open it. 

Step 3
Please remove the device cable (Provided) network port and the power terminal. Use the inner hex wrench (Provided) to remove the 2 inner hex screws from the dome driver module. Then please follow the prompt on the device to push the metal hook to two sides. Remove the dome driver module from the chassis. See Figure 3‑3.
[image: image19.png]



Figure 3‑3 Installation 2
Step 4
Adjust the chassis of the device to the proper position and pull the cable to the cable exit of the installation surface. Line up the holes of the chassis to the three expansion bolt holes you dug in Step 1. Take three ST3.0 self-tapping screws and secure them in the three plastic expansion bolts. Now the chassis is secure on the installation surface.  

Important 

Please earth the device GND hole [image: image20.png]


 (GND) to enhance the reliability of the device. The GND port is near the cable exit of the rear panel. The GND screw thread specification is M3-6mm.

[image: image21.png]



Figure 3‑4 GND hole
Step 5
Please refer to the Step 3 to put the driver module back to the metal hooks of the chassis. Then use the inner hex wrench to secure the two inner hex screws to the chassis. Then connect the network cable and the power terminal. 

Step 6
Adjust the lens to the proper angle according to your monitor requirements. 

a) For the IR series product, you can skip current step and go the step b) directly. For the non-IR series product, push the port slightly to remove the decoration enclosure from the black plastic enclosure. See Figure 3‑5. 

[image: image22.png]



 Figure 3‑5 Installation 3
b) Lens pan rotation angle setup. Please refer to Figure 3‑6 to unfasten the lock screw A and adjust the pan monitor angle to the proper position. Then fix the lock screw A. The pan angle ranges from 0°~+350°.

[image: image23.png]



Figure 3‑6 Installation 4
c). Lens tilt rotation angle. Please refer to Figure 3‑ to unfasten the lock screw B and lock screw C and adjust the tilt monitor angle to the proper position. Then fix the lock screw B and lock screw C. The tilt angle ranges from -23°~+73°.

d). Image pan rotation angle setup. Please refer to Figure 3‑7 to turn lock screw D to adjust the video pan angle. Then fix the lock screw B and C. The video pan angle ranges from 0°~+350°.
[image: image24.png]



Figure 3‑7 Installation 5
e) For the motorized zoom series product, please skip current step. Please refer to chapter 3.2.1 for the lens zoom and focus operation of the manual focus series product.

f) For the IR series product, please skip current step and complete the angle setup directly.  For the non-IR series product, please put back the black plastic decoration enclosure to complete the angle setup. 

Important 

Please note Figure 3‑6 and Figure 3‑7 is based on the IR motorized zoom camera. For the IR manual zoom camera and non-IR series product, the lock screw position and the lens angle adjustment are the same. 

Step 7 

Line up the dome camera protection enclosure to the cable exit on the side panel. Put the enclosure back and then use the inner hex wrench to secure the 3 inner hex screws firmly. Now the installation is complete. 

Note

Usually we recommend, after the installation, please take the three white static protection gaskets from the accessories bag and insert them to the screw holes of the protection enclosure. It is to enhance device reliability. 

3.2.1 Manual Zoom Lens Focus Operation 
The manual zoom lens focus interface is shown as in Figure 3‑8.

Step 1

Slightly loosen the adjusting screw E and push the adjust screw E to make it swing. Adjust the lens focus to the proper position according to the displayed video. 
Step 2
Slightly loosen the adjusting screw F and push the adjust screw F to make it swing. Adjust the lens to get the clear video and then fix the adjusting screw firmly. 

Step 3
When you are securing the adjusting screw F, you can see the video may become blur. Please push the adjusting screw E to adjust the video slightly. Please secure the adjust screw E if you get a clear video. 
[image: image25.png]



Figure 3‑8 Installation 6
3.2.2 Side Cable Exit 

If you adopt side cable exit when you are installing the device, you need to remove the plastic decoration plug from the side of the chassis. Use the proper tool to dig through the part specified in Figure 3‑9 to form a cable exit. Put the plastic decoration plug back to the chassis and then pull the cable through the side panel of the chassis. 

       [image: image26.png]


[image: image27.png]



Figure 3‑9 Installation 7
For some special user, he may need the metal protection tube to protect when he pulls through the cable from the side cable. There is PG11screw thread port when you pull through the cable from the side panel. Please remove the plastic decoration plug from the side panel of the chassis and pull through the cable to the tunnel of the PG11 screw thread. Now secure the tunnel in the PG11 screw threaded hole of the device. 

3.2.3 Cable Connection 

The device reserves two cable exits. The pin diameter shall be less than 15mm. One of the cable exits has M22 screw thread and can work with the default combination cable to remove the risk of the dragging and pulling of the cable. 

The device has two waterproof airproof plugs (One default position is the cable exit of the chassis of the device and the other is in the accessories bag.). The waterproof airproof plug has two functions. One is to fill in the cable exit and pull through the cable. It supports the cable whose diameter ranges from 4.0~6.0. It is very convenient for you to do the waterproof work when you pull the cable through your own exit. Please refer to the steps listed. 

Step 1
Take the waterproof airproof plug out, pull the cable (diameter ranges from 4.0 to 6.0) through the waterproof airproof plug. See Figure 3‑10.
[image: image28.png]:§E>




Figure 3‑10 Cable connection
Step 2

Before you go to the Step 4 in the chapter 3.1 installation steps, please pull through cable with the waterproof airproof plug to the device chassis via the installation hole at the bottom of the chassis and then connect the cable pins. 

Step 3 

Refer to Step 4 and Step 5 in the chapter 3.1 installation steps to install and connect the cable pin to the device and then follow the proper steps to go on the installation. 

Important 

This series product has the power connection pin and I/O connection pin for you to pull through the signal cable. 

3.3 Micro SD Card Installation 

Warning!

Please unplug the device power cable and then shutdown the device before you install the Micro SD card. 
Step 1

Please refer to Step 2 in chapter 3.2 installation steps to open the device protection enclosure. 

Step 2

Please find the “Micro SD” mark inside the device and adjust the Micro SD card direction according to prompt direction. Insert the card to the slot and then install the Micro SD card. See Figure 3‑11.
[image: image29.png]



Figure 3‑11 Micro SD card
4 Quick Configuration Tool

4.1 Overview 

Quick configuration tool can search current IP address, modify IP address. At the same time, you can use it to upgrade the device. 

Please note the tool only applies to the IP addresses in the same segment.

4.2 Operation

Double click the “ConfigTools.exe” icon, you can see an interface is shown as in Figure 4‑1.

In the device list interface, you can view device IP address, port number, subnet mask, default gateway, MAC address and etc. 

[image: image30.png]ConfigTool V1.0.5

e

vice List

s

TP Address

Port

Subnet Mask

Defanlt Gatevsy

Mae Adress

<l

182,188, 1.108

Edit

7

255, 255.255.0

182188 1.1

90:02:49:Tb:50

Tip: You can click “login® button directly and
then input corresponding infornation to login

]

Uperade





Figure 4‑1 Search interface
Select one IP address and then right click mouse, you can see an interface is shown as in Figure 4‑2.

Select the “Open Device Web” item; you can go to the corresponding web login interface.
[image: image31.jpg]Device List

ou can click “login” button directly and
then imput. corresponding infornation to login.

parate | Refresh Login Logeut |

&





Figure 4‑2 Search interface 2
If you want to modify the device IP address without logging in the device web interface, you can go to the configuration tool main interface to set. 

In the configuration tool search interface (Figure 4‑1), please select a device IP address and then double click it to open the login interface. Or you can select an IP address and then click the Login button to go to the login interface. See Figure 4-3.

In Figure 4-3, you can view device IP address, user name, password and port. Please modify the corresponding information to login. 

Please note the port information here shall be identical with the port value you set in TCP port in Web Network interface. Otherwise, you cannot login the device. 

If you are using device background upgrade port 3800 to login, other setups are all invalid.

[image: image32.png]1P Ackress:

User Name:

Passuord:

Port

152.168.1.108

e —





Figure 4‑3 Login prompt
After you logged in, the configuration tool main interface is shown as below. See Figure 4-4.

[image: image33.jpg]etk Frsnten 108 Systen Tnernationl Svten Vst

~General Parameter

Coveenbe [ 2]
T —
Subnet Mask: ~ [255.255.0.0

O —
O

= =





Figure 4‑4 Main interface
For detailed information and operation instruction of the quick configuration tool, please refer to the Quick Configuration Tool User’s Manual included in the resources CD.

5 Web Operation

This series network camera products support the Web access and management via PC.

Web includes several modules: Monitor channel preview, system configuration, alarm and etc.

5.1 Network Connection 

Please follow the steps listed below for network connection.

· Make sure the network camera has connected to the network properly.

· Please set the IP address, subnet mask and gateway of the PC and the network camera respectively. Network camera default IP address is 192.168.1.108. Subnet mask is 255.255.255.0. Gateway is 192.168.1.1

· Use order ping ***.***.***.***(* network camera address) to check connection is OK or not. 

5.2 Login and Main Interface 

Open IE and input network camera address in the address bar. See Figure 5- 1.

[image: image34.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 5- 1 IP address
The login interface is shown as below. See Figure 5- 2.

Please input your user name and password. 

Default factory name is admin and password is admin. 

Note: For security reasons, please modify your password after you first login.
[image: image35.png]



Figure 5- 2 Web login
After you successfully logged in, please install WEB plug-in unit. Please refer to the Web Operation Manual included in the resource CD for detailed operation instruction.
See Figure 5- 3.
[image: image36.png]@hua [ e [t L osoun | sam [ icsou|
o

[o B/} - S > X 76)
720

g





Figure 5- 3 Web monitoring window
6 FAQ

	Bug 
	

	I can not boot up the device.
	Please click RESET button for at least five seconds to restore factory default setup.

	Micro SD card write times
	Do not set the Micro SD card as the storage media to storage the schedule record file. It may damage the Micro SD card duration. 

	I can not use the disk as the storage media. 
	When disk information is shown as hibernation or capacity is 0, please format it first (Via Web). 

	I can not upgrade the device via network.
	When network upgrade operation failed, you can use port 3800 to continue upgrade. 

	Recommended Micro SD card brand
	Kingston 4GB, Kingston 1GB, Kingston 16GB, Transcend 16GB， SanDisk  1G,  SanDisk  4G.
Usually we recommend the 4GB (or higher) or industry-level high speed card in case the slow speed results in data loss.  

	Audio function
	Please use active device for the audio monitor input, otherwise there is no audio in the client-end.

	The lightproof  ring of the IR device 
	The lightproof ring of the IR device lens is the necessary component when it works. You can not view the clear video when the IR light is on if you remove the lightproof ring. 


Appendix Toxic or Hazardous Materials or Elements

	Component Name
	Toxic or Hazardous Materials or Elements

	
	Pb
	Hg
	Cd
	Cr VI
	PBB
	PBDE

	Circuit Board Component
	○
	○
	○
	○
	○
	○

	Device Case
	○


	○
	○
	○
	○
	○

	Wire and Cable
	○


	○
	○
	○
	○
	○

	Packing Components
	○
	○
	○
	○
	○
	○

	Accessories
	○
	○
	○
	○
	○
	○


O: Indicates that the concentration of the hazardous substance in all homogeneous materials in the parts is below the relevant threshold of the SJ/T11363-2006 standard.
X: Indicates that the concentration of the hazardous substance of at least one of all homogeneous materials in the parts is above the relevant threshold of the SJ/T11363-2006 standard. During the environmental-friendly use period (EFUP) period, the toxic or hazardous substance or elements contained in products will not leak or mutate so that the use of these (substances or elements) will not result in any severe environmental pollution, any bodily injury or damage to any assets.  The consumer is not authorized to process such kind of substances or elements, please return to the corresponding local authorities to process according to your local government statutes.
Note
· This user’s manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.
· All trademarks and registered trademarks mentioned are the properties of their respective 
owners.
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website for more information.

Zhejiang Dahua Technology CO., LTD.
Address：No.1199 Bin’an Road, Binjiang District, Hangzhou, PRC.

Postcode: 310053

Tel: +86-571-87688883

Fax: +86-571-87688815

Email:overseas@dahuatech.com

Website: www.dahuatech.com


















































































































Dig through here





Adjusting Screw F





Lock screw B/C


Adjust lens tilt rotation angle. 





Adjusting Screw E



































Lock Screw A


Adjust lens pan rotation angle. 





Lock screw D


Adjust lens pan rotation angle. 








Input your IP address here























PAGE  
iv

