
Block Camera Web3.0 Operation Manual
Version 4.0.0
Table of Contents
11
Network Connection


11.1
Preparation


11.2
Log in


31.3
Live Interface


31.4
Encode Setup


41.5
Video Window Setup


51.6
System Menu


51.7
Video Window Function Option


61.8
PTZ Control


71.9
PTZ Setup/Menu


102
Setup


102.1
Camera


102.1.1
Video


152.1.2
Audio


162.2
Network


162.2.1
TCP/IP


182.2.2
Connection


202.2.3
PPPoE


202.2.4
DDNS


212.2.5
IP filter


222.2.6
SMTP （e-mail）


232.2.7
UPnP


242.2.8
SNMP


262.2.9
Bonjour


272.2.10
Multicast


272.2.11
IEEE802


282.2.12
Qos


292.3
Event


292.3.1
Video detect


332.3.2
Alarm


362.3.3
Abnormity


382.4
Storage


382.4.1
Record schedule and snapshot schedule


382.4.2
Destination


402.4.3
Record control


412.5
System


412.5.1
General


432.5.2
Account


462.5.3
PTZ


472.5.4
Default


482.5.5
Import/Export


482.5.6
Auto maintenance


492.5.7
Firmware update


492.6
Information


492.6.1
Version


492.6.2
Log


502.6.3
Online User


523
Alarm


534
Log out



Important
· The following functions are for reference only. Some series products may not support all the functions listed below. 
· Usually we recommend IE 7 or higher version. For those versions below IE 7, it may not support the operation of some functions.

1 Network Connection 
1.1 Preparation 

This series block camera product supports the Web access and management via PC.
Web includes several modules: monitor channel preview, PTZ control, system configuration, alarm and etc.
Please follow the steps listed below for network connection.

· Make sure the block camera has connected to the network properly.

· Block camera IP address and PC IP address shall be in the same network segment. If there is router, please set the corresponding gateway and subnet mask.

· Use order ping ***.***.***.***(* block camera address) to check connection is OK or not. 

1.2 Log in 

Open IE and input block camera address in the address bar. 

For example, if your device IP is 192.168.1.108, then please input http:// 192.168.1.108 in IE address bar. See Figure 1‑1.

[image: image72.png]Security Setting:

nternet Zone

Settings

O Disable
© Enable

9] Download signed ActiveX controls (not secure)
O Disable
© Enable (not secure)
O Prompt (recommended)

9] Download unsigned Active controls (not secure)
© Disable (recommended)
© Enable (not secure)
O Prompt

19 Intiaize and scrpt ActiveX contros ok merked as sae for s
O Disable (recommended)
© Enable (not secure)
O Prompt

4] Run ctiveX contros and pug-ins
3 bbb e

< I

*Takes effect after you restart Internet Explorer

Reset custom settings




[image: image1.png]GO - [ermmm el ()] oo oL

T @ne= T





Figure 1‑1
The login interface is shown as below. See Figure 1‑2.

Please input your user name and password. 
Default factory name is admin and password is admin. 
Note: For security reasons, please modify your password after you first login.
[image: image2.png]N —





Figure 1‑2 
If it is your first time to login in, system pops up warning information to ask you whether install control webrec.cab or not. Please click OK button, system can automatically install the control. When system is upgrading, it can overwrite the previous Web too.
If you can’t download the ActiveX file, please check whether you have installed the plug-in to disable the control download. Or you can lower the IE security level. See Figure 1‑3.
[image: image3.png]Internet Options

General | Security |Privacy || Content | Connections | Programs | Advanced |

Select a zone to view or change security sektings.

@ ) v e

oot oca Tustedstes ¥
< ] 8
eret =
‘This zone is for Internet websites, =5
ks e ratios o
e

Security level for tis zone
Allowed levels For this zone: Medum ta Hih

Medium-high
- Appropriate for most websites

- Prompts before dowrloading potentialy unsafe
cantent.

~Unsigned Actve controls il not be downloaded

Custom level Default fevel

Reset all zones to defaul evel

m





Figure 1‑3
1.3 Live Interface 

After you logged in, you can see the live monitor window. Now you can operate the block camera via the WEB. See Figure 1‑4. [image: image4.jpg]WEB SERVICEsso
e Protoco! [T |

o0®0
000
speeat-2) RS
)
© s @
©-o

PTZSetings  lienu





Figure 1‑4 
There are six sections:

· Section 1: Encode setup bar
· Section 2: Window adjust bar 
· Section 3: System menu  bar
· Section 4: Window function option bar
· Section 5: PTZ control 
· Section 6:PTZ setup/menu 
1.4 Encode Setup 

The encode setup interface is shown as in Figure 1‑5.
[image: image5.png]Protocal





Figure 1‑5 
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Main stream 
	In normal network width environment, main stream can record audio/video file and realize network monitor. 

	Sub (Extra) stream 
	If network width is not sufficient, you can use sub stream to realize network monitor. It is to reduce network bandwidth usage. 

	Protocol 
	You can select video monitor protocol from the dropdown list. 
There are three options: TCP/UDP/Multicast.


1.5 Video Window Setup  

The interface is shown as in Figure 1‑6.


[image: image6.png]



Figure 1‑6  
Please refer to the following sheet for detailed information. 
	SN
	Parameter
	Function 

	1
	Image control 
	Click it to open picture setup interface. See Figure 1‑7. This interface is on the top right pane. 

	2
	Original size 
	Click this button to go to original size. It is to display the actual size of the video. It depends on the resolution of the video. 

	3
	Full screen 
	Click it to go to full-screen mode.  Double click the mouse or click the Esc button to exit the full screen. 

	4
	Width and height ratio
	Click it to restore original ratio or suitable window. 


	5
	Open/close PTZ 
	Left click it to display or hide the PTZ control interface. 


The picture setup interface is shown as in Figure 1‑7.
[image: image7.png]



Figure 1‑7
Please refer to the following sheet for detailed information. 
	Parameter
	Function 

	Video setup 
	
[image: image8.png]



	Brightness setup icon. It is to adjust monitor video brightness. 
	Note:

· All the operations here apply to WEB end only.

· Please go to Setup->Camera->Conditions to adjust brightness, contrast, hue and saturation setup. 

	
	
[image: image9.png]



	Contrast setup icon. It is to adjust monitor video contrast ness.
	

	
	
[image: image10.png]



	Saturation setup icon. It is to adjust monitor video saturation.
	

	
	
[image: image11.png]



	Hue setup icon. It is to adjust monitor video hue.
	

	
	Reset
	Restore brightness, contrastness saturation and hue to system default setup. 
	


1.6 System Menu 

System menu is shown as in Figure 1‑8.
Please refer to chapter 1.3 Live, chapter 2 Setup, chapter 3 Alarm, chapter 5 Log out for detailed information.  

[image: image12.png](T YT T




Figure 1‑8 
1.7 Video Window Function Option 

The interface is shown as below. See Figure 1‑9.

[image: image13.png]



Figure 1‑9
Please refer to the following sheet for detailed information. 
	SN
	Parameter
	Function 

	1
	Alarm output  
	Click it to generate an alarm output.  
There is one alarm output icons since this series product supports two relay output. 

	3
	Digital zoom
	When video is in original status, click this button you can select any zone to zoom in.  After zoom in ,you can  drag the zoom in area  in the specified area. Right click mouse system restores original status. You can use the wheel to zoom out the video. 

	4
	Snapshot
	You can snapshoot important video. 

You can go to Setup->Camera->Video->Path to modify the local record save path.

	5
	Triple snap 
	Click it, system can snap at 1f/s for three times. All images are memorized in system storage folder.

	6
	Record
	When you click local record button, the system begins recording. 
You can go to Setup->Camera->Video->Path to modify the local record save path.

	7
	Audio output 
	Turn on or off audio when you are monitoring.
You can go to Setup->Camera->Audio to set. 

	8
	Bidirectional talk 
	Click it to begin audio talk. You can go to Setup->Camera->Audio to set bidirectional talk mode.

	9
	Help 
	Click it to open help file. 


1.8 PTZ Control 

Before PTZ operation, please make sure you have properly set PTZ protocol. (Please go to Setup->System->PTZ to set.).

Here you can view direction keys, speed, zoom, focus, iris button. See Figure 1‑10.
· PTZ direction: PTZ supports eight directions: left/right/up/down/upper left/upper right/bottom left/bottom right/ fast positioning.
· Speed: The step 8 speed is faster than step 1. 

[image: image14.png]000
0®o0
000
speedt-2) RS
© = @
© ra: @
(+ O




Figure 1‑10 
1.9 PTZ Setup/Menu 
The PTZ setup/Menu interface is shown as in Figure 1‑11.

[image: image15.png]



Figure 1‑11  

Click PTZ set button, the interface is shown as in Figure 1‑12.Here you can set scan, preset, tour pattern, assistant function and etc. 
[image: image16.jpg]PTZ Settings





Figure 1‑12  

Please refer to the following sheet for PTZ setup information. 

	Parameter 
	Function 

	Scan 
	· Click Setup button, you can set scan left and right limit. 

· Use direction buttons to move the camera to you desired location and then click left limit button. Then move the camera again and then click right limit button to set a right limit.

	Preset 
	· Input the preset value and then click Preset button, the camera turns to the corresponding position of the preset. 

· Click the Set preset button, you can set a preset. Use direction keys to move the camera to your desired location and then input preset value. Click add button, you have set one preset.

· The preset value varies due to different protocols.

	Tour 
	· Click the Setup button, you can begin set tour. 

· Input tour value and then click the Set button. The tour value ranges from 1 to 255. (It may vary due to different protocols.)

· Input preset value in the column. Click Add preset button, you have added one preset in the tour.

Note:

Repeat the above procedures you can add more presets in one tour.

Or you can click delete preset button to remove one preset from the tour.

	Pattern 
	You can input pattern value and then click start button to begin PTZ movement. Please go back to Figure 1‑10 to implement camera operation. Then you can click stop button in Figure 1‑12. Now you have set one pattern.

	Assistant 
	Please input the corresponding aux value here. 

You can select one option and then click AUX on or AUX off button.

	Light and wiper 
	You can turn on or turn off the light/wiper.

	Goto 
	It is the accurate positioning function. Please input corresponding horizontal angle, tilt angle, and block camera zoom speed and then click Go to button to go to a specified position.  One unit of the horizontal angle or tile angle stands for 0.1 degree. 

	Rotation 
	Begin or stop block camera rotation movement. 


The Menu button can control the OSD for detailed block camera or PTZ setup. See Figure 1‑13.
[image: image17.png]PTZSettings  Menu





Figure 1‑13 
2 Setup
2.1 Camera 
2.1.1 Video 
2.1.1.1 Video bit stream 

The video bit stream interface is shown as below. See Figure 2‑1.

[image: image18.png]WEB SERVICEso

Camera Video Snapshat Overlay

Main Stream Sub Stream

Enable
Code-Siream Type | General
Code-Siream Type | General
Encade Mode e
Storage Encode Made H.264
Resolufion 1080P (1920°1080)
Em Resolution D1 (7047480)
Information Frame rale(FPS) )
Frame rale(FPS) 6
Bit Rate Type cER
Bit Rate Type cER
Reference Bif Rafe  4086-8192KbiS
Reference Bif Rate  192-1024K5
Bit Rate 8192
Bit Rate 1024
1 Frame Infervl &0 @0-150)
1 Frame Infervl 2 (6-150)

Watermark Settings

Watermark Character | DigitalCCTV

Default Reffesh





Figure 2‑1
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Main stream 
	Bit stream type 
	It includes general stream, motion stream and alarm stream. You can select different encode frame rates for different recorded events. 
The frame rates of the motion detect and alarm is customized. 

	
	Encode mode 
	There are four options: H.264, H.264B, H.264H, and MJPEG encode mode.

· H.264 : Main Profile encode mode. 

· H.264B :Baseline Profile encode mode.

· H.264H: High Profile encode mode.

· MJPEG: In this encode mode, the video needs general large bit stream to guarantee the video definition. You can use the max bit stream value in the recommend bit to get the better video output effect.  

	
	Resolution 
	There are multiple resolutions. You can select from the dropdown list. 

For each resolution, the recommended bit stream value is different. 

	
	Frame Rate (FPS)
	PAL: 1～25f/s，NTSC: 1～30f/s.. 
The frame rate may vary due to different resolutions. 

	
	Bit Rate Type 
	There are two options: VBR and CBR. 

Please note, you can set video quality in VBR mode. 
In MJPEG mode, the bit stream control mode can only be CBR. 

	
	Recommended Bit 
	Recommended bit rate value according to the resolution and frame rate you have set. 

	
	Bit Rate 
	· In CBR, the bit rate here is a  fixed value.  
· It is the max value in VBR mode. 

	
	I Frame 
	Here you can set the P frame amount between two I frames. The value ranges from 1 to 150.  Default value is 50.

Recommended value is frame rate *2.
Important

I frame interval setup is null if it is the MJPEG encode mode. 

	
	Watermark
	This function allows you to verify the video is tampered or not. 

The max length is 128-digit. The character can only include number, character, underline and hyphen. 

	Sub stream 
	Enable 
	Please check the box here to enable extra stream function.  
This function is enabled by default. 

	
	Bit stream type 
	General bit stream. 

	
	Encode mode 
	There are four options: H.264, H.264B, H.264H, and MJPEG encode mode.

· H.264 : Main Profile encode mode. 

· H.264B :Baseline Profile encode mode. H.264B is mainly for Blackberry cell phone to realize the monitor. You need to enable the sub stream function in your camera and set the resolution as CIF. Then you can monitor via the Blackberry cell phone.
· H.264H: High Profile encode mode. 
· MJPEG: In this encode mode, the video needs general large bit stream to guarantee the video definition. You can use the max bit stream value in the recommend bit to get the better video output effect.   

	
	Resolution 
	There are multiple resolutions. You can select from the dropdown list. 

For each resolution, the recommended bit stream value is different.

	
	Frame Rate (FPS)
	PAL: 1～25f/s，NTSC: 1～30f/s.. 
The frame rate may vary due to different resolutions. 

	
	Bit Rate Type 
	There are two options: VBR and CBR. 

Please note, you can set video quality in VBR mode. 

	
	Recommended Bit 
	Recommended bit rate value according to the resolution and frame rate you have set. 

	
	Bit Rate 
	· In CBR, the bit rate here is a max value. In dynamic video, system needs to lower frame rate or video quality to guarantee the value. 

· It is null in VBR mode. 

· Please refer to recommended bit rate for the detailed information.

	
	I Frame 
	Here you can set the P frame amount between two I frames. The value ranges from 1 to 150.  Default value is 50.

Recommended value is frame rate *2.
Important

I frame interval setup is null if it is the MJPEG encode mode.


2.1.1.2 Snapshot 
The snapshot interface is shown as in Figure 2‑2.
[image: image19.png]Network
Event
Storage
System
Information

Videa Snapshot

Snapshot Type | General
Image Size
Quality

Interval

¥]
¥]

Overlay

Default

Reffesh





Figure 2‑2
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Snapshot type 
	There are two modes: general (schedule) and Event (activation).
General (schedule) snapshot is to snap in the specified period. 

Event (Activation) snapshot Is to snap when the motion detect, camera masking, local alarm event occurrence. 

If you want to use this function, please make sure:

· The event occurred during the specified period.

· Motion detect, video masking, local alarm and corresponding snap function are all enable. 

	Image size 
	It has relationship with the resolution of the main stream. 

	Quality 
	It is to set the image quality. There are six levels. 

	Interval 
	It is to set snapshot frequency. The value ranges from 1s to 7s. 


2.1.1.3 Video Overlay 
The video overlay interface is shown as in Figure 2‑3.
[image: image20.png]WEB SERVICE=s:

Camera Snapshat overlay
S S ABZEOE

[ Frivacy Masking

[ Channel Titi

Storage
System

] Tirme Tite

Information

Default Reffesh





Figure 2‑3
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Privacy mask
	· Here you can privacy mask the specified video in the monitor video. 

· System max supports 4 privacy mask zones.

	Time Title
	· You can enable this function so that system overlays time information in video window. 
· There is no time title if you do not enable this function here. 
· You can use the mouse to drag the time tile position. 

	Channel Title 
	· You can enable this function so that system overlays channel information in video window.
· There is no channel title if you do not enable this function here. 
· You can use the mouse to drag the channel tile position.

	Refresh 
	After you successfully set privacy mask zone, channel title, time title, you can click Refresh button to video the effect. 


2.1.1.4 Path 

The storage path interface is shown as in Figure 2‑4.
Here you can set snap image saved path ([image: image21.png]


 in the preview interface) and the record storage path ([image: image22.png]


 in the preview interface).The snap picture default setup is C:\PictureDownload and record file default setup is C:\RecordDownload. 
Please click the Save button to save current setup. 
[image: image23.png]WEB SERVICEso

Camera Snapshat Overlay

> Audio

Snapshot Path | C:\PictureDownload Browse.
Network

Record Path | C:\RecordDownload Browse.
Event

Storage Default

System
Information





Figure 2‑4
2.1.2 Audio 
The audio interface is shown as below. See Figure 2‑5.
Important 

Please make sure you have enabled the video function, otherwise you can not enable the audio function. 

[image: image24.png]WEB SERVICEso

Camera Audio
;
Network Enable

Event Encode Mode
Storage

Sub Stream

Enatile

Encode Made GT11A

system
u Default

Reffesh

Information





Figure 2‑5
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Audio enable 
	· Main stream: Recorded file only contains video by default. You need to check the audio box here to enable audio function so that the main stream the network transmitted is audio/video composite stream. 
· Sub (Extra) stream: Recorded file only contains video by default. You need to check the audio box here to enable audio function so that the extra stream the network transmitted is audio/video composite stream. 

	Encode mode 
	The encode mode of the main stream and extra stream include PCM, G.711A and G.711Mu. 
The setup here is for audio encode mode and the bidirectional talk encode both. 


2.2 Network 

2.2.1 TCP/IP

The TCP/IP interface is shown as in Figure 2‑6.
[image: image25.png]WEB SERVICEso

Camera TCPIP
Network

HostName 1PDorme
Connection Ethernet Card Wire( DEFAULT)
PPPOE Mode © static O DHER
oD MAC Address
IF Fil 1P Version
MTP(E-mai) 1P Address

Ura Subnet mask

WP Default Gateway
Bonjour Preferred DNS Server

Alternate DNS Server

Enable ARPPing to set P address service,

Default Reffesh

Storage
System
Information





Figure 2‑6
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Host Name 
	It is to set current host device name. 

	Ethernet Card
	Please select the Ethernet port if the device has several network cards. 

	Mode
	There are two modes: static mode and the DHCP mode. 

· The IP/subnet mask/gateway are null when you select the DHCP mode to auto search the IP. 

· If you select the static mode, you need to set the IP/subnet mask/gateway manually.

· Besides, IP/subnet mask/gateway and DHCP are read-only when the PPPoE dial is OK. 

	Mac Address 
	It is to display host Mac address. It is read-only. 

	IP Version 
	It is to select IP version. IPV4 or IPV6.
You can access the IP address of these two versions.
· Please note system needs to check the validity of all IPv6 addresses. The IP address and the default gateway shall be in the same IP section. That is to say, the specified length of the subnet prefix shall have the same string. 
· When PPPoE function is enabled, the IP/subnet mask/default gateway is read-only. You can not set or restore default setup.  

	IP Address 
	Please use the keyboard to input the corresponding number to modify the IP address and then set the corresponding subnet mask and the default gateway. 

	Default Gateway 
	It is the similar gateway of the IPv4. It shall not be left in blank. 

	Preferred DNS
	DNS IP address. It is the similar DNS of the IPv4. It shall not be left in blank.

	Alternate DNS 
	Alternate DNS IP address. It is the similar DNS of the IPv4. It shall not be left in blank.

	Enable ARP/Ping set device IP address service. 
	You can use ARP/Ping command to modify or set the device IP address if you know the device MAC address. 
Before the operation, please make sure the block camera and the PC in the same LAN. This function is on by default. 

You can refer to the steps listed below. 
Step 1: Get an IP address. Set the block camera and the PC in the same LAN.

Step 2: Get the physical address from the label of the block camera.

Step 3: Go to the Run interface and then input the following commands. 

arp  –s  <IP Address>  <MAC>  

ping  –l  480  –t  <IP Address>
Such as：arp  -s  192.168.0.125  11-40-8c-18-10-11

ping  -l  480  -t  192.168.0.125

Step 4: Reboot the device. 

Step 5: You can see the setup is OK if you can see there are output information such as “Reply from 192.168.0.125 …” from the command output lines.  Now you can close the command line. 

Step 6: Open the browse and then input http://<IP  addres>. Click the Enter button, you can access now. 


2.2.2 Connection 
The connection interface is shown as in Figure 2‑7.

[image: image26.png]WEB SERVICE=s«

Camera Connection
Network

Max C
> TCPIP

= t

> prPoE UDF Port
> DDNS

> IPFilter

> SHTP(E-mail)
> UPnP

> s

> Bonjour

> Mulicast

> IEEERD:

> aos

Event
Storage
System
Information

T ST TR AT





Figure 2‑7
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Max connection 
	It is the max Web connection for the same device. The value ranges from 1 to 20. The default setup is 10.

	TCP port 
	The value ranges from 1025 to 65535. The default value is 37777. You can input the actual port number if necessary. 

	UDP port 
	The value ranges from 1025 to 65535. The default value is 37778. You can input the actual port number if necessary.

	HTTP port 
	The value ranges from 1025 to 65535. The default value is 80. You can input the actual port number if necessary.

	RTSP port
	· Usually, the default value is 554. You do not need to input again if you are using the default value. When you are using QuickTime (Apple browser) or VLC play real-time video, you can use the following format to play. The Blackberry also supports this function. 

· Real-time monitor bit stream Url format. Please specify the channel number, bit stream type in the Url if you are requesting real-time monitor bit stream Rtsp stream media service. You still need to provide user name or password if it has verification information. 

· When you are using Blackberry phone to access, the bit stream mode shall be H.264B, resolution is CIF and the audio shall be disabled. 
· The Url format is shown as below:
rtsp://username:password@ip:port/cam/realmonitor?channel=1&subtype=0
You need to input the following items manually. 

Username/password/IP/port/subtype. 

The IP is device IP and the port default value is 554. You can leave it in blank if it is the default value. The channel number begins with 1. 
subtype：bit stream type，main stream is 0（subtype=0）and extra stream is 1（subtype=1）.
You do not need to input the user name and password if you do not need  the verification. Such as:

Main stream：rtsp://ip:port/cam/realmonitor?channel=1&subtype=0

	HTTPS Enable  
	It is to enable HTTPs communication service control. If you enable this function, you can use https://ip:port to login the device. In data encryption protection mode, you can use https://ip to login if you are using the default port. 

	HTTPS Port 
	The HTTPs communication port value ranges from 1025 to 65535. 



Important

The value 0 to 1024 (excluding the default value of HTTP/RTSP/HTTPs), 1900, 3800, 5000, 5050, 9999, 37776, 37780-37880, 39999 and etc are the special ports value and they are not open for user to set. 
2.2.3 PPPoE

The PPPoE interface is shown as in Figure 2‑8.
Input the PPPoE user name and password you get from the IPS (internet service provider) and enable PPPoE function. Please save current setup and then reboot the device to get the setup activated. 

Device connects to the internet via PPPoE after reboot. You can get the IP address in the WAN from the IP address column.
PPPoE is set to connect to the internet. You can get an account from your IPS (Internet service provider), you can set here to dial to the interface. You can see the registered IP address in the interface if your setup is right. 
Please note, you need to go to the IP address item to view the device current device information. You can access the client-end via this address. 
[image: image27.png]Camera
Network

> TCPIP

> Connection
>

> DDNS

IP Fiter

> SMTR(E-mail)

> UPnP
> s

> Bonjour

> Mulicast

> IEEERD:

> aos
Event
Storage
System
Information




Figure 2‑8
2.2.4 DDNS
The DDNS interface is shown as in Figure 2‑9.
The DDNS is to set to connect the various servers so that you can access the system via the server. Please go to the corresponding service website to apply a domain name and then access the system via the domain. It works even your IP address has changes. 
[image: image28.png]WEB SERVICEso

T ST TR AT

Camera
Network

> TCPIP

> Connection
> PPPOE

> Dormain Name | none
> IP Filter

> SMTP(E-mail)
> UPnP

> anip

> Bonjour

> Muticast

> IEEERD:
> aos
Event
Storage
System
Information




Figure 2‑9
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Server Type
	You can select DDNS protocol from the dropdown list and then enable DDNS function. 

	Server IP
	DDNS server IP address

	Server Port 
	DDNS server port.

	Domain Name 
	Your self-defined domain name.

	User 
	The user name you input to log in the server.

	Password
	The password you input to log in the server.

	Update period 
	· Device IP and service connection refresh period. 

· The default setup is 10 minutes. 


2.2.5 IP filter
The IP filter interface is shown as in Figure 2‑10.
You can enable IP filter function so that some specified IP user can access the block camera. 
You can add IP address or IP address section. 

If you do not check the box here, it means there is on access limit. 
[image: image29.png]IP Filter

Trusted Sites

Trusted Sites
1P address.

UPnP
NP
B0

Remove All

Default Reffesh

Event

Storage

System

Information





Figure 2‑10
2.2.6 SMTP （e-mail）
The SMTP interface is shown as in Figure 2‑11.

[image: image30.png]WEB SERVICEso

Camera SMTP(E-mail)
Network
SMTP Server

> TCPIP cort

Anonymity
Usernarme.
> DN anonymity

Password | eese
> IP Filter

Sender
SMTP(E-mail) none

UPnP Adtheriication | None
e Tite SD Message Atachment

Bonjour Mail Receiver

Mutia:

IEEE!

Interval i Second(0~3600)

& O HealtnMail  Update Periog Second (1-3600)

System Emall Test

Information
Default Reffesh





Figure 2‑11
Please refer to the following sheet for detailed information.

	Parameter 
	Function 

	SMTP Server 
	Input server address and then enable this function. 

	Port 
	Default value is 25. You can modify it if necessary.

	Anonymity 
	For the server supports the anonymity function. You can auto login anonymously. You do not need to input the user name, password and the sender information.  

	User Name
	The user name of the sender email account.

	Password
	The password of sender email account.

	Sender
	Sender email address.


	Authentication (Encryption mode) 
	You can select SSL or none. 

	Title (Subject)
	Input email subject here. 


	Attachment 
	System can send out the email of the snapshot picture once you check the box here. 

	Mail receiver
	Input receiver email address here. Max three addresses.


	Interval 
	The send interval ranges from 0 to 3600 seconds. 0 means there is no interval.

Please note system will not send out the email immediately when the alarm occurs. When the alarm, motion detection or the abnormity event activates the email, system sends out the email according to the interval you specified here. This function is very useful when there are too many emails activated by the abnormity events, which may result in heavy load for the email server.

	Health mail enable 
	Please check the box here to enable this function. 



	Update period (interval) 
	This function allows the system to send out the test email to check the connection is OK or not. 

Please check the box to enable this function and then set the corresponding interval. 

System can send out the email regularly as you set here. 

	Email test 
	The system will automatically sent out an email once to test the connection is OK or not .Before the email test, please save the email setup information. 


2.2.7 UPnP
It allows you to establish the mapping relationship between the LAN and the public network.
Here you can also add, modify or remove UPnP item. See Figure 2‑12.

In the Windows OS, From Start->Control Panel->Add or remove programs. Click the “Add/Remove Windows Components” and then select the “Network Services” from the Windows Components Wizard. 
Click the Details button and then check the “Internet Gateway Device Discovery and Control client” and “UPnP User Interface”. Please click OK to begin installation. 
Enable UPnP from the Web. If your UPnP is enabled in the Windows OS, the block camera can auto detect it via the “My Network Places”
[image: image31.png]WEB SERVICEso

Camera
Network

> TCPIP

> Connection
> PPPOE

> DDN

> P

> SMTP(E-mail)

> Muttc
> IEES
>

Storage
System
Information

UPnP

[ Enable Status : Mapping Failed

Port Mapping List

Service Name

Protocol

Internal Port

External Port

Add Mapping

Reffesh





Figure 2‑12
2.2.8 SNMP
The SNMP interface is shown as in Figure 2‑13.
The SNMP allows the communication between the network management work station software and the proxy of the managed device. Please install the software such as MG MibBrowser 8.0c software or establish the SNMP service before you use this function. You need to reboot the device to activate the new setup. 
[image: image32.png]WEB SERVICE=s:

Camera SNMP
Network

> TCPIP

> Connection
> PPPOE

> DDNS

IP Fiter
> SMTP(E-mail)
> UPnP

> Bonjour
> Mulicast

> IEEERD:

> aos
Event
Storage
System
Information

161
public

1
[ snmPvt

[
[mENT%

O snp

T ST TR AT





Figure 2‑13
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	SNMP Port 
	The listening port of the proxy program of the device. It is a UDP port not a TCP port. The value ranges from 1 to 65535. The default value is 161

	Read Community 
	It is a string. It is a command between the manage process and the proxy process. It defined the authentication, access control and the management relationship between one proxy and one group of the managers. Please make sure the device and the proxy are the same. 
The read community will read all the objects the SNMP supported in the specified name. The default setup is public.

	Write Community 
	It is a string. It is a command between the manage process and the proxy process. It defined the authentication, access control and the management relationship between one proxy and one group of the managers. Please make sure the device and the proxy are the same. 

The read community will read/write/access all the objects the SNMP supported in the specified name. The default setup is write.

	Trap address
	The destination address of the Trap information from the proxy program of the device. 



	Trap port 
	The destination port of the Trap information from the proxy program of the device. It is for the gateway device and the client-end PC in the LAN to exchange the information. It is a non-protocol connection port. It has no effect on the network applications. It is a UDP port not TCP port. The value ranges from 1 to 165535. The default value is 162.

	SNMP version 
	· Check SNMP v1, system only processes the information of V1.
· Check SNMP v2, system only processes the information of V2.

· Check SNMP v3, you can set account and password. You need to set the corresponding account and password for security verification when the server wants to access the device. At the same time, the V1 and V2 option is null.  


2.2.9 Bonjour
The Bonjour interface is shown as below. See Figure 2‑14.

Bonjour is based on the multicast DNS service from the Apple. The Bonjour device can automatically broadcast its service information and listen to the service information from other device.
You can use the browse of the Bonjour service in the same LAN to search the block camera and then access if you do not know the block camera information such as IP address.  
You can view the server name when the block camera is detected by the Bonjour. Please note the safari browse support this function. Click the “Display All Bookmarks: and open the Bonjour, system can auto detect the block camera of the Bonjour function in the LAN. 
[image: image33.png]Camera Bonjour,

Network

>

>

>

>

>

TCRIP Enatile
Cannection
PPPOE
DDNS

IP Fiter
SMTP(E-mail)
UPnP

SNMP.

Multicast
IEEEBD2
Qos

Event

Storage

System

Information




Figure 2‑14
2.2.10 Multicast
The multicast interface is shown as in Figure 2‑15.
Multicast is a transmission mode of data packet. When there is multiple-host to receive the same data packet, multiple-cast is the best option to reduce the broad width and the CPU load.  The source host can just send out one data to transit. This function also depends on the relationship of the group member and group of the outer. 
Here you can set multicast address and port. You also need to go to Live interface to set the protocol as Multicast. 
[image: image34.png]WEB SERVICEso

Camera Multicast
Network

> TCPIP

> Connection
> PPPOE

> DDNS

> IP Filter

> SMTR(E-mail)
> UPnP

> anip

> Bonjour

> IEEEaD:
> aos
Event
Storage
System
Information





Figure 2‑15
2.2.11 IEEE802
IEEE802.1X works standing for local and metropolitan area networks and port based network access control protocol. It supports manual operation of the client to choose means of authenticating by which to control it to access to the Local Area Networks or not. It supports the ability to authenticate, to calculate fee, to ensure security and to maintain requirements. See Figure 2‑16.

[image: image35.png]Camera IEEES02
Network
> TcRiP Enable

> Gonnection Authentication
> FPROE

> DDNS

> IPFilter

> SMTR(E-mail)
> UPnP

> s

> Bonjour

> Mulicast

>

> aos

Event
Storage
System
Information

T ST TR AT





Figure 2‑16
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Authentication
	PEAP (protected EAP protocol).  

	Username
	It needs the username to login, which is authenticated by the server.

	Password
	Please input password here. 


2.2.12 Qos

The QoS interface is shown as below. See Figure 2‑17.
Qos (Quality of Service) is network security mechanism. It is a technology to fix the network delay and jam problem and etc. For the network service, the quality of service includes the transmission bandwidth, delay, the packet loss and etc. We can guarantee the transmission bandwidth, lower the delay, and reduce the loss of the data packet and anti-dither to enhance the quality. 
We can set the DSCP (Differentiated Services Code Point) of the IP to distinguish the data packet so that the router or the hub can provide different services for various data packets. It can select the different queues according to the priority of the packets and select the bandwidth of the each queue. It can also discard at the different ratio when the broad bandwidth is jam.
[image: image36.png]WEB SERVICEso

Camera Qos

Network
Realtime Monitar
> TCPIP

Cammand

> Connestion
> FPROE

> DDNS

> IPFilter

> SHTP(E-mail)
> UPnP

> s

> Bonjour

> Mulicast

> IEEEaD:

>

Event
Storage
System
Information





Figure 2‑17
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Real-time monitor 
	The data packet of the network video monitor.  

	Command 
	The non-monitor packet such as device setup and search. 


2.3 Event 

2.3.1 Video detect

2.3.1.1 Motion Detect 
The motion detect interface is shown as in Figure 2‑18.
[image: image37.jpg]WEB SERVICESs»

Camera Motion Detect Video Masking
Network
Event Enable

Working Period

2 dam Anti-Dither Second(0~100)  Sensitiity |3

> Abnormaiity
Area

storage
system Record

Information Record Delay Second(10~300)

Relay-out
Alarm Delay Second(10~300)

Send E-Mail

Pz Actvation|None ] Address| o

Snapshat

Default Refresh.





Figure 2‑18
[image: image38.png]Working Period

Period 1
[ Period 2
[ Period 3
[ Period 4
[ Period 5
[ Period &

Sunday

[ii]
[ii]
[ii]
[ii]
[ii]
[ii}

[ii]
[ii]
[ii]
[ii]
[ii]
[ii}

[ Monday [
w - s
w - s
w - s
w - s
w - s
w H =

setup

setup

setup

setup

setup

"

setup

Tuesday [] Wednesday [] Thursday [] Friday [] Saturday

59
59
59
59
59
59

59
59
59
59
59
59

save

cancel





Figure 2‑19
[image: image39.png]



Figure 2‑20
Please refer to the following sheet for detailed information. 
	Parameter
	Function 

	Enable 
	You need to check the box to enable motion detection function. 

	Sensitivity 
	There are six levels. The sixth level has the highest sensitivity.

	Region 
	· There are six levels.  The sixth level has the highest sensitivity.

· Region: If you select motion detection type, you can click this button to set motion detection zone.  The light blue zones are the valid motion detect area. All area are the valid motion detect zone by default. You can use mouse to set invalid area.  
· Do remember clicking OK button to save your motion detection zone setup.

	Working Period 
	· Motion detection function becomes activated in the specified periods. See Figure 2‑19.
· There are six periods in one day. Please draw a circle to enable corresponding period.

· Select date. If you do not select, current setup applies to today only. You can select all week column to apply to the whole week.

· Click OK button, system goes back to motion detection interface; please click save button to exit.

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  
Please note the relay output number here is for reference only. The alarm output number may vary due to different series products.

	Alarm Delay
	System can delay the alarm output for specified time after motion detect alarm ended. The value ranges from 10s to 300s. 

	Record channel
	System auto activates motion detection channel to record once motion detect alarm occurs (working with motion detection function). Please note you need to go to Storage-> Schedule to set motion detect record period and go to Storage->Record control to set current channel as auto record.

	Record Delay 
	System can delay the record for specified time after motion detect alarm ended. The value ranges from 10s to 300s.

	Send Email 
	If you enabled this function, System can send out email to alert you when alarm occurs and ends.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.
· The event type includes: preset, tour and pattern and etc. 

	Snapshot 
	It is snapshot activation function.


2.3.1.2 Video Masking 
The video masking interface is shown as in Figure 2‑21.

[image: image40.jpg]Camera
Network
Event

> Alam
> Abnormality
storage
system
Information

Motion Detect

Enable

Working Period

Record

Record Delay

Relaj-out

Alam Delay

Send E-Mail
F1Z

Snapshot

Video Masking

Second(10~300)

Secand(10~300)

Actaton[ e [v] Aess[ 0

Default Reffesh





Figure 2‑21
Please refer to the following sheet for detailed information. 
	Parameter
	Function 

	Enable 
	You need to check the box to enable video masking function. 

	Sensitivity 
	There are six levels. The sixth level has the highest sensitivity.

	Working Period 
	· Video masking function becomes activated in the specified periods. 

· There are six periods in one day. Please draw a circle to enable corresponding period.

· Select date. If you do not select, current setup applies to today only. You can select all week column to apply to the whole week.

· Click OK button, system goes back to motion detection interface; please click save button to exit.

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Relay out
	Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when video masking alarm occurs.  
Please note the relay output number here is for reference only. The alarm output number may vary due to different series products.

	Alarm Delay 
	System can delay the alarm output for specified time after video masking alarm ended. The value ranges from 10s to 300s. 

	Record channel
	System auto activates motion detection channel to record once video masking alarm occurs (working with motion detection function). Please note you need to go to Storage-> Schedule to set motion detect record period and go to Storage->Record control to set current channel as auto record.

	Record Delay 
	System can delay the record for specified time after video masking alarm ended. The value ranges from 10s to 300s.

	Email 
	If you enabled this function, System can send out email to alert you when alarm occurs.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.
· The event type includes: preset, tour and pattern and etc. 

	Snapshot 
	It is snapshot activation function. 


2.3.2 Alarm 
2.3.2.1 Alarm activation 

The alarm activation interface is shown as in Figure 2‑22.

[image: image41.jpg]WEB SERVICESso

Camera Relay Activation Relay-out

Network

Enable
Event

b Relayin Alamt

> Working Period |, Setup

> Abnomality Anti-Dither Second(0~100) Sensor Type NO

Storage

Syetent Record

Information Record Delay Second(10-300)

Relay-out

Alarm Delay Second(10~300)

Send E-Hail

P12 Actvation| None ] address 0

Snapshot

Default





Figure 2‑22
Please refer to the following sheet for detailed information. 
	Parameter
	Function 

	Enable 
	You need to check the box to enable this function. 

	Working Period 
	· This function becomes activated in the specified periods. 

· There are six periods in one day. Please draw a circle to enable corresponding period.

· Select date. If you do not select, current setup applies to today only. You can select all week column to apply to the whole week.

· Click OK button, system goes back to motion detection interface; please click save button to exit.

	Anti-dither
	System only memorizes one event during the anti-dither period. The value ranges from 0s to 100s.

	Sensor type 
	There are two options: NO/NC. From NO to NC, system enables alarm. From NC to NO, system disables alarm. 

	Relay out
	· Enable alarm activation function. You need to select alarm output port so that system can activate corresponding alarm device when alarm occurs.  
· Please note the relay output number here is for reference only. The alarm output number may vary due to different series products.

	Alarm Delay
	System can delay the alarm output for specified time after alarm ended. The value ranges from 10s to 300s. 

	Record Channel
	System auto activates motion detection channel to record once alarm occurs (working with motion detection function). Please note you need to go to Storage-> Schedule to set current channel as general record.

	Record Delay
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Send Email 
	If you enabled this function, System can send out email to alert you when alarm occurs and ends.

	PTZ 
	· Here you can set PTZ movement when alarm occurs. Such as go to preset x when there is an alarm.
· The event type includes: preset, tour and pattern. 

	Snapshot 
	You need to input capture channel number so that system can backup motion detection snapshot file.


2.3.2.2 Relay output 

The relay output interface is shown as in Figure 2‑23.
[image: image42.jpg]Camera
Network
Event

> Video Detect

> Abnormaiity

storage

system

Information

Relay Activation

Relay-out

Trigger





Figure 2‑23
Please refer to the following sheet for detailed information. 

	Parameter 
	Function 

	Alarm output
	There are two output channels. Please click the corresponding button. If you want to enable the alarm activation output function, please press the corresponding button and then trigger. 

Please note the interface here is for reference only. The alarm output number may vary due to different series products. 

	Trigger 
	When the alarm output channel is open, the output port can immediately output an alarm once it occurred. 

	Refresh 
	Refresh alarm output status. 


2.3.3 Abnormity

It includes five statuses: No SD card, capacity warning, SD card error, and disconnection and IP conflict. There are two interfaces for you reference. See Figure 2‑24 through Figure 2‑28.

[image: image43.jpg]WEB SERVICESso

Camera No SD Card Capacity Warning SD Card Error Disconnection 1P Conflict

Network -
et Enable

> Video Detect Relay-out

> Alam Relaj-outDelay 19 | second(10~300)

Send E-Hail

Default Refresh

system

Information





Figure 2‑24
[image: image44.jpg]WEB SERVICE=so

Camera No SD Card Capacity Warning SD Card Error Disconnection 1P Conflict
Network

o O Enable

e CapasiyLimit| 10

> Alam Relay-out

> Relay-outDelay |10 Second(10~300)
Storaye Send E-Mail
system

Refresh.

Information





Figure 2‑25
[image: image45.jpg]WEB SERVICESso

Camera No SD Card Capacity Warning SD Card Error Disconnection IP Conflict
Network

Event O Enable

> Video Dete Relaj-out

> Aam Relaj-outDelay | 10 | second(10~300)

Send E-blail
storage

Default

system

Information





Figure 2‑26
[image: image46.jpg]WEB SERVICESs»

Camera No SD Card Capacity Warning D Card Error Disconnection 1P Conflict
Network

Evert O Enable

> Video Detect Record

> Aam RecordDelay [ 10 Second(10~300)

Relaj-out

Storage Relsj-outDelay [ 10 Second(10~300)

system

Default Refresh

Information





Figure 2‑27
[image: image47.jpg]WEB SERVICESso

Camera No SD Card Capacity Warning SD Card Error Disconnection IP Conflict
Network

Event O Enable

> Video Dete Record

> Aam RecordDelay |10 | secondto-200)

Relay-out

Storage Relay-outDelay |10 Second(10~300)
system

Information Defaut, Reffesh





Figure 2‑28
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Event Type 
	· The abnormal events include: no disk, no space, disk error, net error, offline, IP conflict. 
· You need to draw a circle to enable this function. 

	Record
	System auto activates channel to record once an alarm occurs (For offline type only. See Figure 2‑28. ). 
You need to check the box to enable this function. 

	Record Delay
	System can delay the record for specified time after alarm ended. The value ranges from 10s to 300s.

	Relay Out
	The corresponding alarm output channel when alarm occurs. You need to check the box to enable this function. 

	Relay out Delay 
	The alarm output can delay for the specified time after alarm stops. The value ranges from 10s to 300s.

	Send email 
	If you enable this function, system can send out email to alarm the specified user. 


2.4 Storage 
2.4.1 Record schedule and snapshot schedule 
In these two interfaces, you can add or remove the schedule record/snapshot setup. See Figure 2‑29.
There are three record modes: general (auto), motion detect and alarm. There are six periods in one day. Please make sure you have enabled the corresponding record mode in the Setup->Storage->Conditions. 
You can view the current time period setup from the color bar. 

· Green color stands for the general record/snapshot.

· Yellow color stands for the motion detect record/snapshot.
· Red color stands for the alarm record/snapshot. 

[image: image48.png]Camera
Network
Event

Storage

Schedule.

> Record Cantral

System

Information

Sunday
Manday
Tuesday
Wednestay
Thursday
Friday

Saturday

M General  Motion M Alarm

12 14 18 18 20 22 24

Default

Reffesh





Figure 2‑29
2.4.2 Destination 

The destination interface is shown as in Figure 2‑30.

It is to set the storage mode of the block camera record file or snapshot pictures. There are two options: local storage/FTP. You can only select one mode. System can save according to the event types. It is corresponding to the three modes (general/motion/alarm) in the Schedule interface. Please check the box to enable the save functions. 
[image: image49.png]WEB SERVICESs:

Camera
Network
Event
Storage
system
Information

Path

Record
Event Type
Local

FTP.

Motion Detect
2
=

Default

Reffesh

Snapshot
Event Type
Local

FTP.

Hotion Dete
2
=





Figure 2‑30
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Event Type 
	It includes: general, motion detect and alarm.  

	Local 
	It is to save in the SD card. 

	FTP 
	It is to save in the FTP server. 


The local interface is shown as in Figure 2‑31. Here you can view local SD card information. You can also operate the read-only, write-only, hot swap and format operation. 
[image: image50.png]Camera
Network
Event

Storage

> Schedule
Destination

> Record Control

system

Information

Free Capacity/Total Capacity

Read Only

Read & Wite.

Hot Swap

Reffesh





Figure 2‑31
The FTP interface is shown as in Figure 2‑32. You need to check the box to enable the FTP function. When network disconnect occurred or there is malfunction. Emergency storage can save the record/snapshot picture to the local SD card. 
[image: image51.png]WEB SERVICEso

Camera
Network
Event

Storage

> Schedule

> Record ¢
System
Information

Enatile
Server P

Fort 2

User Narme. anonymity
Password

Remote directary

Emergency (Local)

(0~65535)

Default

Reffesh





Figure 2‑32
2.4.3 Record control 
The record control interface is shown as in Figure 2‑33.
[image: image52.png]WEB SERVICEso

Camera Record Control
Network

PackDuraion |60 Minute (1+120)
Event

Pre-eventRetord |5 Second (0+5)
Storage

DiskFul Overwite &

Record Mode @ Automatic O Manual O Off

Recordsteam | wain steam ]
Resord Conirl

System Default Refresh
Information





Figure 2‑33
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Pack Duration
	Here you can select file size. Default setup is 60 minutes.

	Pre-record 
	Please input pre-record value here. 

For example, you can input 4 here so that system can read the previous four seconds video before the alarm occurrence from the buffer and record the 4 seconds video in the file. 
Please note, if there is no record when alarm record or motion detect record occurred, system can record the N seconds’ video before the event occurrence in the file. 

	Disk Full
	There are two options: stop recording or overwrite the previous files when HDD is full.
· Stop: Current working HDD is overwriting or current HDD is full, it will stop record. 

· Overwrite: Current working HDD is full; it will overwrite the previous file.

	Record Mode 
	There are three modes: Auto/manual/close.  

	Record Bit Stream 
	You can select main stream or extra stream. 


2.5 System 
2.5.1 General 
The general interface includes the general setup and the date/time setup. 
2.5.1.1 General
The general interface is shown as in Figure 2‑34.
[image: image53.png]WEB SERVICE®: (ST ST TN IR

Camera General
Network
Event

Storage

[

System
;

> PTZ Setings

> Default
> ImporiExport

> Auto Maintain

Ungrade
Information




Figure 2‑34
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Device Name
	It is to set device name. 

	Video Standard
	This is to display video standard.

	Language 
	You can select the language from the dropdown list. 
Please note the device needs to reboot to get the modification activated. 


2.5.1.2 Date and time 
The date and time interface is shown as in Figure 2‑35
[image: image54.png]WEB SERVICEso

Camera General

Network

Date Format
Event

Time Format
Storage

i Time Zone

Current Time
General

DST Enable
DST Type
Start Time

End Time

Date&Time

VearHoritDay ]
24-Hourbased systern ||
o080 5

012- 06 - 13 [16:39: 11

ODate @ Wesk
s [w][1st_[v][5undey [] 00 - 00

s [w][2na _[][Monsay []00 - 00

Synehronize with NTF

NTP Server

Information
Port

clockisc.org

123

Update Period 60 Minute(0-30)

Default Reffesh





Figure 2‑35
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Date format
	Here you can select date format from the dropdown list.

	Time Format
	There are two options: 24-H and 12-H.


	Time zone 
	The time zone of the device. 



	System time 
	It is to set system time. It becomes valid after you set. 

	Sync PC
	You can click this button to save the system time as your PC current time. 

	DST
	Here you can set day night save time begin time and end time. You can set according to the date format or according to the week format. 

	NTP  
	You can check the box to enable NTP function.  

	NTP server 
	You can set the time server address. 

	Port 
	It is to set the time server port. 

	Update period 
	It is to set the sync periods between the device and the time server. 
The update function is null if the value is 0. 


2.5.2 Account 
Note:
· For the character in the following user name or the user group name, system max supports 6-digits. The space in the front or at the end of the string is null. The valid string includes: character, number, and underline. 

· The max user amount is 20 and the max group amount is 8. You can add or delete user group. 

· The factory default setup includes two levels: user and admin. .User management adopts group/user modes. The user name and the group name shall be unique. One user shall be included in only one group. 
2.5.2.1 User name 
In this interface you can add/remove user and modify user name. See Figure 2‑36.
[image: image55.png]WEB SERVICEso

Camera Account

sbucri User Group

Eveny User Name Remark
Storage admin adrmin s account
System 833888 998888 's account

P — 666666 666666 's account

Upgrade

Information

Authority List
Live Flayback Record control Backup FTZ Account Alarm Log Search
ClearLag Upgrade Auto Maintain General VideofAudio Schedule/Destination Network Apnormality
Video Detect FTZ Settings DefaulfimportExport. Conditions

Add User





Figure 2‑36
Add user: It is to add a name to group and set the user rights. See Figure 2‑37.
Here you can input the user name and password and then select one group for current user. 
Please note the user rights shall not exceed the group right setup. 

For convenient setup, please make sure the general user has the lower rights setup than the admin. 

[image: image56.png]Add User ]

UserName
Password

Confirm Password

Group. [sgmin |
Remark

Authority List P Al

¥ ShutdonnReboot 4
W Lie

¥ Record control
w strage
wetz

save Cancel





Figure 2‑37
Modify user 
It is to modify the user property, belonging group, password and rights. See Figure 2‑38.
Modify password 
It is to modify the user password. You need to input the old password and then input the new password twice to confirm the new setup. Please click the OK button to save. 

For the user of the account rights, he can modify the password of other users. 

[image: image57.png]Modify User ]

User Name [sgmin |
W fidify Password

Old Password

New Password

Confirm Password

Group.

Remark
Autnonty List oA
7 ShutdonnReboot 4
" Lie
17 Record cotrol
i strage

W P12

save. Cancel





Figure 2‑38
2.5.2.2 Group
The group management interface can add/remove group, modify group password and etc. 

The interface is shown as in Figure 2‑39.

[image: image58.png]WEB SERVICEso

Camera Account

Network User Narme
Event
Storage

System

> Default
> ImportE
> Auto Maintain

> Upgrade

Group

Group Name
admin
user

Remark
administrator group
user group

Information

Authority List
Live
ClearLag
Video Detect

Flayback Record control
Upgrade Auto Maintain
PTZ Settings

Backup FTZ Account Alarm
General VideofAudio Schedule/Destination Network

DefaulfimportExport. Conditions

Log Search
Apnormality

Add Group





Figure 2‑39
Add group: It is to add group and set its corresponding rights. See Figure 2‑40.
Please input the group name and then check the box to select the corresponding rights. It includes: live playback, record control, backup, PTZ control, user management and etc. 
[image: image59.png]Add Group

Group.
Remark
Authority List Al
I~ Shutdown/Reboot
I~ Live
I~ Record control
I™ Storage
[mlard

save.

Cancel





Figure 2‑40
Modify group 

Click the modify group button, you can see an interface is shown as in Figure 2‑41.

Here you can modify group information such as remarks and rights. 
[image: image60.png]Modify Group

Group. admin -

Remark administrator group

Authority List P Al

7 Shutdown/Reboot
W Live

¥ Record control
 Storage

¥ Tz

save Cancel





Figure 2‑41
2.5.3 PTZ 
The PTZ interface includes two interfaces. 

2.5.3.1 Network PTZ 
The network PTZ interface is shown as in Figure 2‑42.

[image: image61.png]WEB SERVICE®: (ST ST TN IR

Camera Network PTZ
Network

Pratocol | PEL

Event

System

> General

> Account
>

> Default
> ImporiExport

> Auto Maintain

Ungrade
Information




Figure 2‑42
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Protocol
	Select the corresponding dome protocol.


2.5.3.2 Analog PTZ Setting
The analog PTZ setting interface is shown as in Figure 2‑43.
[image: image62.png]WEB SERVICEso

Camera Network PTZ Settings | Analog PTZ Settings

Network

Address |1
Event

Baudrate | 9600 )
Storage ]

aa Data it
Stop Bt
Parly | None ]

SiZsetnos Default Reffesh

> Default

> ImporExport

Information





Figure 2‑43
Please refer to the following sheet for detailed information.
	Parameter 
	Function 

	Address
	Set the software address you can control the device via the RS485 BUS. Default value is 1. 

	Baud Rate
	Set the baud rate you can control the device via the RS485 BUS. Default setup is 9600.

	Data Bit 
	Display data bit value.

	Stop bit 
	Display stop bit value. 

	Parity 
	Set the parity you can control the device via the RS485 BUS. 


2.5.4 Default 

The default setup interface is shown as in Figure 2‑44.

Please note system can not restore some information such as network IP address.
[image: image63.png]Ll i ST T BT ST

Camera Default
Network
Event

Storage

System
> General
> Account

> PTZSefings

> ImporiExport

> Auto Maintain

Ungrade
Information




Figure 2‑44
2.5.5 Import/Export 

The interface is shown as in Figure 2‑45.
[image: image64.png]WEB SERVICE®: (ST ST TN IR

camera Import/Export
Network
Event Backup Path

System

> General

> Account
> PTZ Setings
> Default

> Auto Maintain

Upgrade
Information




Figure 2‑45
Please refer to the following sheet for detailed information. 
	Parameter 
	Function 

	Import
	It is to import the local setup files to the system.

	Export 
	It is to export the corresponding system setup to your local PC. 



2.5.6 Auto maintenance 
The auto maintenance interface is shown as in Figure 2‑46.
Here you can select auto reboot and auto delete old files interval from the dropdown list.
If you want to use the auto delete old files function, you need to set the file period. 
[image: image65.png]WEB SERVICER (ST ST TN IR

Camera
Network
Event

O Auto Delete Ol Fil

Storage

System Manual t
—

> Account

> PTZ Setings

> Default

> ImporExport

Upgrade
Information




Figure 2‑46
2.5.7 Firmware update
The firmware interface is shown as in Figure 2‑47.
Please select the upgrade file and then click the update button to begin firmware update. 

Important

Do not turn off the device power, disconnect the device, reboot or shutdown the device during the update period. 

Please reboot the device if you update the improper program, otherwise some function module of the device may become null!
[image: image66.png]WEB SERVICES, (ST ST TN IR

Camera Upgrade
Network
Event e
Storage
System

> General

> Account

> PTZ Setings
> Default
> ImporiExport

> Auto Maintain

Information




Figure 2‑47
2.6 Information 
2.6.1 Version 
The version interface is shown as in Figure 2‑48.
Here you can view system software version, WEB version, release date and etc. Please note the following information is for reference only. 
[image: image67.png]WEB SERVICEso

Camera Version

Network
Software Version  2.103.0000.0.%, build : 2012-08-13

Event

WEBVersion 3000
Storage

PizVersion 1.02.0 RHNCDE
System

CarmeraVersion  01.01.00
Information

> L CapyRight 201,41l Rights Reserved
oy

> online U





Figure 2‑48
2.6.2 Log 

Here you can view system log. See Figure 2‑49.

[image: image68.png]WEB SERVICEso

Camera
Network
Event
Storage
System
Information
>y

Log

> onlin

StartTime|2012- DB - 12 | 16 © 45 38

Tyne | Al Search

End Time 2012 - 06 - 13

164538

Detailed Information

4« 11 D B GoTo

®

Clear





Figure 2‑49
Please refer to the following sheet for log parameter information. 
	Parameter 
	Function 

	Type
	Log types include: system operation, configuration operation, data operation, alarm event, record operation, and user management, log clear.

	Start time
	Set the start time of the requested log.

	End time
	Set the end time of the requested log.

	Search 

	You can select log type from the drop down list and then click search button to view the list.
You can click the stop button to terminate current search operation. 

	Detailed information 
	You can select one item to view the detailed information.

	Clear
	You can click this button to delete all displayed log files. 

	Backup 
	You can click this button to backup log files to current PC.


2.6.3 Online User

The online user interface is shown as in Figure 2‑50.
Here you can view online user name, group name, IP address and login time. 
[image: image69.png]WEB SERVICEso

Camera Online User

sbucri Username User Loca Group 1P Address User Login Time

Event 1015243 2012-06-13 16:21:35
Storage

System
Information

Reffesh





Figure 2‑50
3 Alarm
Click alarm function, you can see an interface is shown as in Figure 3‑1.

Here you can set device alarm type and alarm sound setup. When the specified alarm occurred (you have subscribed), system can record the corresponding alarm information on the right pane of the alarm list. 
[image: image70.png]WEB SERVICEso

Alarm Type
Motion Detect
Disk Errar

O External larm

Operation

O Prompt

Alarm Tone

Flay Alarm Tone

Tone Path Browse.





Figure 3‑1 

Please refer to the following sheet for detailed information. 

	Type 
	Parameter 
	Function 

	Alarm type 
	Motion detect 
	System alarms when motion detection alarm occurs,

	
	Disk full
	System alarms when disk is full.

	
	HDD malfunction
	System generates an alarm when HDD is malfunction.

	
	Camera masking 
	System alarms when camera is viciously masking. 

	
	External alarm 
	Alarm input device sends out alarm. 

	Operation 
	Prompt 
	System automatically pops up alarm dialogue box. 

	Alarm audio 
	Audio 
	When alarm occurs, system auto generates alarm audio. The audio supports customized setup. 

	
	Path 
	Here you can specify alarm sound file. 


4 Log out

Click log out button, system goes back to log in interface. See Figure 4‑1.
You need to input user name and password to login again. 
[image: image71.png]" http://10:15.5°64: BOBE/ = Windows Interner Explorer = -‘

- I





Figure 4‑1
Note:
· This manual is for reference only. Slight difference may be found in user interface.

· All the designs and software here are subject to change without prior written notice.
· All trademarks and registered trademarks are the properties of their respective owners. 
· If there is any uncertainty or controversy, please refer to the final explanation of us.

· Please visit our website for more information.
Input your IP address here





























1      2          3          4          5            6














PAGE  
8

_1380970326

_1380970338

_1380970349

_1380970309

